3GPP TSG-RAN WG3 Meeting #122	R3-237338

Chicago, USA, 13th – 17th November 2023

Agenda Item:	17.3

Source:	ZTE

Title:	Discussion on remaining issues for network verified UE location

Document for:	Discussions & Approval

1	Introduction

In last RAN3 meeting[1], the network verified UE location related issues for NR NTN have been discussed, and the open issues on OAM configuration and potential solutions to mirror positions ambiguity are given as below.

OAM configuration: Whether the NTN related configuration information, e.g., Ephemeris info of the satellites, Association between the satellite and TRP ID are dynamic information？ P1: Ephemeris info of the satellites should be configured to the LMF via OAM. P2: Association between the satellite and TRP ID should also be configured to LMF via OAM. P3: The TRP related information exchange procedure can be applicable to NTN without any standard impact? Location verification solution: Opt1: Reusing the existing NR-CGI IE to transfer the mapped Cell ID. Opt2: Include mapped Cell ID in Measurement Quantities Value IE for E-CID MEASUREMENT INITIATION REQUEST NRRPa message and Define a new Mapped Cell ID IE in E-CID Measurement Result information element Opt3: By implementation Opt4: Provide additional indication from LMF to gNB on the top of the mapped Cell ID Opt5: Provide the right or left of the satellite obit from gNB to LMF

In addition, RAN2 has sent an LS on NW verified UE location failure during cell change in [2].

In this contribution, we provide discussion on the remaining open issues for network verified UE location and analyze whether the incoming LS has any impact on RAN3.

- Discussion

2.1 OAM Configuration

In last meeting, the OAM configuration to LMF has been discussed, and there are three potential proposals on the table.

For potential P1, although the ephemeris information could be changed with the time, it is predictable within the validity duration, and this info can be regarded as semi-static. In addition, the OAM requirement on the ephemeris info has already been introduced to gNB in TS 38.300, it is reasonable to follow the similar principle to configure the ephemeris info to the LMF via OAM. Furthermore, with the help of the NTN control center, it is not a problem to align the configuration of the RAN OAM with CN OAM.

Proposal 1: The ephemeris information of satellite should be configured to LMF via OAM.

For potential P2, to obtain the accurate TRP information, the association between the satellite ephemeris information and the TRP ID should be also configured to the LMF via OAM.  With the association info, the LMF is able to obtain the accurate TRP ID and request the detailed TRP Information via the existing NRPPa TRP Information Exchange procedure as defined in TS 38.455 [2]. After that, the subsequent Positioning related procedures could be performed appropriately.

Proposal 2: The association between the TRP ID and the satellite ephemeris information should be configured to the LMF via OAM.

In addition, to avoid the mismatch of the above association information between the gNB and LMF, the association between the TRP ID and the satellite ephemeris information should also be configured to the gNB via OAM. Then, the gNB is able to provide the accurate TRP information.

Proposal 3: The association between the TRP ID and the satellite ephemeris information should be configured to the gNB providing NTN access via OAM.

For potential P3, as mentioned above, the existing NRPPa TRP Information Exchange procedure can be reused for NTN without any enhancement, and the P3 could be a common understanding, RAN3 does not need to make any restriction on this proposal.

2.2 Mirror positions ambiguity

Regarding the mirror positions ambiguity issue, five options were provided by companies to solve this issue in last meeting, e.g.,extend the usage of mapped cell ID, enhance the existing E-CID method, leave it by implementation and introduce a new indication  from gNB to LMF.

In last meeting, some companies were in favour of the Option 2, they thought the existing E-CID method should be enhanced to support the NTN. Of course, Option 2 is a possible way to solve the mirror positions ambiguity issue. While, to avoid the specification impact, the other option should be also considered  if it has not impact on RAN3 when applied.

To be more specific, for Option 1, as discussed in [3], the UE actual position and mirror position can be located in different cells. In this case, network is easy to solve the mirror image ambiguity issue based on the coverage area of the cell which the UE locates in. The cell which UE locates in can be known via the cell ID included in RTT measurement report, which is already implemented in current specification. Even if the actual position and mirror position are within same cell, the UE position can also be identified based on the neighbor RSRP measurements or UL-AoA, which are also implementation issue. This solution can be regarded as gNB implementation, and there is no spec impact on RAN3. In addition, as discussed in [4], when the mirror ambiguity occurs, the LMF may verify both positions compared with the reported UE locations, when both positions are failed for the verification, the LMF deems the reported UE location is not verified, otherwise, the UE reported location is considered to be verified. This method is the simplest solution and it can be left for LMF implementation. This solution can be regarded as LMF implementation, and there is also no spec impact on RAN3. Based on the above analysis, the potential solutions for Option 1 have no impact on RAN3.

Proposal 4: The mirror positions ambiguity could be solved by gNB or LMF implementation without any impact on RAN3.

Alternatively, if companies still prefer to enhancing the existing E-CID to solve the mirror positions ambiguity issue, there is no need to modify the stage 3 IEs of the existing E-CID positioning related procedure.And extending the usage of the Mapped Cell ID to support the E-CID positioning in TS 38.300 is sufficient to solve this issue.

2.3 LS from RAN2

From the incoming LS [5], RAN2 asks RAN3 whether existing procedure or existing error cause can handle the following scenarios with the intention of avoid significant delay in starting positioning with the new satellite.

Scenario 1: Handover between TN and NTN where LMF needs to be aware that UE capabilities may be different. Scenario 2: Satellite switch with PCI unchanged where SRS and PRS configurations remain the same, but LMF would need to have updated satellite information. Scenario 3: Intra-gNB handover

For Scenario 1, handover between TN and NTN, if the LMF wants to obtain the UE capabilities, the impact on NAS signalling should be considered instead of the NRPPa signalling. Therefore, the existing procedure is enough.

For Scenario 2, satellite switch with PCI unchanged, LMF is able to obtain the updated TRP information by using the existing TRP Information Exchange procedure. Therefore, the existing  procedure is enough.

For Scenario 3, intra-gNB handover, as given in TS 38.455, the existing cause value “Serving NG-RAN node changed” is able to handle this scenario.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, Requested Item not Supported, Requested Item Temporarily not Available, ..., Serving NG-RAN node changed, Requested Item not Supported on Time ) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Unspecified, Abstract Syntax Error (Falsely Constructed Message), ...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Unspecified, ...) |  |

Based on the above analysis, for the network verified UE location failure during cell change has not significant impact on RAN3, and RAN3 shall send the reply LS to RAN2 to indicate RAN3’s view on this issue.

Proposal 5: Send the reply LS to RAN2 to indicate that the network verified UE location failure has no impact on RAN3.

Conclusion

Proposal 1: The ephemeris information of satellite should be configured to LMF via OAM.

Proposal 2: The association between the TRP ID and the satellite ephemeris information should be configured to the LMF via OAM.

Proposal 3: The association between the TRP ID and the satellite ephemeris information should be configured to the gNB providing NTN access via OAM.

Proposal 4: The mirror positions ambiguity could be solved by gNB or LMF implementation without any impact on RAN3.

Proposal 5: Send the reply LS to RAN2 to indicate that the network verified UE location failure has no impact on RAN3.

The TP to NTN BL CR 38.300, draft CR to 38.305 on OAM configuration for network verified UE location, and the reply LS to RAN2 is given in the Annex.

References

- Chairman Notes in RAN3#121bis

- TS 38.455

- R1-2306564, Discussion on network-verified UE location for NR NTN, ZTE

- R1-2307544, Discussion on network verified UE location for NR NTN, OPPO

- R3-237138, LS on NW verified UE location failure during cell change, RAN2

Annex

Draft CR on 38.305

3GPP TSG-RAN WG3 #122	R3-23xxxx

Chicago, USA, 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | OAM requirement for network verified UE location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In NTN, especially for the LEO and MEO scenarios, since the TRP is located on the satellite which is earth-moving, the TRP information should also be dynamically changed with the movement of the satellite, such as the geographical coordinates. In this case, the LMF cannot obtain the accurate TRP information to trigger the appropriate Positioning related procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description that the satellite ephemeris information and the association between the TRP ID and the satellite ephemeris information should be configured to LMF via OAM. Impact Analysis: This CR has an impact under protocol point of view. The impact can be considered isolated because the change affects only the Positioning for NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LMF cannot obtain the accurate TRP information to trigger the appropriate Positioning related procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

5.4.4	Location Management Function (LMF)

The LMF manages the support of different location services for target UEs, including positioning of UEs and delivery of assistance data to UEs. The LMF may interact with the serving gNB or serving ng-eNB for a target UE in order to obtain position measurements for the UE, including uplink measurements made by an NG-RAN and downlink measurements made by the UE that were provided to an NG-RAN as part of other functions such as for support of handover.

The LMF may interact with a target UE in order to deliver assistance data if requested for a particular location service, or to obtain a location estimate if that was requested.

The LMF may interact with multiple NG-RAN nodes to provide assistance data information for broadcasting. The assistance data information for broadcast may optionally be segmented and/or ciphered by the LMF. The LMF may also interact with AMFs to provide ciphering key data information to the AMF as described in greater detail in TS 23.273 [35].

For positioning of a target UE, the LMF decides on the position methods to be used, based on factors that may include the LCS Client type, the required QoS, UE positioning capabilities, gNB positioning capabilities and ng-eNB positioning capabilities. The LMF then invokes these positioning methods in the UE, serving gNB and/or serving ng-eNB. The positioning methods may yield a location estimate for UE-based position methods and/or positioning measurements for UE-assisted and network-based position methods. The LMF may combine all the received results and determine a single location estimate for the target UE (hybrid positioning). Additional information like accuracy of the location estimate and velocity may also be determined.

The LMF may interact with the AMF to provide (updated) UE Positioning Capability to AMF and to receive stored UE Positioning Capability from AMF as described in TS 23.273 [35].

The following NTN related parameters shall be provided by OAM to the LMF:

-	Ephemeris information, including the accurate satellite position and velocity at the time of measurement.

-	Association between the TRP ID and the satellite ephemeris information.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

Text Proposal for NTN BL CR 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 16.14.7	O&M Requirements

The following NTN related parameters shall be provided by O&M to the gNB providing NTN access:

-	Ephemeris information describing the orbital trajectory information or coordinates for the NTN payload. This information is provided on a regular basis or upon demand to the gNB;

-	Two different sets of ephemeris format shall be supported:

-	Set 1: NTN payload position and velocity state vectors:

-	Position;

-	Velocity.

-	Set 2: At least the following parameters in orbital parameter ephemeris format, as specified in NIMA TR 8350.2 [51]:

-	Semi-major axis;

-	Eccentricity;

-	Argument of periapsis;

-	Longitude of ascending node;

-	Inclination;

-	Mean anomaly at epoch time.

-	The explicit epoch time associated to ephemeris data;

-	The location of the NTN Gateways;

NOTE 1:	The ephemeris of the NTN payloads and the location of the NTN Gateways, are used at least for the Uplink timing and frequency synchronization. It may also be used for the random access and the mobility management purposes.

-	Additional information to enable gNB operation for feeder/service link switch overs.

NOTE 2:	The NTN related parameters provided by O&M to the gNB may depend on the type of supported service links, i.e., Earth-fixed, quasi-Earth-fixed, or Earth-moving.

-	Association between the TRP ID and the satellite ephemeris information.

NOTE 3:	The above association information should be used for calculating the accurate TRP information.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

Draft reply LS to RAN2

Title:	[Draft] Reply LS on NW verified UE location failure during cell change

Response to:	R3-237138

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	ZTE (to be RAN3)

To:	RAN2

Cc:	N/A

Contact Person:

      - Name:	Jiren Han

      - E-mail Address:	han.jiren@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall Description:

RAN3 thanks RAN2 for the LS on NW verified UE location failure during cell change.

For Scenario 1, RAN3 thinks it only impacts on NAS signalling rather than the NRPPa signalling, and the existing procedure in RAN3 is sufficient.

For Scenario 2, RAN3 thinks the LMF is able to obtain the updated TRP information by using the existing TRP Information Exchange procedure without any enhancement.

For Scenario 3, the existing cause value “Serving NG-RAN node changed” given in TS 38.455 is able to handle this scenario.

Therefore, from RAN3’s prospective, the existing procedure or existing error cause can handle the above scenarios.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 kindly asks RAN2  to consider the above answer and provide feedback if any.

3. Date of Next RAN3 Meeting:

TSG RAN WG3 Meeting #123     26th, Feb – 2nd, Mar. 2024 		 Athens, Greece