3GPP TSG-RAN WG3 Meeting #122	R3-237456

Chicago, USA, 13th – 17th November, 2023

Agenda item:	10.2.5

Source:	Qualcomm Incorporated

Title:	SON enhancements for NR-U

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss the open issues on SON enhancements for NR-U based on the discussion in the previous RAN3 meetings.

# Discussion

  - MRO for NR-U

In the previous meeting, RAN3 agreed on the following working assumption in order to identify if there were DL LBT failures during handover execution to assist the source node while performing MRO.

WA: RAN3 agrees to enable reporting of number of DL LBT failures from the target node to the source node in case of failed HO attempt (assuming the UE can be identified). Details are FFS.

In our view, the target gNB can inform source gNB regarding the DL LBT failures encountered during a failed HO attempt by indicating the “number of DL LBT failures” in a class-2 message with UE associated signaling (e.g., by using UE XnAP ID). We don’t need to specify how the target gNB computes the number of DL LBT failures (e.g., whether it counts LBT failures for MSG2 or MSG4 or both) similar to how we don’t specify any such UE behavior when it counts number of UL LBT failures. We therefore have the following proposals:

Proposal 1: In case of HOF, target gNB can inform source gNB regarding the DL LBT failures encountered during the failed HO attempt by indicating the “number of DL LBT failures” in a class-2 message with UE associated signaling (e.g., by using UE XnAP ID).

There was also the following FFS captured last meeting:

FFS whether in case of DL LBT problems at the last serving node that led to the Too Early HO / HO to Wrong Cell, the last serving node skips informing the source node or informs it with additional information

In case of RLF shortly after a successful HO resulting in a Too Early HO/HO to Wrong cell, the last serving node doesn’t need to inform the source node with any LBT information (e.g., the number of DL LBT failures encountered when the Tstore_UE_cntxt (defined in TS 38.300) is running), similar to how UE doesn’t report the number of UL LBT failures post a successful HO. The last serving node can simply send the HANDOVER REPORT with the HO cause as it sends today.

We therefore have the following proposal:

Proposal 2: In case of RLF shortly after a successful HO resulting in a Too Early HO/HO to Wrong cell, the last serving node doesn’t need to inform the source node with LBT related information when the Tstore_UE_cntxt timer was running (e.g., the number of LBT failures encountered when Tstore_UE_cntxt timer was running). The last serving node can simply send the HANDOVER REPORT (optionally with the RLF Report Container) as it is sent in current specifications.

FFS if reporting of the number of UL LBT failures in case of non-HOF RLF can be addressed in Rel.18

The above FFS corresponds to a pure RLF case or Too late HO case, where there was an RLF encountered in the source cell before the HO is executed. In such a case, knowing the number of UL LBT failures in source cell is not really useful for any MRO related optimizations. Further, RAN2 is also discussing whether to count such LBT failures in source cell during a successful HO (e.g., by counting the number of LBT MAC CE sent) and this is not agreed yet. So we propose the following:

Proposal 3: There is no need for UE to report the number of UL LBT failures in source cell in case of a pure-RLF case or before a Too late HO.

  - MLB for NR-U

FFS on whether to add a load metrics on the number of UL consistent LBT failures in the Resource Status Update

We don’t see a strong need to define a new load metric “number of UL consistent LBT failures” in Rel-18 and can be deprioritized. Therefore, the following is proposed:

Proposal 4:  There is no need to define a new load metrics “number of UL consistent LBT failures” in the Resource Status Update message in Rel-18

# Conclusion

Proposal 1: In case of HOF, target gNB can inform source gNB regarding the DL LBT failures encountered during the failed HO attempt by indicating the “number of DL LBT failures” in a class-2 message with UE associated signaling (e.g., by using UE XnAP ID).

Proposal 2: In case of RLF shortly after a successful HO resulting in a Too Early HO/HO to Wrong cell, the last serving node doesn’t need to inform the source node with LBT related information when the Tstore_UE_cntxt timer was running (e.g., the number of LBT failures encountered when Tstore_UE_cntxt timer was running). The last serving node can simply send the HANDOVER REPORT (optionally with the RLF Report Container) as it is sent in current specifications.

Proposal 3: There is no need for UE to report the number of UL LBT failures in source cell in case of a pure-RLF case or before a Too late HO.

Proposal 4:  There is no need to define a new load metrics “number of UL consistent LBT failures” in the Resource Status Update message in Rel-18

# 4. References