3GPP TSG-RAN3 #121-bis	R3-237491

13th–17th November 2023

Chicago, USA

Title: 	AI/ML Network Energy Saving

Source: 	Ericsson, Deutsche Telekom, AT&T

Agenda item: 	12.2.2.3

Document Type:	Discussion and Decision

# Introduction

The discussion on the Energy Cost IE for AI/ML Network Energy Saving was continued during RAN3#121-bis and the following agreements were captured:

RAN3 agree that ng-eNBs (E-UTRA nodes connected to 5GC) are not in the scope of the Rel-18 AI/ML for NG-RAN WI, and provide feedback on this to SA5 in the Reply LS.

The area where the “Energy Consumption – to Energy Cost” mapping rule is applicable is up to the Operator.

In addition, a reply LS to SA5 was agreed and SA5’s questions were answered as follows [1]:

Q1. Who should be responsible for defining the unified mapping rule: 3GPP / NG-RAN node vendor / Operator / Other?

[RAN3 answer to Q1.]: It is up to the Operator to define the unified “Energy Consumption – to – Energy Cost” mapping rule.

Q2. Should the calculation of the Energy Cost be based on instantaneous power consumption measurement (in W) (implying that the Energy Cost can be constantly updated) or on min/max/mean energy consumption measurements (in KwH) made over a past period (whose duration can be configured by the Operator)?

[RAN3 answer to Q2.]: RAN3 assumes that a NG-RAN node is capable to self-measure its Energy Consumption and then derive the corresponding Energy Cost, the latter being a dimensionless index which is derived on the basis of averaged measurements of the NG-RAN node’s consumed energy. It is up to the OAM to configure the Energy Consumption values corresponding to the minimum and maximum Energy Cost index values. By means of configuration the OAM provides a recommended time interval within which to perform the average.

Q3. Should the Energy Cost be calculated in near-real time or non-real time (see above)?

[RAN3 answer to Q3.]: Given that the Energy Cost index is derived on the basis of averaged measurements of the NG-RAN node’s consumed energy, it cannot be defined as real time or non-real time.

Q4. What is your view on how to apply this Energy Cost concept to split NG-RAN nodes, where NG-RAN node components can be geographically distributed and/or virtualized?

[RAN3 answer to Q4.]: In Rel-18 the Energy Cost index concept is limited to non-split NG-RAN nodes.

Moreover, RAN3 would like to provide further clarifications on the SA5 understanding of the problem to be solved. In particular:

- Energy Cost metric applicability to ng-eNBs – RAN3 agreed that ng-eNBs (E-UTRA nodes connected to 5GC) are not in the scope of the Rel-18 AI/ML for NG-RAN WI.

- Whether the “Energy Consumption – to – Energy Cost” mapping rule should be unified across the whole Operator’s network – RAN3 wants to further clarify that it is up to the Operator to define the area where the mapping rule is applicable.

In this paper we build on the agreements and reply LS to SA5 above and address the remaining FFS’s related to the Energy Cost IE captured in the BLCR to TS 38.423 [2].

# Discussion

In the last meeting RAN3 has agreed on the following:

It is up to the Operator to define the unified “Energy Consumption – to – Energy Cost” mapping rule.

The area where the “Energy Consumption – to Energy Cost” mapping rule is applicable is up to the Operator.

In addition, RAN3’s reply LS to SA5 captured the following:

It is up to the OAM to configure the Energy Consumption values corresponding to the minimum and maximum Energy Cost index values. By means of configuration the OAM provides a recommended time interval within which to perform the average.

Figure 1 summarizes our understanding of the above RAN3 agreements from the last meeting. While we believe a linear energy consumption to Energy Cost mapping rule is reasonable and often most appropriate, non-linear mapping rules, as shown in Figure 1, are generally possible and will not be precluded by 3GPP specifications.

Moreover, SA5 may specify means to configure via OAM an energy consumption value, arbitrarily denoted z1, corresponding to the minimum Energy Cost index 0, and an energy consumption value, z2, corresponding to the maximum Energy Cost index, currently 10000, as well as a recommended time interval / duration, d, within which to perform averaging.

Figure 1: Examples of Energy Consumption to Energy Cost mapping rules.

Currently, the semantics description of the Energy Cost in the BLCR to TS 38.423 [2] specifies the following:

The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. Energy Consumption is measured on a linear scale. (FFS)

The last, underlined sentence comes from a time when RAN3 assumed 3GPP will specify that Energy Cost has a linear relation to energy consumption. However, this will not be the case as per latest RAN3 agreements on the matter. We believe this sentence could easily be misinterpreted as a limitation to a linear relationship. To honor the agreements and to prevent misunderstanding of the XnAP specification, we propose to remove that sentence together with the FFS.

Proposal 1: Remove “Energy Consumption is measured on a linear scale. (FFS)” from the semantics description of the Energy Cost in the BLCR to TS 38.423.

Proposal 2: The semantics description of the Energy Cost in TS 38.423 should specify: “The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption”.

Lastly, there still is an FFS on the assigned criticality level of the Energy Cost index in the BLCR to TS 38.423 [2]. We propose to set the assigned criticality to “ignore”, which mirrors what RAN3 did for the Cell Info Result List, and to remove the FFS.

Proposal 3: Set assigned criticality of the Energy Cost in TS 38.423 to “ignore” and remove the FFS.

# Conclusion

In this paper addressed the remaining FFS’s related to the Energy Cost IE captured in the BLCR to TS 38.423 [2]. The corresponding proposals are listed below:

Proposal 1: Remove “Energy Consumption is measured on a linear scale. (FFS)” from the semantics description of the Energy Cost in the BLCR to TS 38.423.

Proposal 2: The semantics description of the Energy Cost in TS 38.423 should specify: “The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption”.

Proposal 3: Set assigned criticality of the Energy Cost in TS 38.423 to “ignore” and remove the FFS.

A TP mirroring the proposals above is available in R3-237492.

# References

[1]		R3-235743, Reply LS on Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index, RAN3

[2]		R3-235955, (BLCR to 38.423) for AI/ML for NG-RAN, Ericsson, Nokia, Nokia Shanghai Bell