3GPP TSG-RAN WG3 Meeting #122	R3-237506

Chicago, USA, November 13th – November 17th, 2023

# Agenda Item:	31.1

# Source:	Ericsson, Huawei, China Telecom

# Title:	Correction for RRC Inactive

# Document for:	Discussions & Approval

# 1	Introduction

In this paper we discuss a correction for the definition of the I-RNTI IE in XnAP.

# 2	Discussion

As of today there are two places in TS 38.300 where the I-RNTI structure is described, Annex C (Informative) and Annex F (Normative). Annex F has been introduced in Rel-17, when RAN3 agreed to introduce an interoperable structure for the I-RNTI.

In Annex C the following text is found:

The I-RNTI provides the new NG-RAN node a reference to the UE context in the old NG-RAN node. How the new NG-RAN node is able to resolve the old NG-RAN ID from the I-RNTI is a matter of proper configuration in the old and new NG-RAN node.

Table C-1 below provides some typical partitioning of a 40bit I-RNTI, assuming the following content:

-	UE specific reference: reference to the UE context within a logical NG-RAN node;

-	NG-RAN node address index: information to identify the NG-RAN node that has allocated the UE specific part;

NOTE:	RAT-specific information may be introduced in a later release, containing information to identify the RAT of the cell within which the UE was sent to RRC_INACTIVE. This version of the specification only supports intra-RAT mobility of UEs in RRC_INACTIVE.

-	PLMN-specific information: information supporting network sharing deployments, providing an index to the PLMN ID part of the Global NG-RAN node identifier.

Table C-1: I-RNTI reference profiles

| Profile ID | UE specific reference | NG-RAN node address index (e.g., gNB ID, eNB ID) | RAT-specific information | PLMN-specific information | Comment |
| --- | --- | --- | --- | --- | --- |
| 1 | 20 bits (~1 million values) | 20 bits (~1 million values) | N/A | N/A | NG-RAN node address index may be very well represented by the LSBs of the gNB ID. This profile may be applicable for any NG-RAN RAT. |
| 2 | 20 bits (~1 million values) | 16 bits (65.000 nodes) | N/A | 4 bits (Max 16 PLMNs) | Max number of PLMN IDs broadcast in NR is 12. This profile may be applicable for any NG-RAN RAT. |
| 3 | 24 bits (16 million values) | 16 bits (65.000 nodes) | N/A | N/A | Reduced node address to maximise addressable UE contexts. This profile may be applicable for any NG-RAN RAT. |

As it can be noticed, the Table C-1 shows three different structures for the I-RNTI:

- In profile ID =1, the I-RNTI has a structure made of two parts: 1) UE reference, 2) NG-RAN node address index

- In profile ID =2, the I-RNTI has a structure made of three parts: 1) UE reference, 2) NG-RAN node address index, 3) PLMN specific

- In profile ID =1, the I-RNTI has the same structure as Profile ID =1, with different split of bits between the two parts.

Let’s consider now the text reported in TS 38.423 for I-RNTI definition:

#### 9.2.3.46	I-RNTI

The I-RNTI is defined for allocation in an NR or E-UTRA serving cell as a reference to a UE Context within an NG-RAN node. The I-RNTI is partitioned into two parts, the first part identifies the NG-RAN node that allocated the I-RNTI and the second part identifies the UE context stored in this NG-RAN node, refer to Annex C in TS 38.300 [9], or the I-RNTI is partitioned into three parts, the first part indicates the length of NG-RAN Node ID part of the NG-RAN Node that allocated the I-RNTI, the second part identifies the NG-RAN node that allocated the I-RNTI and the third part identifies the UE context stored in this NG-RAN node, refer to Annex F in TS 38.300[9].

The part marked in yellow refers to the text which existed before the Annex F was agreed. There are at least two problems with such text. First of all, the presence of Annex F makes Annex C obsolete. At least from rel-17, the I-RNTI structure needs to be according to the content of Annex F, and this is not visible. For previous releases, the I-RNTI structure, as described in Annex F, was not supported. However, the text in yellow only accounts for one of the options reported in Annex C. Therefore, in referring to Annex C, the used text is not correct.

It should at least be made clear that Annex C is an informative text, while Annex F is a normative text. The existing text can be modified as follows:

#### 9.2.3.46	I-RNTI

The I-RNTI is defined for allocation in an NR or E-UTRA serving cell as a reference to a UE Context within an NG-RAN node. The I-RNTI is partitioned into two parts, the first part identifies the NG-RAN node that allocated the I-RNTI and the second part identifies the UE context stored in this NG-RAN node, refer to the informative Annex C in TS 38.300 [9], or the I-RNTI is partitioned into three parts, the first part indicates the length of NG-RAN Node ID part of the NG-RAN Node that allocated the I-RNTI, the second part identifies the NG-RAN node that allocated the I-RNTI and the third part identifies the UE context stored in this NG-RAN node, refer to the normative Annex F in TS 38.300[9].

Proposal 1: In the definition for the I-RNTI IE, make a distinction between the references to normative text (Annex F) and the informative text (Annex C).

Proposal 2: Agree to the proposed CR for TS 38.423 in R3-23707.

# Conclusion

In this paper we discussed a correction for XnAP to align the stage 3 description for I-RNTI. The following proposal is made:

Proposal 1: In the definition for the I-RNTI IE, make a distinction between the references to normative text (Annex F) and the informative text (Annex C).

Proposal 2: Agree to the proposed CR for TS 38.423 in R3-23707.