3GPP TSG-RAN WG3 Meeting #122	R3-237676

Chicago, USA, Nov 13- Nov 17, 2023

Agenda item:	26.3

Source:	CMCC, Nokia, Nokia Shanghai Bell

Title:	Discussion on network slice service continuity

Document for:	Discussion

Introduction

During previous meeting, there is no conclusion achieved on network slice service continuity scenario, only one remaining issue is left as follows.

Add both Alternative S-NSSAI and Original S-NSSAI IE in the NGAP PDU Session Setup Request and the XnAP Handover Request message?

Since this meeting will be the last meeting for RAN Slicing WI in Rel-18, we hope that companies only focus on the issue above and reach consensus on the service continuity scenario. This contribution will continue to discuss the network slice service continuity based on the left issue.

Discussion

Network Slice Replacement

From the SA2 specification, it is found that the Network Slice Replacement feature is used to replace an S-NSSAI with an Alternative S-NSSAI when an S-NSSAI becomes unavailable or congested based on the notification originated from NSSF, PCF and OAM. After the AMF determine that an S-NSSAI is to be replaced with alternative S-NSSAI, PDU session could be established towards the alternative S-NSSAI or retained by using existing PDU session associated with alternative S-NSSAI. Anyway, the SMF shall sends the alternative S-NSSAI to NG-RAN in N2 SM information as specified in SA2 CR.

During a new PDU Session establishment procedure towards an S-NSSAI, - if the UE is provided with the mapping of the S-NSSAI to an Alternative S-NSSAI, the UE provides both the Alternative S-NSSAI and the S-NSSAI in the PDU Session Establishment message. When the AMF receives the Alternative S-NSSAI and the S-NSSAI in the PDU Session Establishment message, or when the AMF receives only the S-NSSAI in PDU Session Establishment message the AMF determines that the S-NSSAI is to be replaced with the Alternative S-NSSAI, the AMF includes both the Alternative S-NSSAI and the S-NSSAI to the SMF. - if the UE is not provided with the mapping of the S-NSSAI to the Alternative S-NSSAI, the UE provides the S-NSSAI in the PDU Session Establishment message. When the AMF determines that the requested S-NSSAI is to be replaced with the Alternative S-NSSAI and if the UE supports Network Slice Replacement, the AMF performs UE Configuration Update procedure to reconfigure the UE with the Alternative S-NSSAI. The AMF continues the PDU Session establishment procedure with the Alternative S-NSSAI and provides both the Alternative S-NSSAI and the S-NSSAI to the SMF. The SMF proceeds with the PDU Session establishment using the Alternative S-NSSAI. The SMF sends the Alternative S-NSSAI to NG-RAN in N2 SM information and to UE in PDU Session Establishment Accept message. For existing PDU Session associated with an S-NSSAI that is replaced with the Alternative S-NSSAI, after the AMF sends mapping of the S-NSSAI to the Alternative S-NSSAI to the supporting UE in UE Configuration Update message, the AMF sends updates to the SMF of the PDU Session, e.g. triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be transferred to Alternative S-NSSAI and includes the Alternative S-NSSAI as follows (see details in clause 4.3.3 of TS 23.502 [3]): - If the SMF determines that the PDU Session needs to be retained (e.g. if the anchor UPF can be reused with the alternative S-NSSAI and SSC mode 1), the SMF sends the Alternative S-NSSAI to the UPF in the N4 message, to the NG-RAN in N2 message and to the supporting UE in PDU Session Modification Command message. - If the SMF determines that the PDU Session needs to be re-established, the SMF sends the Alternative S-NSSAI to the supporting UE either in PDU Session Modification Command if the PDU Session is of SSC mode 3, or in PDU Session Release if the PDU Session is of SSC mode 2 or SSC mode 1, to trigger the re-establishment of the PDU Session. The UE includes both, the S-NSSAI and the Alternative S-NSSAI in the PDU Session Establishment message.

From the highlights described, there will be impact on NGAP associating with PDU Session Resource Setup procedure and PDU Session Resource Modify procedure.

For PDU session establishment procedure, a S-NSSAI IE is already included in PDU SESSION RESOURCE SETUP/MODIFY REQUEST message. While SMF sends the replacing S-NSSAI IE to NG-RAN node, a new separate S-NSSAI IE is introduced to represent the original slice. The reason is that we desire to adopt a future-proof solution. Since CN does not aware the slice resource is unavailable or congested in the NG-RAN node, there is a possibility that NG-RAN node may not support the alternative slice due to temporary overloaded or unavailable.

Observation 1: Since CN does not aware the slice resource is unavailable or congested in the NG-RAN node, there is a possibility that NG-RAN node may not support the alternative slice due to temporary overloaded or unavailable.

Assumed that the resource for the original slice in CN network is relieved or available again, CN sends the PDU Session Resource Setup/Modify procedure to indicate NG-RAN node continue to use original slice, but at the moment original slice is overloaded in NG-RAN node. If reusing existing S-NSSAI to indicate original slice in PDU Session Resource Setup/Modify procedure, the NG-RAN node will fail to establish the PDU session with original slice. If introducing a separate IE indicating the current replaced slice to differentiate the replacing slice, NG-RAN node can decide to whether using the replaced slice based on its congestion situation. In spite of the fact that SA2 does not want to consider the scenario that slice is unavailable or congested in the NG-RAN node, from our view, introducing a new separate IE for replaced slice provides a feasible and future-oriented solution to solve the mismatch between the congestion in CN and NG-RAN node.

Observation 2: Introducing a new separate IE for replaced slice provides a feasible and future-oriented solution to solve the mismatch between the congestion in CN and NG-RAN node.

Proposal 1: It is proposed to introduce a new IE for replaced slice in PDU Session Resource Setup/Modify procedure.

Network Slice Replacement during HO

From SA2’s conclusion, it is specified as follows:

During a handover procedure, if an S-NSSAI has to be replaced with an Alternative S-NSSAI, the handover procedure (including any PDU session associated with the S-NSSAI to be replaced) shall continue unaffected by the Network Slice Replacement. Any Network Slice Replacement for the S-NSSAI shall not take place during the handover.

SA2 already agrees that any Network Slice Replacement for the S-NSSAI shall not take place during the handover. From our perspective, network slice replacement may perform before handover instead of during handover. Before handover, CN can trigger the network slice replacement by establishing new PDU session or modifying the existing PDU session, ensuring the service continuity for the potential upcoming handover.

Assumed that a UE in gNB1 is served by slice 1 and the slice availability for gNBs within a certain area is shown as follows:

- gNB1 supports slice 1 and slice 2.

- gNB2 supports slice 1 and slice 2.

- gNB3 supports slice 2.

- gNB4 supports slice 1 and slice 2.

Due to the slice 1 in CN is congested or unavailable, CN triggers the network slice replacement for UE1 by replacing associated slice from slice 1 to slice 2. Then, gNB1 receives the remapped information and decides to trigger the handover later and only includes the slice 2 in the handover request message to gNB2. gNB2 does not know whether slice 2 is an alternative slice. If gNB2 triggers subsequent handover, gNB2 will choose between gNB3 and gNB4. Actually, candidate gNB4 should have been a potential target supporting both original slice and alternative slice for the UE. If gNB2 selects gNB3 as the target node, UE cannot be served by slice 1 since it is not supported by gNB3 if the slice 1 in CN is relieved or becomes available again. From the perspective of selecting candidate nodes, gNB4 supporting original slice is better than gNB3 since it provides another option for UE. It is possible that original slice may support a better SLA compared to alternative slice. In order to avoid the case mentioned above, we think it is required to signal the original slice IE and alternative slice IE respectively in the Xn Handover Request message, providing a possibility that UE can be served by original slice again.

Observation 3: Signalling both the original slice and alternative slice IE in the Handover Request message provides a possibility that UE can be served by original slice again.

Proposal 2: It is proposed to signal both the original and alternative slice IE and alternative slice IE respectively in the Handover Request message.

Conclusions

In this paper, we provide our insights on network slice service continuity. The following observations and proposals are listed below:

Observation 1: Since CN does not aware the slice resource is unavailable or congested in the NG-RAN node, there is a possibility that NG-RAN node may not support the alternative slice due to temporary overloaded or unavailable.

Observation 2: Introducing a new separate IE for replaced slice provides a feasible and future-oriented solution to solve the mismatch between the congestion in CN and NG-RAN node.

Observation 3: Signalling both the original and alternative slice IE in the Handover Request message provides a possibility that UE can be served by original slice again.

Proposal 1: It is proposed to introduce a new IE for replaced slice in PDU Session Resource Setup/Modify procedure.

Proposal 2: It is proposed to signal both the original and alternative slice IE and alternative slice IE respectively in the Handover Request message.

Reference

- TR 23.700-41, Study on enhancement of network slicing;

- TS 23.501, System architecture for the 5G System (5GS);