3GPP TSG-RAN WG3 Meeting #122                 	                                             R3-237678

Chicago, USA, Nov 13- Nov 17, 2023

Agenda Item:	25.2.1

Source:	CMCC

Title:	Discussion on PDU Set handling

Document for:	Discussion

Introduction

During the last meeting, the conclusions and remaining issues of XR enhancement are listed below:

RAN3#121bis： QFI needs to be included in Xn-U and F1-U for downlink data. Option1: define PSI and EoDB into the existing Frames (e.g frame with PDU Type =0) of NG-U/F1-U Option2: define PSI and EoDB into a new Frame (PDU type) in the existing container of NG-U/F1-U Option3: Define a new GTP-U extension PDU Set container for PSI and EoDB Rule out option 2. Postpone to next meeting the decision between option 1 and option 3. If a gNB supports PDU Set handling, all parts support it: DU, CU-CP, CU-UP. A new support indication should be included in the SMF-related IEs, i.e. - PDU Session Management case: PDU Session Resource Setup Response Transfer, PDU Session Resource Modify Response Transfer, - Handover case: Path Switch Request Transfer, and Handover Request Acknowledge Transfer IEs. FFS whether to use an explicit PDU Set handling Support indicator, or indicate a list of supported NGAP IE IDs from NG-RAN upon request from SMF WA: Taking Opt1 (Explicit PDU Set handling Support indicator) for XR in R18. Solution down-selection and Stage2 and stage3 details

In this contribution, we will continue discussing PDU Set handling in non-homogeneous deployment.

Discussion

GTP-U Containers

During last meeting, companies do not reach consensus on whether to create a new GTP-U extension PDU Set container or reuse existing frame of NG-U/F1-U by adding the QoS Set parameters. The proponents of introducing a new GTP-U extension PDU Set container supports that the new GTP-U container does not bring non backward compatible issue where the NG-RAN node before rel-18 could not handle the extendable part of PDU Set QoS parameters if reusing existing mechanism.

Actually, from our view, there are no NBC issues foreseen when people decide to reuse existing PDU frame. As the TS 29.281 described below, it is specified that the Extension Header Length field in general format of GTP-U Extension Header tells the length of the Extension Header. If a NG-RAN node does not have the capability to identify the PDU Set QoS prameters, it will ignore the unknown part in the bottom of Extension Header Content by skipping directly after it knows the length of Extension Header. Thus, the legacy NG-RAN node could handle the readable part and skip the unknown part of the frame through fetching the Extension Header Length in advance. This mechanism actully is workable in pre-release when introducing other features.

Observation 1: There are no NBC issues foreseen where the legacy NG-RAN node could handle the readable part and skip the unknown part of the frame through fetching the Extension Header Length in advance.

### 5.2.1 General format of the GTP-U Extension Header The format of GTP-U Extension Headers is depicted in figure 5.2.1-1. The Extension Header Length field specifies the length of the particular Extension header in 4 octets units. The Next Extension Header Type field specifies the type of any Extension Header that may follow a particular Extension Header. If no such Header follows, then the value of the Next Extension Header Type shall be 0. Octets 1; Extension Header Length. 2 – m; Extension Header Content. m+1; Next Extension Header Type. Figure 5.2.1-1: Outline of the Extension Header Format The length of the Extension header shall be defined in a variable length of 4 octets, i.e. m+1 = n*4 octets, where n is a positive integer.

Considered that the XR WI will be closed in upcoming meeting, we intend to adopt a simple way by adding PDU QoS parameters in existing GTP-U Extension container. We have to admit that introducing a new container is a future proof solution but needs to spend time if designing a new specificaiton. Due to the limit time to consider the impact on existing user plane protocol after introducing a new container, we prefer to reuse the existing frames in existing container in the last meeting of Rel-18.

Proposal 1: Due to the limit time to consider the impact on existing user plane protocol after introducing a new container, we prefer to reuse the existing frames in existing container in the last meeting of Rel-18.

PDU Set based handling in NG-RAN

Another issue debated in last meeting is how to exchange the capability of PDU Set QoS handling between different nodes in network. There are two options are provided:

Option 1: Explicit PDU Set handling Support indicator;

Option 2: Indicate a list of supported NGAP IE IDs from NG-RAN upon request from SMF;

Before the end of the meeting, companies reached a work assumption that taking Option 1 for XR in R18. As the SA2 CR already specified, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN node responses the SMF with an indication of whether the PDU Set QoS handling is supported in PDU Session Resource Setup/Modify procedure. Additionally, it mentions the NG-RAN provides the SMF with an indication of whehther the target NG-RAN node supports PDU Set based handling during the Xn/NG handover. The similar scenario is also applied in Rel-17 MBS by introducing the MBS Support Indicator IE. Compared with Option2, Option 1 seems to follow the instruction from SA2 and is easy to implement. From our point of view, we can turn this work assumption into agreement in this meeting. Meanwhile, followed by the conclusion in last meeting, a new PDU Set based handling support Indication IE indicating the PDU Set QoS handling is supported for the NG-RAN node should be introduced in the SMF related IEs over NGAP and XnAP.

Proposal 2: Turn the work assumption into agreement: Taking Opt1(Explicit PDU Set handling Support indicator) for XR in R18.

Non-homogeneous Scenario

After deciding the way of signaling the capability of PDU Set QoS handling, we need to discuss solutions on three possible cases described below to support non-homogeneous scenario:

Case1: NG-RAN node supports PDU set QoS parameters, and the corresponding QoS flow is setup successfully

Case2: NG-RAN node supports PDU set QoS parameters, but the corresponding QoS flow is setup as legacy QoS flow

Case3: NG-RAN node does not support PDU set QoS parameters, the corresponding QoS flow is setup as legacy QoS flow

Considering the QoS flow setup procedure, SMF initiates the PDU Session Resource Setup procedure to setup the PDU Set QoS flow. For Case1, as NG-RAN node supports PDU Set QoS parameters, it should include the PDU Set based handling support indicator after receiving the PDU session resource setup request message. For Case2, NG-RAN node supports PDU Set QoS parameters, but the corresponding QoS flow is setup as legacy QoS flow as a result of no enough resources to support PDU Set QoS parameters. From our view, SMF should be aware of whether the PDU Set QoS flow is successfully established or not, since SMF decides to configure UPF to activate/deactivate PDU Set identification and marking. Thus, another new indicator indicating whether the PDU Set QoS flow is successfully established or not should be introduced. For Case3, SMF is aware of NG-RAN node does not support PDU set QoS parameters through the absence of PDU Set based handling indicator and then configures UPF to deactivate PDU Set identification and marking.

Observation 2: SMF should be aware of whether the PDU Set QoS flow is successfully established or not, since SMF decides to configure UPF to activate/deactivate PDU Set identification and marking.

Observation 3: SMF is aware of NG-RAN node does not support PDU set QoS parameters through the absence of PDU Set based handling indicator.

Proposal 3: A new indicator indicating whether the PDU Set QoS flow is successfully established or not should be introduced.

Considered that the mobility case in non-homogeneous scenario, it is possible that source NG-RAN node or target NG-RAN may not support PDU Set QoS handling. Two cases should be analyzed separately.

- Souce NG-RAN node supports PDU Set QoS handling and target NG-RAN node does not support PDU Set QoS handling.

During Xn handover, source NG-RAN node supporting PDU Set QoS handling sends the PDU Set QoS Parameters in the HANDOVER REQUEST message. If target NG-RAN node does not support PDU Set QoS handling, PDU Set based handling support indication is not included in HANDOVER REQUEST ACKNOWLEDGEMENT message. Then, source NG-RAN node will forward legacy data packets without including PDU Set QoS parameters over Xn after it understands target NG-RAN node does not support PDU Set QoS handling. During NG handover, if target NG-RAN node does not support PDU Set QoS handling, PDU Set based handling support indication is not included in PATH SWITCH REQUEST message, which is similar with aforementioned case3. Likewise, SMF will initiates to configure UPF to deactivate PDU Set identification and marking.

Proposal 4: During Xn/NG handover, data packets without including PDU Set QoS parameters are sent to target NG-RAN node from source NG-RAN node/UPF if target NG-RAN node does not support PDU Set QoS handling.

- Souce NG-RAN node does not support PDU Set QoS handling and target NG-RAN node supports PDU Set QoS handling.

The source NG-RAN node will forward legacy data packets over Xn. Then, target NG-RAN node indicates its capability to SMF by including the PDU Set based handling support indicator in Path Switch Request Transfer IE. As receving the indication, SMF will initiate the PDU Session Resource Modify procedure, providing the PDU Set QoS parameters in the QoS flow to target NG-RAN node and configuring UPF to deactivate PDU Set identification and marking.

Proposal 5: The source NG-RAN node will forward legacy data packets over Xn if it does not support PDU Set QoS handling.

Conclusions

In this contribution, related to XR enhancement, we provide our understanding on the PDU Set handling. The following observations or proposals are listed below:

Observation 1: There are no NBC issues foreseen where the legacy NG-RAN node could handle the readable part and skip the unknown part of the frame through fetching the Extension Header Length in advance.

Observation 2: SMF should be aware of whether the PDU Set QoS flow is successfully established or not, since SMF decides to configure UPF to activate/deactivate PDU Set identification and marking.

Observation 3: SMF is aware of NG-RAN node does not support PDU set QoS parameters through the absence of PDU Set based handling indicator.

Proposal 1: Due to the limit time to consider the impact on existing user plane protocol after introducing a new container, we prefer to reuse the existing frames in existing container in the last meeting of Rel-18.

Proposal 2: Turn the work assumption into agreement: Taking Opt1(Explicit PDU Set handling Support indicator) for XR in R18.

Proposal 3: A new indicator indicating whether the PDU Set QoS flow is successfully established or not should be introduced.

Proposal 4: During Xn/NG handover, data packets without including PDU Set QoS parameters are sent to target NG-RAN node from source NG-RAN node/UPF if target NG-RAN node does not support PDU Set QoS handling.

Proposal 5: The source NG-RAN node will forward legacy data packets over Xn if it does not support PDU Set QoS handling.

References

- S2-2308246, PDU Set QoS handling: non-homogenous support;