3GPP TSG-RAN3 Meeting #122	R3-237718

Chicago, USA, 13th – 17th November, 2023

Title: 	Further discussions on the support of QoE measurement for MBS service

Source: 	Huawei

Agenda item:	11.2

Document Type:	discussion and decision

1. Introduction

With only two meetings left, this contribution mainly focuses on the remaining issues regarding MBS QoE and RRC_INACTIVE/RRC_IDLE states.

2. Discussion

2.1 UE based or CN based

In the last meeting, RAN3 has the following work assumption on how the new gNB knows the QoE configuration information.

WA: UE based solution for IDLE QoE configuration retrieve in Rel-18 IDLE/INACTIVE QoE.

RAN3 also receives the reply LSs from RAN2 (reply LS from SA2 is still pending). RAN2 agreed it is feasible for gNB to store and retrieve IDLE/INACTIVE QoE configurations via UE based solution and agreed UE to transmit session status indication to the new gNB when UE moves from RRC_IDLE/INACTIVE to RRC_CONNECTED.

Technically it is also feasible for CN to store IDLE/INACTIVE QoE configurations, but we think CN does not know the latest session status indication because the session status indication may change when UE is in RRC_IDLE/INACTIVE. Also, considering the timeline, we propose to turn the work assumption to agreement.

- Turn WA to agreement: UE based solution for IDLE QoE configuration retrieve in Rel-18 IDLE/INACTIVE QoE

2.2 Whether the new communication service mode IE also contains a third codepoint - for unicast

RAN3 has agreed to introduce “multicast” and “broadcast” in XnAP and NGAP for QMC configuration. The remaining issue is whether to introduce a “unicast” indication for MBS QoE configuration.

In our understanding, if this MBS service type is “unicast”, there is no need to indicate to NG-RAN node or UE, since the whole procedure is based on legacy PDU session setup, and QoE measurement behaviour also follows the legacy way, i.e. just like R17 QoE behaviour. With this understanding, we think there is no need to introduce the third point of “unicast”.

- There is no need to introduce the third point of “unicast”.

On the other hand, in real practice there is a case that the MBS service type might be switched from multicast to unicast, if this case happens, it would raise a further question that whether the ongoing QoE measurement for multicast should be discarded or not. While the answer should be straightforward, the ongoing QoE measurement should be continued, since from the application layer point of view, the MBS service is still ongoing.

In addition, according to the previous replies from SA4, SA4 think the QoE results are useful only if all the QoE results during the service session are collected. Therefore, it is better that the network continues to collect the QoE results for the same service, i.e. continuous collection during the whole session time of the service carried by MBS regardless MBS’s service type.

- The ongoing QoE measurement for multicast service should be continued if the MBS service type is switched from multicast to unicast.

With proposal 3, we think there is a need to introduce a new indication to command the NG-RAN node to continue the QoE measurement when switching happens, and we think the indication is included only if the multicast indication is included in MBS QMC configuration.

- Introduce another IE to indicate that the multicast QoE measurement should be continued if switched to unicast.

2.3. Whether communication mode is indicated in QoE report upon switching between unicast and multicast

In our understanding, the motivation of this issue is to know the if the communication mode corresponding to a QoE result has been switched or not. On one hand, we think MCE only cares about the final QoE results and does not care about the communication mode, on the other hand, even if operators want to know the communication mode corresponding to QoE results, we think operators can know it via the MDT alignment info. The switching between unicast and multicast only happens when UE moves between multicast-supporting cell and multicast non-supporting cell. Therefore, operators can know whether UE moves between multicast-supporting cell and multicast non-supporting cell based on the cell id in the immediate MDT results.

In addition, according to the specification in SA4, UE reports QoE results periodically or upon the end of service session. The communication mode may change during the duration corresponding to one QoE result. It means each QoE result may correspond to more than one switch of communication mode, which also leads to another issue that  it might be difficult to indicate the communication mode correctly in the QoE report, with simple switch indication.

- No need to introduce the communication mode in the QoE report.

2.4 Whether MDT alignment indication is needed in new gNB when UE returns from RRC_IDLE to RRC_CONNECTED

For the alignment for the MBS broadcast QoE measurement in RRC_IDLE and RRC_INACTIVE, RAN3 has not discussed how to perform the alignment. With only two meetings left, we propose that RAN3 postpone the discussion of the alignment of MBS broadcast QoE measurement and logged MDT. Therefore we think logged MDT alignment indication is not needed in this release.

For the alignment for MBS multicast QoE measurement in RRC_CONNECTED, UE and the old gNB will release the QoE measurement configuration and immediate MDT configuration. The new gNB will configure the new MBS multicast QoE measurement when UE returns from RRC_IDLE to RRC_CONNECTED based on the QMC configuration from OAM/CN if received. If the alignment is needed, OAM/CN can send the immediate MDT alignment indication to the new gNB in the QMC configuration. Therefore the new gNB does not need to retrieve the MDT alignment indication from UE or the old gNB.

- New gNB does not need to retrieve MDT alignment indication from UE or the old gNB when UE returns from RRC_IDLE to RRC_CONNECTED

- A simple TP to 38.413 reflecting the proposal 3&4 could be seen in [2].

3. Conclusion

Based on the discussion in this paper, we propose the following observations and proposals:

- Turn WA to agreement: UE based solution for IDLE QoE configuration retrieve in Rel-18 IDLE/INACTIVE QoE

- There is no need to introduce the third point of “unicast”.

- The ongoing QoE measurement for multicast service should be continued if the MBS service type is switched from multicast to unicast.

- Introduce another IE to indicate that the multicast QoE measurement should be continued if switched to unicast.

- No need to introduce the communication mode in the QoE report.

- New gNB does not need to retrieve MDT alignment indication from UE or the old gNB when UE returns from RRC_IDLE to RRC_CONNECTED

4. Reference

[1]	R2-2311409, Reply LS on QMC support in RRC_IDLE and RRC_INACTIVE, RAN2

[2]	R3-237719, (TP to BL CR of 38.413) on support of QoE measurement continuity in unicast for MBS service, Huawei, China Telecom, China Unicom