3GPP TSG-RAN WG3 #122                                                                               R3-237746

Chicago, USA, 13th – 17th Nov 2023

Title: 	Summary of offline discussion on CB: # 5_UserConsent

Source: 	Huawei

Agenda item:	8.3

Document Type:	discussion and decision

1. Introduction

This contribution tries to capture the offline discussion on user consent:

CB: # 5_UserConsent

- Discuss the open issues above

- Capture agreements and provide reply LS

(moderator - HW)

Summary of offline disc R3-237746

2	For the Chairman’s Notes

- for signalling based MDT measurement, CN will perform the checking behavior, which means OAM also needs to configure AMF according to local regulations

- for management based MDT measurement, RAN node will perform the checking behavior. In case no MDT PLMN list(use consent) is received from AMF or OAM, RAN node should derive a list and configure to UE.

Reply LS: R3-237480 revised in R3-237953

CR to 38.401: R3-237725 revised in: R3-237947

CR to 37.320: R3-237727 revised in: R3-237948

3. Discussion

3.1 For signaling based MDT measurement

CN to check the OAM configuration, and to trigger Trace Activation towards the RAN in case of:

- Signalling based MDT measurements require user consent and user consent is available

- Signalling based MDT measurements do not require user consent

In case of 2) discuss whether the AMF signals the Signalling Based MDT PMN List to the RAN

Companies’ view:

| Company | Comment |
| --- | --- |
| Huawei | Technically possible, since user consent is normally applicable to all PLMNs, and if a UE says user consent No to all the PLMNs, the PLMN list actually should be empty, while local regulation may requires no user consent to PLMN_a for some MDT measurement data types, then the MDT PLMN list could include PLMN_a for these MDT measurement data types. However, such behaviour is transparent/invisible to RAN. |
| ZTE | For S-based MDT, it seems not necessary for CN to check OAM configuration. Based on legacy specification, CN provides MDT configuration to gNB. If local regulation allowed measurements, the gNB triggers the MDT session. If not, the gNB limits measurements subject to local regulation and trigger MDT session, or the gNB fails the MDT session. |
| Ericsson | I have modified the question above to reflect what discussed online. According to TS38.401: “The AMF starts a trace session and sends a TRACE START message to the gNB. The AMF shall consider the MDT user consent information when activating an MDT trace session for the UE as defined in TS 32.422 [20].” Therefore, it is the AMF that performs user consent check for Signalling Based MDT. With this understanding, we agree that the OAM needs to configure the AMF with a list of MDT measurements that are subject to user consent. The AMF needs to check the Signalling Based MDT measurements against the list provided by OAM and the AMF has to decide whether the Trace Activation can be triggered towards the RAN, based on whether user consent is available for measurements requiring it. We do not think the AMF should modify the MDT PLMN List. The MDT PLMN List for the UE is retrieved from the UDM and it is independent of user consent. Therefore there is no reason for the AMF to manipulate this parameter, which is decided by the operator to indicate where MDT should be triggered. In case the Signalling Based MDT configuration does not contain any measurement that requires user consent, the AMF can trigger Trace Activation towards the RAN for the UE. The AMF shall include the Signaling Based MDT PLMN List in the Trace Activation, in order to give to the RAN information on how to configure the plmn-IdentityList over RRC. |
| CMCC | For signalling based MDT, it is reasonable for CN checking if user consent is required for a specific type of data configured by OAM. If the s-based MDT measurements require user consent and user consent is available, AMF will directly send the PLMN list to NR-RAN node in the MDT activation message. If the s-based MDT measurements does not require user consent, we think it is fine to send the legacy PLMN list from AMF to NG-RAN, but some stage2 specification description needs to be added to regulate the RAN behaviour since some PLMN may not in the legacy PLMN list where measurement can be collected without checking user consent. |
| Qualcomm | Agree that this OAM configuration can be provided to CN and checked by CN. But we should check with SA2 which CN entity checks the OAM configuration (it could be AMF, or it could be the UDM as well). |
| Nokia | Agree with 1 and 2 above. In case of 2, the RAN should generate the Signalling Based MDT PLMN List corresponding to the list of equivalent PLMNs and use it for propagation of immediate MDT configuration over the Xn interface in case of immediate MDT or for sending to the UE (MDT PLMN List) in case of logged MD. |
| Samsung | Agree CN to check if the user consent is required or not. We think the proper node is the AMF. In case the user consent is not applied to the specific MDT measurement, since the signalling PLMN list need to be sent to the UE and we don’t want to change UE behaviour. therefore the AMF can set the signalling MDT PLMN list equal to ePLMN list. Then that means the MDT can be measure in all PLMNs and there is no UE consent control. |

3.2 For management-based MDT measurement

How does RAN node configure UE if there is no MDT PLMN list(use consent) received from AMF or OAM?

During online discussion, it seems that there are two options:

- As mentioned above, RAN node may generate an MDT PLMN list and configure this list to UE

- RAN node just ignore the concerned UE, i.e. not select this UE for MDT measurement

Here we assume that for signalling based, anyway AMF should give an MDT PLMN list, so the scenario here mainly points to management-based MDT measurement.

| Company | Comment |
| --- | --- |
| Huawei | Either way is fine, we think both options could work; for the first option, additional behaviour for RAN node is needed but it is RAN node internal behaviour, we are not sure if there is a need to capture anything; for the second option, it is a simpler way and also RAN node implementation. |
| ZTE | We prefer the second one (i.e. ignore the concerned UE), this solution is the safest way for gNB to select UE for logged MDT. I checked the TS32.422 quoted by R3-237477 and find out the description is cover how to use MDT PLMN list in RAN, but there is no specific definition of how to provide MDT PLMN list from gNB to UE. In turn , I checked TS37.320, and it clearly defined that the MDT PLMN list to the UE should be either Management Based MDT PLMN list. The information comes from AMF so it seems not allowed for gNB to generate the information to the UE. TS 37.320 5.1.1.1.1 Configuration parameters ---omit - (optionally) MDT PLMN List, indicating the PLMNs where measurement collection and log reporting is allowed. It is either the Management Based MDT PLMN List or the Signalling Based MDT PLMN List, depending on how the Logged MDT task was initiated (see 5.1.3). |
| Ericsson | The second option is not in line with the solution from SA3 because it implies that if a user did not give user consent, the user cannot be selected for Management Based MDT even if the measurements do not require user consent. Therefore, the only valid option is that the RAN derives the plmn-IdentityList to signal to the UE over RRC from the MDT PLMN List received as part of the Management Based MDT configuration. Section 4.1.1.9.2 of TS32.422 mentions that the Management Based MDT Activation received by the OAM includes: The management system sends a Trace Session activation request to the gNB. This request includes the parameters for configuring UE measurements: - Job type. […] - MDT PLMN List […] At the same time, TS32.422 states the following: 5.10.24 MDT PLMN List This is an optional parameter indicating the PLMNs where measurement collection, status indication and log reporting is allowed. E.g. the UE performs these actions for Logged MDT when the RPLMN is part of this set of PLMNs. Maximum of 16 PLMNs can be defined. To the UE it is communicated as the plmn-IdentityList. Hence using the MDT PLMN List in the Management Based MDT configuration is in line with the specifications. |
| CMCC | We think that both solutions are workable. For Option 1, like Ericsson said, it is inline with the SA3 reply LS, RAN derives a new plmn-IdentityList to signal to the UE over RRC from the MDT PLMN List received as part of the Management Based MDT configuration. For Option 2, people in RAN3 should note that this case must be a corner case. If there is no MDT PLMN list received from AMF, that means user does not give consent for the MDT collection to all PLMNs. Based on the implementation, RAN may not select the concerned UE. Option 2 is workable with special treatment regard to the corner case. |
| InterDigital | Agree with Ericsson that the second option is not inline with the SA3 decisions, it may be safer but for example use in AI/ML will be more important since it is a tool that works to provide measurement for all purposes, and it could severely constrain AI/ML in areas requiring user consent, and the SA3 decision is correct for AI/ML anyway |
| Qualcomm | Option 2 is not appropriate as commented by few other companies above. Regarding Option 1, RAN “generating” the MDT PLMN list is not probably the right wording, but we agree in principle. In particular, we think the section 4.9.2 from TS 32.422 would be impacted (e.g., the following sentences) and we need to check with SA5 to modify this: The AMF shall also check the roaming status of the user. If the user is within his home operator’s PLMNs and the user has given his consent, the AMF shall send the Management based MDT PLMN List IE to the gNB during the UE context setup procedure. Otherwise the AMF shall not send the Management based MDT PLMN List IE to the gNB. |
| Nokia | It seems reasonable that the RAN node will use the MDT PLMN list provided with the MDT configuration if user consent is not required, and configure this list to UE. We agree that option 2 is not appropriate. |
| Samsung | If no management based MDT PLMN List is received for a UE and OAM configure a MDT measure needn’t user consent, we think the NG-RAN can select this UE. It is the NG-RAN decision. In order not to introduce any change in Uu, the NG-RAN can generating the MDT PLMN list sending to the UE. |

3.3 Any clarifications needed for logged MDT measurement reporting

The RAN node determines the logged MDT measurement reports route to TCE are subject to user consent by means of OAM configuration?

| Company | Comment |
| --- | --- |
| Huawei | Not sure if there are any issues here. In our understanding, as long as the report is sent to RAN node, this report is anyway collected within valid/legal area, the receiving RAN node just needs to forward to corresponding TCE, that’s all. |
| ZTE | We assume only RAN node do the OAM local regulation checking. Then it is possible the Node triggers logged MDT may have different regulation in the Node receives logged MDT reports. Then it is possible the reports of certain measurements are not allowed. |
| CMCC | There is no issue foreseen on this statement. RAN node only needs to check whether the measurement needs to be subject to user consent configured by OAM. If some measurements are not subject to the user consent, RAN can collect and report it even though the user does not give consent for the measurement. |
| Qualcomm | Similar HW as others above. Don’t think RAN node need to check OAM configuration when sending logged MDT reports to TCE. |
| Samsung | Same view as HW. Reporting is already controlled by the UE, the RAN node just forward it to the TCE. |

3.3 Stage 2 updates

CR to 37.320

=> The main open issue here is, whether to update the definition of MDT PLMN list or not.

CR to 38.401

=> The main open issue here is, whether to capture AMF behaviour explicitly in 38.401 or not.

| Company | Comment |
| --- | --- |
| Huawei | For 37.320, we don’t think there is a need to touch MDT PLMN list definition, what we need to is to have clarifications for contexts related with MDT PLMN list, e.g. in [3] [6] [11]. For 38.401, no strong opinion, some simply clarifications on AMF behaviour could be considered, e.g. some simplified description in [4] or just take [9] as baseline. |
| ZTE | We are open for updates and think it is better to wait until common understanding achieved |
| CMCC | For 37.320, we think that it is more appropriate to regulate the NG-RAN behaviour by adding the notes in specification rather than changing the definition of MDT PLMN list. For 38.401, no strong view, open for discussion. |
| Qualcomm | We can use HW’s draft CRs to 37.320 and 38.401 as baseline and work on them. We can LS SA2 to check impact on 5GC as well. |
| Samsung | Open to discuss. |

4. Conclusion

Based on the discussion in this paper, we propose the following:

5. Reference

- R3-237425, User Consent Principles (InterDigital, Ericsson)

- R3-237477, Discussion on user consent for trace reporting (Ericsson, InterDigital)

- R3-237478, Correction of user consent for MDT (Ericsson, InterDigital)

- R3-237479, Correction to 38.401 on the user consent for trace reporting (Ericsson, InterDigital)

- R3-237480, [draft] Reply LS on the user consent for trace reporting (Ericsson)

- R3-237549, Correction of user consent for MDT (Nokia, Nokia Shanghai Bell)

- R3-237681, Discussion on user consent for trace reporting (CMCC)

- R3-237724, Further discussions on the user consent for trace reporting (Huawei)

- R3-237725, Correction to 38.401 on the user consent for trace reporting (Huawei, CMCC)

- R3-237726, [draft] Reply LS to R3-234493 (S3-234267) on the user consent for trace reporting (Huawei)

- R3-237727, Correction to 37.320 on the user consent for trace reporting (Huawei, CMCC)

- R3-237604, Discuss user consent for trace reporting (ZTE)

- R3-237605, Correction of user consent for MDT (ZTE)