3GPP TSG RAN WG3#122	R3-238029

Chicago, IL, USA, November 13 – 17,2023

Title:	LS on RAN3 agreements on mobile IAB

Response to:

Release:	Rel-18

Work Item:	NR_mobile_IAB-Core

Source:	ZTE

To:	SA2

Cc:

Contact person:

Name:	Ying Huang

Tel. Number:

E-mail Address:	huang.ying11@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R3-237430, R3-238021

1. Overall Description:

RAN3 reached following consensus that may affect SA2 specifications:

- TAC/RANAC: The mobile IAB-DU’s TAC/RANAC is only configured by the OAM.

- N2 based handover of mobile IAB with no PDU session activated: in some cases, a mobile IAB may have no PDU sessions activated. In these cases, RAN3 agreed that, during the N2 handover preparation procedure, the AMF shall ignore the PDU Session Resource List IE, as captured in the TP to 38.413 BL CR R3-237430.

- The IAB-donor-CU serving the mobile IAB-MT sends an NGAP indication to AMF in the NGAP UE CONTEXT RELEASE REQUEST message to indicate that the IAB MT can be deregistered and the indication to be captured via a new cause value.”, as captured in the TP to 38.413 BL CR R3-238021.

2. Actions:

To SA2 group.

ACTION: 	RAN3 kindly asks SA2 to take the above RAN3 decision into account and take corresponding action if needed.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting	 #123, Feb 26 to March 1, 2024       Athens, Greece

TSG-RAN3 Meeting	 #123bis, April 15 to 19, 2024           PRC