3GPP TSG-RAN WG3 Meeting #122	R3-238018

Chicago, USA, 13 - 17 November 2023

Agenda Item:	14.2

Source:	Huawei, Nokia, Nokia Shanghai Bell, NEC, Google, ZTE, Samsung, CATT, Ericsson

Title:	(TP for L1L2Mob BLCR for TS 38.401): LTM procedure update

Document for:	Other

Introduction

This document contains the TP to LTM BLCR to 38.401.

TS 38.401 TP

#### 8.2.1.X	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.x-1: Intra-gNB-DU LTM

- The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing  measurements of neighbouring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate LTM configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing one target candidate cell ID, the LTM configuration ID of the candidate cell, and LTM configuration ID mapping list, the CSI Resource Configuration. The gNB-CU requests PRACH resources from the gNB-DU. The gNB-CU may request the gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration.

- If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations  (e.g., TCI state configuration, RACH configuration, and the CSI Report Configuration for the accepted target candidate cell.

NOTE: Steps 3 and 4 may be initiated multiple times for LTM candidate cell preparation of multiple cells including the source cell.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the collected TCI state configurations and CU may send the CSI resource Configuration for the accepted target candidate cells.

- The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes the generated CSI report configuration of the source cell.

NOTE: In case of subsequent LTM, UE CONTEXT MODIFICATION procedure may be invoked per candidate cell to transfer to the candidate gNB-DU the CSI resource Configuration, TCI state information, RACH Configuration, and the LTM configuration IDs of the candidate cells.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- Early synchronization is performed as specified in TS 38.300 [2].

- The UE sends the L1 measurement result to the gNB-DU. The gNB-DU decides to execute LTM.

- The gNB-DU sends the Cell Switch command to the UE.

- The gNB-DU sends the UL CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the TCI state ID.

- The target gNB-DU detects the UE access as specified in TS 38.300 [2].

- The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target Cell ID.

- The UE sends an RRCReconfigurationComplete message to the gNB-DU.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

---------------------------------------------------------Next change -------------------------------------------------------------

#### 8.2.1.Y	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.Y-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.Y-1: Inter gNB-DU LTM

1. The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing  measurements of neighboring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2. The gNB-CU determines to initiate LTM configuration.

3. The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing one target candidate cell ID, the LTM configuration ID of the candidate cell, LTM configuration ID mapping list, and the CSI Resource Configuration. The gNB-CU indicates the source gNB-DU ID, and requests PRACH resources from the Candidate gNB-DU. The gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration.

4. If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations (e.g., TCI state configuration and RACH configuration) and the CSI report configuration for the accepted target candidate cell.

NOTE: UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.X Intra-gNB-DU LTM.

5. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the CSI resource configuration, TCI state configuration, RACH configuration, and the TCI state configuration for the accepted target candidate cell(s) in other gNB-DUs.

6. The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes the generated CSI report configuration.

7. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing TCI state information, RACH Configuration, and the LTM configuration IDs of the candidate cells in other candidate gNB-DU(s),. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may also provide an updated CSI Resource Configuration to the candidate gNB-DU(s).

NOTE: The candidate cell may be the same cell as source cell.

8. The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration. The candidate gNB-DU may also respond the updated CSI Report Configuration.

9. The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10. The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11. The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12. The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13. Early synchronization is performed as specified in TS 38.300 [2].

14 - 15. The candidate gNB-DU sends the TA, the associated CFRA resource information, the candidate cell ID and the source gNB-DU ID to the source gNB-DU via DU-CU TA INFORMATION TRANSFER and CU-DU TA INFORMATION TRANSFER messages.

16. The UE sends the L1 measurement result to the source gNB-DU.

17. The source gNB-DU decides to execute LTM to a candidate target cell.

18. The source gNB-DU sends the Cell Switch command to the UE.

19. The source gNB-DU sends the UL CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the LTM command to the UE including the target cell ID and the TCI state ID.

20. The gNB-CU sends the target cell ID and the TCI state ID to the target gNB-DU in the DL CELL CHANGE NOTIFICATION message.

21. The target gNB-DU detects the UE access as specified in TS 38.300 [2].

22. The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

23.  The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

- The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

25. The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

26. The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

---------------------------------------------------------next change -------------------------------------------------------------