3GPP TSG-RAN3 Meeting #122	R3-23xxxx

# Chicago, USA, 13th – 17th November, 2023

Agenda item:	25.2.1

Source:	Nokia, Nokia Shanghai Bell, Xiaomi, Ericsson, Qualcomm Inc., Samsung, China Telecom

Title:		(TP for TS 38.473 BL CR) Introduction of Direction Information for PDU Set QoS Parameters

Document for:	Discussion and Decision

# 1 Introduction

This TP proposes to define separate DL and UL PDU Set QoS parameters by adding a Direction Information IE.

Start of Change

#### 9.3.1.x	PDU Set QoS Parameters

This IE defines the PDU Set QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set Delay Budget | O |  | Extended Packet Delay Budget 9.3.1.145 | PDU Set Delay Budget as defined in TS 23.501 [21]. |
| PDU Set Error Rate | O |  | Packet Error Rate 9.3.1.52 | PDU Set Error Rate as defined in TS 23.501 [21]. |
| PDU Set Integrated Handling Information | O |  | ENUMERATED(true, false, …) | PDU Set Integrated Handling Information as defined in TS 23.501 [21]. |
| Direction Information | O |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) | Direction Information for the PDU Set QoS Parameters. |

Next Change

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

** Unchanged text skipped **

PRSTransmissionTRPItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDUSetQoSParameters	::= SEQUENCE {

pduSetDelayBudget						ExtendedPacketDelayBudget											OPTIONAL,

pduSetErrorRate							PacketErrorRate														OPTIONAL,

pduSetIntegratedHandlingInformation		ENUMERATED {true, false, ...}										OPTIONAL,

directionInformation					ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}  	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { PDUSetQoSParameters-ExtIEs } }	OPTIONAL

}

PDUSetQoSParameters-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

End of Change