3GPP TSG-RAN WG3 #122                                                                               R3-23XXXX

Chicago, USA, 13th – 17th Nov 2023

Title: 	Summary of offline discussion on CB on QoE others in 11.4

Source: 	Huawei

Agenda item:	8.3

Document Type:	discussion and decision

1. Introduction

This contribution tries to capture the offline discussion on QoE others in 11.4

2	For the Chairman’s Notes

3. Discussion

3.1 QoE measurement status indication to target node during CHO

According to the contributions in [1] and [2], there are mainly two options.

- UE-based solution

- Network-based solution

Companies’ view:

| Company | Comment |
| --- | --- |
| Huawei | We think UE-based approach should be enough, it is simple and re-use existing UE behaviour, then we may need an LS to RAN2 indicating RAN3 conclusion |

3.2 Inter-RAT mobility

According to the contributions, we could summarize the following issues:

- Clarifications to CGI, indicating either NR CGI or E-UTRA CGI in 38.413 [6]

- Release configured QoE measurement configurations during HO from NR to LTE [4]

- Reuse the legacy QoE configuration release procedure or,

- The UE release all the NR QoE configurations when it receives MobilityFromNRComman

- Stage 2 clarifications or stage 3 clarifications or both

Stage 2: 38.300 [7], stage 3: 38.413/38.423. [1]

| Company | Comment |
| --- | --- |
| Huawei | For 1), we think the clarification is correct, the CGI could indicate either NR or E-UTRA; For 2), we think the reuse of existing procedure is a simpler approach; For 3), at least stage 2 clarifications to 38.300 is anyway needed; for clarifications to stage 3 38.423 and 38.413, we think there is no need, it is network implementation to follow the restriction which follows the general instructions captured in stage 2. |

3.3 Further additions to deactivation of RVQoE over F1

Any further stage 2 clarifications needed in 38.401, and stage 3 in 38.473 [5]?

| Company | Comment |
| --- | --- |
| Huawei | Seems not needed. For stage 2 to 38.401, seems not needed at all, anyway, deactivation purpose is clear, the added texts mainly touch stage 3 details. |

4. Conclusion

Based on the discussion in this paper, we propose the following:

5. Reference

- R3-237181, (TPs for QoE BL CRs for TS 38.413 and TS 38.423) Enhancements of Rel-17 QoE and RVQoE Features (Ericsson)

- R3-237722, Further discussion on remaining open issues and leftovers (Huawei)	discussion

- R3-237658, Further discussion on other issues (ZTE, China Telecom, China Unicom, CMCC)	discussion

- R3-237243, Discussion on intra-5GC Inter-RAT handover (NEC)	discussion

- R3-237659, (TPs for BL CR of TS38.401&38.473) Deactivation of RVQoE reporting over F1AP (ZTE, China Telecom, China Unicom)	other

- R3-237717, (TP to BL CR of 38.413) Inter-RAT mobility for QoE measurement (China Unicom)	other

- R3-237723, (TP to BL CR of 38.300) on intra-system inter-RAT handover and CHO for QoE measurement (Huawei, China Telecom, China Unicom)	other