3GPP TSG-RAN WG3 #122							R3-237869

Chicago, USA, 13th – 17th Nov, 2023

Agenda Item:	15.3

Source:	CATT

Title:	(TP to BLCR for TS 38.470) Support of MBS reception in RRC_Inactive state

Document for:	Approval

- Introduction

According to the following agreements in R3-237895, we provide TP to BLCR for TS 38.470.

To introduce SIBx in the gNB-DU System Information IE.

Introduce a new F1AP procedure to deliver IEs including  MBS-NeighbourCellList IE , thresholdMBS-List IE, RRC Multicast MTCH Neighbour Cell Information IE and ThresholdIndex IE

Explicit indication is introduced to indicate the multicast RRC Inactive reception mode activation/deactivation.

Explicit indication is introduced to indicate stop of broadcasting  RRC Multicast MTCH Neighbour Cell Information IE and ThresholdIndex IE

Removal Editor’s note on MBS Multicast Configuration Response Information IE

Replace the current Editor’s Note on Indication for Multicast RRC_INACTIVE Reception IE with the semantic description “Corresponds to information contained the inactiveReceptionAllowed as specified in TS 38.331 [8].”

- TP to 38.470

- 5.2.2	System Information management function

Scheduling of system broadcast information is carried out in the gNB-DU. The gNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

The gNB-DU is responsible for the encoding of the NR-MIB message. In case broadcast of SIB1 and other SIBs is needed, the gNB-DU is responsible for the encoding of the SIB1 message, SIB10, SIB12, SIB13, SIB14, SIB15, SIB17, SIB18, SIB20 and SIBX, and the gNB-CU is responsible for the encoding of other SIBs. The gNB-DU may re-encode SIB9. The gNB-DU is responsible for the generation of the SystemInformation message.

NOTE:	The SIB19 is generated by the gNB-DU.

The gNB-CU is responsible for receiving the positioning assistance information from LMF, e.g the positioning related SIBs. The gNB-CU transparently sends the positioning assistance information to the gNB-DU. The gNB-DU is responsible for broadcasting the positioning assistance information in Positioning SI message(s).

To support Msg3 based on-demand SI and RRC Dedicated SIB Request as described in TS 38.331 [11], the gNB-CU can confirm the received SI request from the UE by including the UE identity, and command the gNB-DU to broadcast the requested SystemInformation messages including the other SI.

To support UE RRC Positioning SI acquisition mechanism, as described in TS 38.331 [11], the gNB-CU can confirm the received positioning SI request from the UE by including the UE identity, and command the gNB-DU to broadcast the requested positioning SI messages.

-----------------Next Change-------------------

- 6.1.13	NR MBS procedures

The F1 MBS procedures are listed below:

-	Broadcast Context Setup;

Broadcast Context Release (gNB-CU initiated);

-	Broadcast Context Modification;

-	Broadcast Context Release Request (gNB-DU initiated);

-	Multicast Group Paging procedure;

-	Multicast Context Setup;

-	Multicast Context Release (gNB-CU initiated);

-	Multicast Context Modification;

-	Multicast Context Release Request (gNB-DU initiated);

-	Multicast Distribution Setup;

-	Multicast Distribution Release;

-	Multicast Context Notification;

-    Multicast Common Configuration.

-----------------End of the Change-------------------