3GPP TSG-RAN WG3 Meeting #123-bis	R3-241840

Changsha, China, 15 – 19 April 2024

Agenda item:	17.2

Source:	Nokia

Title:	Considerations on On-Demand SSB SCell operation for UEs in connected mode

Document for:	Discussion

# 1	Introduction

In this paper we provide an initial analysis of potential RAN3 impacts of on-demand SSB SCell operation for UEs in connected mode based on the current discussion state in RAN1 [1, 2].

# 2	Discussion

SSB-less operation for SCell is supported for certain NR Carrier Aggregation (CA) scenarios from Rel-15 (i.e. for intra-band contiguous scenarios). In this case the UE will use SSB transmitted by the PCell for coarse synchronisation in frequency/time domain with the SSB-less SCell, if some conditions are valid (e.g. Receive Time Difference (RTD) and reception power difference is within the thresholds defined in RAN4). On-demand SSB SCell operation, as targeted for the present Rel-19 work, is expected to enhance the flexibility of SSB-less SCell operation by transmitting on-demand SSB only when required for specific UEs. RAN1 therefore is discussing on-demand SSB operation for the following CA scenarios:

- Contiguous Intra-band CA scenario

- Non-Contiguous Intra-band CA scenario

- Inter-band CA scenario

RAN1#116 also identified the following two cases for the of-demand SSB operation:

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

In practice, the network may not be able to configure a cell as SSB-less SCell for all UEs (i.e. case#1 may not be always feasible). As presented in Figure 1, Cell 3 could be configured to be SSB-less and UEs could use SSB from Cell 2 to obtain time and frequency synchronization. However, if Cell 2 is fully occupied and a new UE that only supports 2 CC CA and does not support inter-band SSB-less SCell, needs to be configured for CA operation then Cells 1 and 3 need to be used. The on-demand SSB could then be beneficial in Cell 3.

Cell 2 Cell 1 Pcell SSB f1 f2 No SSB SSB Inter-band Band 1 Band 2 f2 f1 Cell 3 Intra-band contiguous f3 f3 Figure-1: Example scenario with and without SSB

We further notice the following agreement from RAN1#116:

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Our current understanding is that network triggering of SSB transmission will be decided by the gNB-DU in the discussed scenarios. Inter-node exchange of e.g. MTC (Measurement Timing Coordination) information could in principle be impacted, but this information is already encoded in RRC inter-node message for exchange over F1 and Xn interfaces.

No RAN3 impact is therefore currently foreseen for on-demand SSB SCell operation for UEs in connected mode, but RAN3 should further monitor further agreements taken by RAN1 and RAN2 for this topic.

Observation: No RAN3 impact is currently foreseen for on-demand SSB SCell operation for UEs in connected mode.

Proposal: RAN3 to monitor further agreements taken by RAN1 and RAN2 for on-demand SSB SCell operation for UEs in connected mode.

# 3	Conclusions

We have made the following proposal:

Observation: No RAN3 impact is currently foreseen for on-demand SSB SCell operation for UEs in connected mode.

Proposal: RAN3 to monitor further agreements taken by RAN1 and RAN2 for on-demand SSB SCell operation for UEs in connected mode.

# References

[1]		R1-2401985, On-demand SSB SCell Operation, Nokia, Nokia Shanghai Bell

[2]		R2-2403599, On-demand SSB SCell Operation, Nokia, Nokia Shanghai Bell