3GPP TSG-RAN WG3 Meeting #123bis	R3-242094

Changsha, China, April 15th – 19th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1409 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 LTM correction for UE-based TA measurement configuration from CU to DU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-18 Intra-CU LTM specified two mechanisms of pre-calculating UL TA values for faster RACH-less access to a candidate cell (called “early TA acquisition”): - NW obtains UL TA values of candidate cells before cell switch (by PDDCH-ordering the UE to initiate RACH) and configures the pre-acquired TA value to the UE in the LTM Cell Switch Command. - “UE-based TA measurements”, where CU configures the UE to derive UL TA on its own based on the Rx timing difference between the current serving cell and candidate cell, together with the current TA value. The second mechanism has been designed in a way that it is not activated unless the serving DU indicates so (set to “FFF”) via LTM Cell Switch Command (even though configured to the UE via RRC by CU). However, DU is currently obvilous of whether CU configured the UE-based TA measurements to the UE and for which cells. As a result, the serving DU (who is not aware) won’t be able to activate the UE based TA measurement at all. Instead, it will execute the first mechanism for a candidate cell (for which the UE-based TA measurement has already been configured) to provide the TA value in the LTM Cell Switch Command, where the UE has to unnecessarily follow the PDCCH order and initiate RACH to that candidate cell. Due to its cascaded design, the UE-based TA measurement feature may not work as intended, if such configuration from CU is not known to DU who controls early RACH and decides cell switch. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the UE CONTEXT MODIFICATION REQUEST > Early Sync Information List IE, added two OCTET STRINGs of - Candidate Cell UE based TA measurement Configuration IE (= ltm-UE-MeasuredTA-ID-r18 IE in TS 38.331) - Serving Cell UE based TA measurement Configuration IE (= ltm-ServingCellUE-MeasuredTA-ID-r18 IE in TS 38.331) so that CU can provide the UE-based TA measurement configured to the UE for the candiate cells and serving cell toward a DU. The corresponding descriptions and semantics are added. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on LTM early UL acquisition function between CU and DU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE-based TA measurement feature in Rel-18 LTM may not work as intended. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4.2, 9.2.2.7, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.4	UE Context Modification (gNB-CU initiated)

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.4.2	Successful Operation

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the Network Controlled Repeater Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its authorization information for the UE accordingly. If the Network Controlled Repeater Authorized IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing as a Network Controlled Repeater.

If the LTM Indicator IE set to "true" is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT MODIFICATION RESPONSE message.

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include the Reference Configuration Information IE in the LTM Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message to provide lower layer configuration for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the requested LTM candidate cell.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Request for RACH Configuration IE set to "true" is contained in the Early Sync Information Request IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for early TA acquisition, and include the Early UL Sync Configuration and/or Early UL Sync Configuration for SUL IE  in the UE CONTEXT MODIFICATION RESPONSE message.

If the Early Sync Information List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the Candidate Cell UE based TA measurement Configuration IE and/or the Serving Cell UE based TA measurement Configuration IE are contained in the Early Sync Information List IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account when deciding the early TA acquisition.

If the LTM Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the gNB-DU.

If the LTM Cells to be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the configured candidate cells in the list.

If the Complete Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete configuration.

If the Direct Path Addition IE is contained in the Path Addition Information IE which is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns the direct path addition for the included SpCell ID IE as specified in TS 38.401 [4] and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the Indirect Path Addition IE is contained in the Path Addition Information IE, the gNB-DU shall, if supported, consider that the request concerns the indirect path addition for the MP Remote UE using PC5 link and use it as specified in TS 38.401 [4]. If the N3C Indirect Path Addition IE is contained in the Path Addition Information IE, the gNB-DU shall, if supported, consider that the request concerns the indirect path addition for the MP Remote UE using N3C and use it as specified in TS 38.401 [4].

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.2.2	UE Context Management messages

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| ////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| Early Sync Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | 9.3.1.293 |  | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>Candidate Cell UE based TA measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID-r18 IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >Serving Cell UE based TA measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID-r18 IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | ignore |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying or L2 N3C relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N or U2U relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE may be present if the CHO Trigger IE is present and set to "CHO-cancel". |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.4.5	Information Element Definitions

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### -- E

EarlyULSyncConfig ::= SEQUENCE {

rACH 								RACHConfiguration,

lTMgNB-DU-IDs-PreambleIndexList		LTMgNB-DU-IDs-PreambleIndexList							OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { EarlyULSyncConfig-ExtIEs} } OPTIONAL,

...

}

EarlyULSyncConfig-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EarlySyncInformation-Request  ::= SEQUENCE {

requestforRACHConfiguration					RequestforRACHConfiguration									OPTIONAL,

lTMgNB-DU-IDsList							LTMgNB-DU-IDsList											OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { EarlySyncInformation-Request-ExtIEs} } OPTIONAL,

...

}

EarlySyncInformation-Request-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EarlySyncInformation  ::= SEQUENCE {

tCIStatesConfigurationsList		TCIStatesConfigurationsList,

earlyULSyncConfig				EarlyULSyncConfig									OPTIONAL,

earlyULSyncConfigSUL			EarlyULSyncConfig									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { EarlySyncInformation-ExtIEs} } OPTIONAL,

...

}

EarlySyncInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EarlySyncInformation-List ::= SEQUENCE {

EarlySyncInformation-CandidateCell-List			SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF EarlySyncInformation-CandidateCell-Item,

EarlySyncInformation-ServingCell				EarlySyncInformation-ServingCell-Item																OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { EarlySyncInformation-List-ExtIEs } }						OPTIONAL,

...

}

EarlySyncInformation-List-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

EarlySyncInformation-CandidateCell-Item ::= SEQUENCE {

nRCGI				NRCGI,

tCIStatesConfigurationsList		TCIStatesConfigurationsList							OPTIONAL,

earlyULSyncConfig				EarlyULSyncConfig									OPTIONAL,

earlyULSyncConfigSUL			EarlyULSyncConfig									OPTIONAL,

tAAssistanceInfo				TAAssistanceInfo									OPTIONAL,

uEbasedTAmeasurementConfig		OCTET STRING										OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { EarlySyncInformation-CandidateCell-Item-ExtIEs } }	OPTIONAL,

...

}

EarlySyncInformation-CandidateCell-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

EarlySyncInformation-ServingCell-Item ::= SEQUENCE {

uEbasedTAmeasurementConfig		OCTET STRING,

iE-Extensions					ProtocolExtensionContainer { { EarlySyncInformation-ServingCell-Item-ExtIEs } }	OPTIONAL,

...

}

EarlySyncInformation-ServingCell-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-MeasurementQuantities ::= SEQUENCE (SIZE (1.. maxnoofMeasE-CID)) OF ProtocolIE-SingleContainer { {E-CID-MeasurementQuantities-ItemIEs} }

E-CID-MeasurementQuantities-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-E-CID-MeasurementQuantities-Item	CRITICALITY reject	TYPE E-CID-MeasurementQuantities-Item		PRESENCE mandatory}

}

E-CID-MeasurementQuantities-Item ::= SEQUENCE {

e-CIDmeasurementQuantitiesValue				E-CID-MeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { E-CID-MeasurementQuantitiesValue-ExtIEs} } OPTIONAL

}

E-CID-MeasurementQuantitiesValue-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////