3GPP TSG-RAN WG3 Meeting #123bis 	R3-242173

# Changsha, China, 15 – 19 April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1394 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for LTM for reconfiguration with sync |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google, Inc, CATT, Nokia, Ericsson, LG Electronics, Huawei, China Telecom, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - RAN2 had concluded and captured in the TS 38.331 that the reconfigurationWithSync IE is reused for LTM (after RAN2#123). However, it is not clearly captured in the F1AP specification how the gNB-DU regards the request from the gNB-CU as a reconfiguration with sync when it is for LTM in F1AP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - UE context setup: - Add in the paragraph “If the LTM Indicator IE set to "true" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, …” that the gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331. - UE context modification: - Add in the paragraph “If the LTM Indicator IE set to "true" is contained in the LTM Information Setup IE included in the UE CONTEXT MODIFICATION REQUEST message, …” that the gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331. Imapct analysis: This CR has isolated impact on LTM procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LTM function is not fully supported and correctly captured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 8.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-241857 Rev1: Reword the changes in the UE Context Modification procedure; fix cover page issue; remove the changes overlapped with another agreed CR. |  |  |  |  |  |  |  |  |

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the servingCellMO List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, select servingCellMO after determining the list of BWPs for the UE and include the list of servingCellMOs that have been encoded in CellGroupConfig IE as ServingCellMO-encoded-in-CGC List IE in the UE CONTEXT SETUP RESPONSE message.

If the Configured BWP List IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, take it into account when requesting the gNB-DU for generating preconfigured measurement GAP for the indicated BWPs.

If the SpCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly.

If the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the DRX Cycle IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall use the provided value from the gNB-CU.

If the UL Configuration IE in DRB to Be Setup Item IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the SRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB. If the Additional Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup the indicated RLC entities for the indicated SRB. If the SDT RLC Bearer Configuration IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT SRB indicated by the SRB ID IE. If the SRB Mapping Info IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the SRB Mapping Info IE for the SRB identified by the SRB ID IE and the Uu Relay RLC channel identified by the SRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of SRB data to Uu Relay RLC channel.

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB. If the SDT RLC Bearer Configuration IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT DRB indicated by the DRB ID IE. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

If the BH RLC Channel To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Setup Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE as follows:

-	if the IP to layer2 Traffic Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the IP to layer2 Traffic Mapping Info To Add IE, if present, for the egress BH RLC channel identified by the BH RLC CH ID IE, and shall remove the previously stored mapping information as indicated by the IP to layer2 Mapping Traffic Info To Remove IE, if present. The gNB-DU shall use the mapping information stored for the mapping of IP traffic to layer 2, as specified in TS 38.340 [30].

-	if the BAP layer BH RLC channel Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the BAP layer BH RLC channel Mapping Info To Add IE, if present, for the egress or ingress BH RLC channel identified by the BH RLC CH ID IE, and shall remove the previously stored mapping information as indicated by the BAP layer BH RLC channel Mapping Info To Remove IE, if present. The gNB-DU shall use the mapping information stored when forwarding traffic on BAP sublayer, as specified in TS 38.340 [30].

If two UL UP TNL Information IEs are included in UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT SETUP RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT SETUP REQUEST message for a DRB, the gNB-DU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT SETUP RESPONSE message and setup one or two additional RLC entities for the indicated DRB. The gNB-CU and the gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2].

If Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activating/deactivating CA based PDCP duplication for the DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when activating/deactivating CA based PDCP duplication for the DRB with more than two RLC entities.

If DC Based Duplication Configured IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. If DC Based Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activating/deactivating DC based PDCP duplication for this DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT SETUP REQUEST message for a DRB, the gNB-DU shall, if supported, take it into account when activating/deactivating DC based PDCP duplication for the DRB with more than two RLC entities. If the Primary Path Indication IE is included in the RLC Duplication Information IE, the gNB-DU shall, if supported, take it into account when performing DC based PDCP duplication for the DRB with more than two RLC entities.

If UL PDCP SN length IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. If the Additional RRM Policy Index IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall, if supported, contain the Additional RRM Policy Index IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20]. The gNB-DU shall, if supported, store the received Additional RRM Policy Index in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is available at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT SETUP REQUEST. The gNB-DU may use it for RRM purposes.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT SETUP RESPONSE message, the result for all the requested DRBs, SRBs, BH RLC channels, Uu RLC channels, PC5 Relay RLC channels, and SL DRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to Setup List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of BH RLC channels which are successfully established shall be included in the BH RLC Channel Setup List IE;

-	A list of BH RLC channels which failed to be established shall be included in the BH RLC Channel Failed to be Setup List IE;

-	A list of SL DRBs which are successfully established shall be included in the SL DRB Setup List IE;

-	A list of SL DRBs which failed to be established shall be included in the SL DRB Failed to Setup List IE.

-	A list of Uu Relay RLC channels which are successfully established shall be included in the Uu RLC Channel Setup List IE;

-	A list of Uu Relay RLC channels which failed to be established shall be included in the Uu RLC Channel Failed to be Setup List IE;

-	A list of PC5 Relay RLC channels which are successfully established shall be included in the PC5 RLC Channel Setup List IE;

-	A list of PC5 Relay RLC channels which failed to be established shall be included in the PC5 RLC Channel Failed to be Setup List IE;

If Duplication Indication IE in SL DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message,the gNB-DU shall, if supported, generate two PC5 RLC bearer configurations for the indicated SL DRB.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB or SL DRB or a BH RLC channel or a Uu RLC channel or a PC5 Relay RLC channel, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

For EN-DC operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15].

For NG-RAN operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the DRB Information IE.

For DC operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE at the gNB acting as secondary node. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

For sidelink operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE if the gNB-CU receives sidelink related UE information from UE. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information as defined in TS 38.331 [8].

If the HandoverPreparationInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU of the gNB acting as master node shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. The gNB-CU shall only initiate the UE Context Setup procedure for handover or secondary node addition when at least one DRB is setup for the UE, or at least one BH RLC channel is set up for IAB-MT. If the HandoverPreparationInformation IE containing the sidelink related UE information is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information as defined in TS 38.331 [8].

If the received CU to DU RRC Information IE does not include source cell group configuration, the gNB-DU shall generate the cell group configuration using full configuration. Otherwise, delta configuration is allowed.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message.

If the MeasConfig IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall deduce that changes to the measurements configuration need to be applied. If the measObjectToAddModList IE is included in the MeasConfig IE, then the frequencies added in such IE are to be activated. Then the gNB-DU shall decide if measurement gaps are needed or not and, if needed, the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message. If the measObjectToRemoveList IE is included in the MeasConfig IE, the gNB-DU shall ignore it.

If the NeedForGapsInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

If the NeedForGapNCSG-InfoEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

If the NeedForInterruptionInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT SETUP REQUEST message shall be ignored. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The UEAssistanceInformationEUTRA IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformationEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring LTE sidelink resources for the UE.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the Masked IMEISV IE is contained in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be set up with an appropriate cause value for each SCell failed to setup.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT SETUP REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT SETUP RESPONSE message and set to "Not-supported", the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

If the ServCellInfoList IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall take it into account to generate the content of inter-node RRC message, i.e., CG-Config or CG-ConfigInfo, as described in TS 38.331 [8].

If the Full Configuration IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

If the C-RNTI IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

The UE Context Setup Procedure is not used to configure SRB0.

If the UE CONTEXT SETUP REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE via SRB1.

If the Notification Control IE is included in the DRB to Be Setup List IE contained in the UE CONTEXT SETUP REQUEST message and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store the received UL PDU Session Aggregate Maximum Bit Rate and use it when enforcing uplink traffic policing for non-GBR Bearers for the concerned UE as specified in TS 23.501 [21].

The gNB-DU shall store the received gNB-DU UE Aggregate Maximum Bit Rate Uplink and use it for non-GBR Bearers for the concerned UE.

If the UE CONTEXT SETUP REQUEST message contains the QoS Flow Mapping Indication IE, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the UE CONTEXT SETUP REQUEST message contains the New gNB-CU UE F1AP ID IE, the gNB-DU shall, if supported, replace the value received in the gNB-CU UE F1AP ID IE by the value of the New gNB-CU UE F1AP ID and use it for further signalling.

If the RAN UE ID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store and replace any previous information received.

If the Trace Activation IE is included in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, initiate the requested trace function as described in TS 32.422 [29].

In particular, the gNB-DU shall, if supported:

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT and Trace", initiate the requested trace session and MDT session as described in TS 32.422 [29];

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT Only", initiate the requested MDT session as described in TS 32.422 [29] and the gNB-DU shall ignore Interfaces To Trace IE, and Trace Depth IE. If the Management Based MDT PLMN List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [29].

For each QoS flow whose DRB has been successfully established and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [21].

If the UE CONTEXT SETUP REQUEST message contains the Configured BAP Address IE, the gNB-DU shall, if supported, store this BAP address configured for the corresponding child IAB-node and use it as specified in TS 38.340 [30].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Setup List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30].

If the F1-C Transfer Path IE is included in UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account.

If the NR V2X Services Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the LTE V2X Services Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the NR UE Sidelink Aggregate Maximum Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for NR V2X services.

If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for LTE V2X services.

If the PC5 Link Aggregate Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for NR V2X services as defined in TS 23.287 [40].

If the NR A2X Services Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the LTE A2X Services Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the NR UE Sidelink Aggregate Maximum Bit Rate for A2X IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for NR A2X services.

If the LTE UE Sidelink Aggregate Maximum Bit Rate for A2X IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for LTE A2X services.

If the TSC Traffic Characteristics IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE. If the RAN Feedback Type IE is included in the TSC Assistance Information Uplink IE of the TSC Traffic Characteristics IE, the gNB-DU shall, if supported, take this information into account when determining the feedback to provide in the TSC Traffic Characteristics Feedback IE in the UE CONTEXT SETUP RESPONSE message.

If the Conditional Inter-DU Mobility Information IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider that the request concerns a conditional handover, conditional PSCell addition, conditional PSCell change, or subsequent CPAC for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Target gNB-DU UE F1AP ID IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU shall replace the existing prepared conditional handover, conditional PSCell addition or conditional PSCell change, or subsequent CPAC identified by the Target gNB-DU UE F1AP ID IE and the SpCell ID IE.

If the Serving NID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall combine the Serving NID IE with the Serving PLMN IE to identify the serving NPN, and may take it into account for UE context establishment.

If the Estimated Arrival Probability IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If the S-CPAC Request IE is included within the Conditional Inter-DU Mobility Information IE in the UE CONTEXT SETUP REQUEST message and is set to "initiation", the gNB-DU shall, if supported, consider that the procedure is triggered for S-CPAC preparation.

If for a given E-RAB for EN-DC operation the ENB DL Transport Layer Address IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If for a given Qos flow for NG-RAN operation the PDCP Terminating Node DL Transport Layer Address IE is included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the F1-C Transfer Path NRDC IE is included in UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account.

If the MDT Polluted Measurement Indicator IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account as specified in TS 38.401 [4].

If the SCG Activation Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU may use it to configure SCG resources as specified in TS 37.340 [7], and if supported, shall include the SCG Activation Status IE in the UE CONTEXT SETUP RESPONSE message. If the SCG Activation Request IE in the UE CONTEXT SETUP REQUEST message is set to “Activate SCG”, the gNB-DU shall activate the SCG resources and set the SCG Activation Status IE in the UE CONTEXT SETUP RESPONSE message to “SCG Activated”.

If the Old CG-SDT Session Info IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, retrieve the old CG-SDT resource configuration and old UE context based on the indicated gNB-CU F1AP UE ID and gNB-DU F1AP UE ID.

If the 5G ProSe Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for 5G ProSe services.

If the 5G ProSe PC5 Link Aggregate Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for 5G ProSe services as defined in TS 23.304 [44].

If the Uu RLC Channel To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the Uu Relay RLC channel configurations for a L2 U2N Relay UE.

If the PC5 RLC Channel To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the PC5 Relay RLC channel configurations for a L2 U2N Remote UE, a L2 U2U Remote UE or a L2 U2U Relay UE.

If the Path Switch Configuration IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to configure the path switch from direct path to indirect path as specified in TS 38.401 [4].

If the MUSIM-GapConfig IE is contained in the CU to DU RRC Information IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, decide to use this IE for MUSIM gap configuration or select another one based on the received UEAssistanceInformation IE. If gNB-DU selects a different MUSIM gap configuration from received UEAssistanceInformation IE, then it shall include the selected MUSIM gap information to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message.

If MUSIM-GapConfig IE is not contained in the CU to DU RRC Information IE, then gNB-DU shall, if supported, send the selected MUSIM gap configuration based on the received UEAssistanceInformation IE, to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message. When MUSIM-GapConfig IE is received, the gNB-CU should use this value.

If the gNB-DU UE Slice Maximum Bit Rate List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store and use the information for the uplink traffic policing for each concerned slice as specified in TS 23.501 [21].

If the Multicast MBS Session Setup List IE is contained in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, store and use the information for configuring MBS Session Resources, if applicable.

If the UE Multicast MRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for configuring MBS Session Resources, if applicable. And if the MBS PTP Retransmission Tunnel Required IE is included in the UE Multicast MRB to Be Setup Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Retransmission F1-U tunnel. If the MBS PTP Forwarding Tunnel Required Information IE is included in the UE Multicast MRB to Be Setup Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Forwarding F1-U tunnel. If the Source MRB ID IE is included in the UE Multicast MRB to Be Setup Item IEs IE, the DU shall, if supported, use it to identify the MRB configuration as provided to the UE in the source cell and take it into account for configuring MBS Session Resources.

If the Dedicated SI Delivery Indication IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, take it into account for the system information delivery to the UE as described in TS 38.331 [8].

If the PDU Set QoS Parameters IE is included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store this information and use it as specified in TS 23.501 [21].

If the ECN Marking or Congestion Information Reporting Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it accordingly for the specific DRB. If the ECN Marking or Congestion Information Reporting Status IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it to deduce if ECN marking or congestion information reporting is active or not active.

If the InterFrequencyConfig-NoGap IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the ul-GapFR2-Config IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the TwoPHRModeMCG IE or the TwoPHRModeSCG IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use this value as described in TS 38.331 [8].

If the MBSInterestIndication IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

If the ncd-SSB-RedCapInitialBWP-SDT IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the Network Controlled Repeater Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it is set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized as Network Controlled Repeater.

If the musim-CapabilityRestrictionIndication IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

If the LTM Indicator IE set to "true" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, provide the lower layer configuration in the Reference Configuration Information IE in the LTM ConfigurationIE in the UE CONTEXT SETUP RESPONSE message for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration(s) in the CellGroupConfig IE for the requested LTM candidate cell.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Request for RACH Configuration IE set to "true" is contained in the Early Sync Information Request IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, include the Early UL Sync Configuration and/or Early UL Sync Configuration for SUL IE for early TA acquisition (early UL synchronisation), in the UE CONTEXT SETUP RESPONSE message.

If the LTM gNB-DUs List is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use this information to assign RACH resources for early TA acquisition.

If the Early Sync Information IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early sync information from the accepted candidate cell in the candidate gNB-DU.

If the LTM Configuration IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the candidate gNB-DU.

If the Complete Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete configuration.

If the Indirect Path Addition IE is contained in the Path Addition Information IE which is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns the indirect path addition for the MP Remote UE using PC5 link and use it as specified in TS 38.401 [4].

If the N3C Indirect Path Addition IE is contained in the Path Addition Information IE, the gNB-DU shall, if supported, consider that the request concerns the indirect path addition for the MP Remote UE using N3C and use it as specified in TS 38.401 [4].

If the S-CPAC Lower Layer Reference Config Request IE set to "true" is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, provide the lower layer configuration in the Reference Configuration Information IE in the S-CPAC Configuration IE in the UE CONTEXT SETUP RESPONSE message for the gNB-CU to generate the S-CPAC reference configuration.

If the Complete Configuration Indicator IE set to "complete" is contained in the S-CPAC Configuration IE included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider that the S-CPAC candidate configuration is a complete configuration.

If the musim-CandidateBandList IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for temporary capability restriction.

If the DL LBT Failure Information Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the gNB-CU has requested the DL LBT failure information of the UE in the target cell during handover.

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for the UE’s Ranging and Sidelink Positioning service.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: UE Context Setup Request procedure: unsuccessful Operation

If the gNB-DU is not able to establish an F1 UE context, or cannot even establish one bearer it shall consider the procedure as failed and reply with the UE CONTEXT SETUP FAILURE message. If the Conditional Inter-DU Mobility Information IE or the LTM Indicator IE was included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall include the received SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT SETUP FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT SETUP REQUEST message, it shall reply with the UE CONTEXT SETUP FAILURE message with an appropriate cause value. Further, if the Candidate SpCell List IE is included in the UE CONTEXT SETUP REQUEST message and the gNB-DU is not able to accept the SpCell ID IE, the gNB-DU shall, if supported, include the Potential SpCell List IE in the UE CONTEXT SETUP FAILURE message and the gNB-CU should take this into account for selection of an opportune SpCell. The gNB-DU shall include the cells in the Potential SpCell List IE in a priority order, where the first cell in the list is the one most desired and the last one is the one least desired (e.g., based on load conditions). If the Potential SpCell List IE is present but no Potential SpCell Item IE is present, the gNB-CU should assume that none of the cells in the Candidate SpCell List IE are acceptable for the gNB-DU.

#### 8.3.1.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value. If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT SETUP REQUEST message and is set to the value "delay critical" but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

In case of "CHO-replace" when the Target gNB-DU UE F1AP ID IE is included, if the candidate cell in the SpCell ID IE included in the UE CONTEXT SETUP REQUEST message was not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore this candidate cell.

<<<<<< NEXT CHANGE >>>>>>

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

If the SpCell ID IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace any previously received value and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the ServCellIndex IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take this into account for the indicated SpCell. If the SpCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly. If the servingCellMO List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, configure servingCellMO after determining the list of BWPs for the UE and include the list of servingCellMOs that have been encoded in CellGroupConfig IE as ServingCellMO-encoded-in-CGC List IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Configured BWP List IE is included in the UE CONTEXT MODIFICATION RESPONSE message the gNB-CU shall, if supported, take it into account when requesting the gNB-DU for generating preconfigured measurement GAP for the indicated BWPs.

If the Preconfigured Measurement GAP Request IE is present in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATON REQUEST message, the gNB-DU shall, if supported, consider that the content of the previous CellGroupConfig IE was not sent to the UE and generate the pre-configured measurement GAP for the indicated BWPs in the MeasConfig IE. If the gNB-DU successfully generates pre-configured measurement GAP for the indicated BWPs, the gNB-DU shall update the CellGroupConfig IE with the content of the previous CellGroupConfig IE and the preconfigured measurement GAP configuration in the UE CONTEXT MODIFICATION RESPONSE message.

If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message and the indicated SCell(s) are already setup, the gNB-DU shall replace any previously received value. If the SCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the SCell To Be Removed List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a list of SCells to be removed.

If the DRX Cycle IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall use the provided value from the gNB-CU. If the DRX configuration indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and set to "release", the gNB-DU shall release DRX configuration.

If the SL DRX Cycle list IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the provided value from the gNB-CU for the indicated RX UE of this UE. If the SL DRX configuration indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and set to "release", the gNB-DU shall, if supported, release SL DRX configuration for the indicated RX UE of this UE.

If the SRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4], and replace any previously received value. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB if the value is set to be "true", or delete the RLC entity of secondary path if the value is set to be "false". If the Additional Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup the indicated RLC entities for the indicated SRB. If the SRB Mapping Info IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the SRB Mapping Info IE for the SRB identified by the SRB ID IE and the Uu Relay RLC channel identified by the SRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of SRB data to Uu Relay RLC channel. If the Path Addition Information IE and the SRB Mapping Info IE are both contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, setup one RLC entity if necessary for the direct path and map the indicated SRB to the Uu Relay RLC channel based on the SRB Mapping Info IE. If the Duplication Indication IE and SRB Mapping Info IE are both contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, setup one RLC entity for the direct path if the value is set to be "true", and map the indicated SRB to the Uu Relay RLC channel based on the SRB Mapping Info IE. If the Additional Duplication Indication IE and SRB Mapping Info IE are both contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup the indicated RLC entities for the indicated SRB, and map the indicated SRB to the Uu Relay RLC channel based on the SRB Mapping Info IE. The number of RLC entities to be set up is the indicated value of Additional Duplication Indication IE minus 1.

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4]. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE, if present, for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

If the BH RLC Channel To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Setup Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE following the behaviour described for the UE Context Setup procedure.

If the BH RLC Channel To Be Modified List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Modified Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE following the behaviour described for the UE Context Setup procedure.

If the BH RLC Channel To Be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall release the BH RLC channels in the list.

If two UL UP TNL Information IEs are included and the DRB Mapping Info IE is not contained in UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT MODIFICATION RESPONSE message and setup two RLC entities for the indicated DRB. If the UL UP TNL Information IE with the DRB Mapping Info IE and the UL UP TNL Information IE without the DRB Mapping Info IE are both contained in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall, if supported, include two DL UP TNL Information IEs in UE CONTEXT MODIFICATION RESPONSE message, setup one RLC entity for the UL UP TNL Information IE without the DRB Mapping Info IE, and map the indicated DRB to the Uu Relay RLC channel based on the DRB Mapping Info IE. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA and multi-path relay as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT MODIFICATION RESPONSE message and setup one or two additional RLC entities for the indicated DRB. The gNB-CU and the gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA and multi-path relay as defined in TS 38.470 [2].

If Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activating/deactivating CA based PDCP duplication or multi-path relay based PDCP duplication for the DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for the DRB with more than two RLC entities.

If DC Based Duplication Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. Otherwise, the gNB-DU shall regard that DC based PDCP duplication is de-configured for this DRB id the value is set to be "false", and it should stop PDCP duplication activation/deactivation by MAC CE. If DC Based Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activating/deactivating DC based PDCP duplication for this DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall, if supported, take it into account when activating/deactivating DC based PDCP duplication for the DRB with more than two RLC entities. If the Primary Path Indication IE is included in the RLC Duplication Information IE, the gNB-DU shall, if supported, take it into account when performing DC based PDCP duplication for the DRB with more than two RLC entities.

For a certain DRB which was allocated with two GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the UE Context Modification procedure, the gNB-DU shall consider that the CA based PDCP duplication or multi-path relay based PDCP duplication for the concerned DRB is de-configured. If such UE Context Modification procedure occurs, the Duplication Activation IE shall not be included for the concerned DRB.

If the UL Configuration IE in DRB to Be Setup Item IE or DRB to Be Modified Item IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the RRC Reconfiguration Complete Indicator IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider the ongoing reconfiguration procedure involving changes of the L1/L2 configuration at the gNB-DU signalled to the gNB-CU via the CellGroupConfig IE for MR-DC operation or standalone operation has been successfully performed when such IE is set to ‘true’; otherwise (when such IE is set to ‘failure’), the gNB-DU shall consider the ongoing reconfiguration procedure has been failed and it shall continue to use the old L1/L2 configuration.

If DL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If UL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If the RLC Failure Indication IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU should consider that the RLC entity indicated by such IE needs to be re-established when the CA-based packet duplication is active, and the gNB-DU may include the Associated SCell List IE in UE CONTEXT MODIFICATION RESPONSE by containing a list of SCell(s) associated with the RLC entity indicated by the RLC Failure Indication IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE. If the UE CONTEXT MODIFICATION REQUEST message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication or multi-path relay based duplication, if configured, for the SRB for the included RRC-Container IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the Transmission Action Indicator IE, the gNB-DU shall stop or restart (if already stopped) data transmission for the UE, according to the value of this IE. It is up to gNB-DU implementation when to stop or restart the UE scheduling.

For EN-DC operation, if the DRB to Be Setup List IE is present in the UE CONTEXT MODIFICATION REQUEST message the gNB-CU shall include the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15]. For NG-RAN operation, the gNB-CU shall include the DRB Information IE in the UE CONTEXT MODIFICATION REQUEST message.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message.

If the MeasConfig IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall deduce that changes to the measurements’ configuration need to be applied. The gNB-DU shall take the received info, e.g. the measObjectToAddModList IE, and/or the measObjectToRemoveList IE into account, when generating measurement gap and when deciding if a measurement gap is needed or not.

If the NeedForGapsInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForInterruptionInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

For DC operation, if the gNB-CU includes the CG-Config IE in the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may initiate low layer parameters coordination taking this information into account.

For sidelink operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE if the gNB-CU receives sidelink related UE information from UE. If the CG-ConfigInfo IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information as defined in TS 38.331 [8].

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT MODIFICATION REQUEST message shall be ignored.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT MODIFICTION REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. If the Additional RRM Policy Index IE is received from an MeNB, the UE CONTEXT MODIFICATION REQUEST message shall, if supported, contain the Additional RRM Policy Index IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20]. The gNB-DU shall, if supported, store the received Additional RRM Policy Index in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is modified at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT MODIFICATION REQUEST. The gNB-DU may use it for RRM purposes.

Only one of the following IEs shall be contained in the UE CONTEXT MODIFICATION REQUEST message: the Uplink TxDirectCurrentList Information IE or the Uplink TxDirectCurrentTwoCarrierList Information IE or the Uplink TxDirectCurrentMoreCarrierList Information IE. If the UE CONTEXT MODIFICATION REQUEST message contains one of the Uplink TxDirectCurrentList Information IE or the Uplink TxDirectCurrentTwoCarrierList Information IE or the Uplink TxDirectCurrentMoreCarrierList Information IE, the gNB-DU may take that into account when selecting L1 configuration.

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The UEAssistanceInformationEUTRA IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformationEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring LTE sidelink resources for the UE.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT MODIFICATION RESPONSE message, the result for all the requested or modified DRBs, SRBs, BH RLC Channels, Uu Relay RLC channels, PC5 Relay RLC channels, and SL DRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to be Setup List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed to be Modified List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to be Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of successfully modified SRBs with logical channel identities for primary path shall be included in the SRB Modified List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of BH RLC channels which are successfully established shall be included in the BH RLC Channel Setup List IE;

-	A list of BH RLC channels which failed to be established shall be included in the BH RLC Channel Failed to be Setup List IE;

-	A list of BH RLC channels which are successfully modified shall be included in the BH RLC Channel Modified List IE;

-	A list of BH RLC channels which failed to be modified shall be included in the BH RLC Channel Failed to be Modified List IE;

-	A list of Uu Relay RLC channels which are successfully established shall be included in the Uu RLC Channel Setup List IE;

-	A list of Uu Relay RLC channels which failed to be established shall be included in the Uu RLC Channel Failed to be Setup List IE;

-	A list of Uu Relay RLC channels which are successfully modified shall be included in the Uu RLC Channel Modified List IE;

-	A list of Uu Relay RLC channels which are failed to be modified shall be included in the Uu RLC Channel Failed to be Modified List IE;

-	A list of PC5 Relay RLC channels which are successfully established shall be included in the PC5 RLC Channel Setup List IE;

-	A list of PC5 Relay RLC channels which failed to be established shall be included in the PC5 RLC Channel Failed to be Setup List IE;

-	A list of PC5 Relay RLC channels which are successfully modified shall be included in the PC5 RLC Channel Modified List IE;

-	A list of PC5 Relay RLC channels which failed to be modified shall be included in the PC5 RLC Channel Failed to be Modified List IE;

-	A list of SL DRBs which are successfully established shall be included in the SL DRB Setup List IE;

-	A list of SL DRBs which failed to be established shall be included in the SL DRB Failed to be Setup List IE;

-	A list of SL DRBs which are successfully modified shall be included in the SL DRB Modified List IE;

-	A list of SL DRBs which failed to be modified shall be included in the SL DRB Failed to be Modified List IE.

If Duplication Indication IE in SL DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, generate two PC5 RLC bearer configurations for the indicated SL DRB.

If Duplication Indication IE is contained in the SL DRB To Be Modified List IE, the gNB-DU shall, if supported, generate two PC5 RLC bearer configurations for the indicated SL DRB, if the value is set to be"true" and duplication is not already configured for the indicated SL DRB.

If Duplication Indication IE is contained in the SL DRB To Be Modified List IE, the gNB-DU shall, if supported, release the additional PC5 RLC configuration for the indicated SL DRB, if the value is set to be "false".

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Setup List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Modified List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30]. Otherwise, if the BAP Control PDU Channel IE is not present for any BH RLC channel, any available BH RLC channel can be used to transmit BAP Control PDUs as specified in TS 38.340 [30].

If the F1-C Transfer Path IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB or SL DRB or a BH RLC channel or a Uu Relay RLC channel or a PC5 Relay RLC channel, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the DU to CU RRC Information IE is included in the UE CONTEXT MODIFICATION RESPONSE message, except for the CG-SDT procedure and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of CG-SDT, and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

If the ServCellInfoList IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall take it into account to generate the content of inter-node message, i.e., CG-Config or CG-ConfigInfo, as described in TS 38.331 [8].

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be set up with an appropriate cause value for each SCell failed to setup.

If the C-RNTI IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT MODIFICATION RESPONSE message and set to "Not-supported", the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

The UE Context Modify Procedure is not used to configure SRB0.

If in the UE CONTEXT MODIFICATION REQUEST, the Notification Control IE is included in the DRB to Be Setup List IE or the DRB to Be Modified List IE and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace the received UL PDU Session Aggregate Maximum Bit Rate and use it as specified in TS 23.501 [21].

If the gNB-DU UE Aggregate Maximum Bit Rate Uplink IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall:

-	replace the previously provided gNB-DU UE Aggregate Maximum Bit Rate Uplink with the new received gNB-DU UE Aggregate Maximum Bit Rate Uplink;

-	use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink for non-GBR Bearers for the concerned UE.

The gNB-DU UE Aggregate Maximum Bit Rate Uplink IE shall be sent in the UE CONTEXT MODIFICATION REQUEST if DRB to Be Setup List IE is included and the gNB-CU has not previously sent it. The gNB-DU shall store and use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink IE.

If the RLC Status IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall assume that RLC has been reestablished at the gNB-DU and may trigger PDCP data recovery.

If the GNB-DU Configuration Query IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall include the DU To CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Bearer Type Change IE is included in DRB to Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall either reset the lower layers or generate a new LCID for the affected bearer as specified in TS 37.340 [7].

For NE-DC operation, if NeedforGap IE is included in the UE CONTEXT MODIFICATION REQUEST message,the gNB-DU shall generate measurement gap for the SeNB.

If the QoS Flow Mapping Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, replace any previously received value and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the Lower Layer presence status change IE set to "suspend lower layers" is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-DU shall keep all lower layer configuration for UEs, and not transmit or receive data from UE.

If the Lower Layer presence status change IE set to "resume lower layers" is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall use the previously stored lower layer configuration for the UE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate a CellGroupConfig IE using full configuration and include it in the UE CONTEXT MODIFICATION RESPONSE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

For each QoS flow whose DRB has been successfully established or modified and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [21].

If the NR V2X Services Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its V2X services authorization information for the UE accordingly. If the NR V2X Services Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the LTE V2X Services Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its V2X services authorization information for the UE accordingly. If the LTE V2X Services Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE LTE Sidelink Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE NR Sidelink Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

If the NR A2X Services Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its A2X services authorization information for the UE accordingly. If the NR A2X Services Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the LTE A2X Services Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its A2X services authorization information for the UE accordingly. If the LTE A2X Services Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the LTE UE Sidelink Aggregate Maximum Bit Rate for A2X IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE LTE Sidelink Aggregate Maximum Bit Rate for A2X, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE A2X services.

If the NR UE Sidelink Aggregate Maximum Bit Rate for A2X IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE NR Sidelink Aggregate Maximum Bit Rate for A2X, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR A2X services.

If the PC5 Link Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE PC5 Link Aggregate Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services as defined in TS 23.287 [40].

If the TSC Traffic Characteristics IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE. If the RAN Feedback Type IE is included in the TSC Assistance Information Uplink IE of the TSC Traffic Characteristics IE, the gNB-DU shall, if supported, take this information into account when determining the feedback to provide in the TSC Traffic Characteristics Feedback IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the CPAC MCG Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CPAC Trigger is set to "CPAC-preparation", the gNB-DU shall, if supported, consider that the request concerns a conditional PSCell addition or conditional PSCell change or subsequent CPAC. The gNB-DU takes the included CG-Config and/or CG-ConfigInfo IE into account, and may provide a corresponding CellGroupConfig IE for MCG configuration preparation in the UE CONTEXT MODIFICATION RESPONSE message. The UE CONTEXT MODIFICATION RESPONSE message also includes a Requested Target Cell ID IE corresponding to the PSCell ID IE in the UE CONTEXT MODIFICATION REQUEST message.

If the CPAC MCG Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CPAC Trigger is set to "CPAC-executed", the gNB-DU shall, if supported, consider that, for the included PSCell ID IE corresponding to the selected PSCell, the UE has successfully executed the CPAC preparation. The gNB-DU shall apply the corresponding CellGroupConfig IE for MCG configuration.

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-initiation", the gNB-DU shall consider that the request concerns a conditional handover, conditional PSCell addition, conditional PSCell change, or subsequent CPAC for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-replace", the gNB-DU shall replace the existing prepared conditional mobility identified by the gNB-DU UE F1AP ID IE and the SpCell ID IE.

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-cancel", the gNB-DU shall consider that the gNB-CU is about to remove any reference to, and release any resources previously reserved for the candidate cells associated to the UE-associated signalling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE. If the Candidate Cells To Be Cancelled List IE is also included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider that only the resources reserved for the cells identified by the included NR CGIs are about to be released by the gNB-CU.

If the S-CPAC Request IE is included within the Conditional Intra-DU Mobility Information IE in the UE CONTEXT MODIFICATION REQUEST message and is set to "initiation", the gNB-DU shall, if supported, consider that the procedure is triggered for S-CPAC preparation.

If the Transmission Stop Indicator IE is included within the DRB to Be Modified Item IE in the UE CONTEXT MODIFICATION REQUEST message and set to “true”, the gNB-DU shall, if supported, stop the data transmission for the DRB. It is up to gNB-DU implementation when to stop the UE scheduling for that DRB.

If the SCG Indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and it is set to “released”, the gNB-DU shall, if supported, deduce that an SCG is removed.

If the Estimated Arrival Probability IE is contained in the Conditional Intra-DU Mobility Information IE included in the UE CONTEXT MODIFICATION REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If the Location Measurement Information IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring measurement gaps for the UE.

If the F1-C Transfer Path NRDC IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account.

If for a given E-RAB for EN-DC operation the ENB DL Transport Layer Address IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If for a given Qos flow for NG-RAN operation the PDCP Terminating Node DL Transport Layer Address IE is included in the UE CONTEXT MODIFICATION REQUEST message, then the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the gNB-DU is an IAB-DU, and if the IAB Conditional RRC Message Delivery Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message together with the RRC-Container IE, and if its value is set to “true”, and if the RRC-Container IE is for a child IAB-MT of the gNB-DU, the gNB-DU shall, if supported, withhold the RRC message until one of the following conditions is met:

If the gNB-DU belongs to a migrating IAB-node, whose co-located IAB-MT has successfully performed the random-access procedure to the target parent node, and if the migrating IAB-node has one or more routing entries for the target path.

The gNB-DU receives a subsequent F1AP message including an RRC-Container IE for the same child node.

If the gNB-DU belongs to a descendant node of the migrating IAB-node, whose co-located IAB-MT has received an RRCReconfiguration message including the intra-donor migration configurations, e.g., new TNL address(es) and the new default UL BAP routing ID.

If the gNB-DU belongs to a migrating IAB-node, whose co-located IAB-MT has successfully performed RLF recovery after handover failure, and if the migrating IAB-node has one or more routing entries for the target path.

If the MDT Polluted Measurement Indicator IE is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-DU shall take this information into account as specified in TS 38.401 [4].

If the SCG Activation Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may use it to configure SCG resources as specified in TS 37.340 [7], and if supported, shall include the SCG Activation Status IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the CG-SDT Query Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message and set to ‘true’, the gNB-DU shall, if supported, provide the CG-SDT related resource configuration for the bearers indicated as SDT bearers in the SDT-MAC-PHY-CG-Config IE within the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU. If the SDT-MAC-PHY-CG-Config IE is also included in the UE CONTEXT MODIFICATION REQUEST message within the CU to DU RRC Information IE, the gNB-DU may provide the delta signalling version of the SDT-MAC-PHY-CG-Config IE within the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

If the 5G ProSe Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its 5G ProSe services authorization information for the UE accordingly. If the 5G ProSe Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the SDT Bearer Configuration Query Indication IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, provide the RLC bearer configuration in the SDT Bearer Configuration Info IE in the UE CONTEXT MODIFICATION RESPONSE message for each bearer indicated as SDT bearer.

If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided 5G ProSe UE PC5 Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

If the 5G ProSe PC5 Link Aggregate Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided 5G ProSe PC5 Link Aggregate Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services as defined in TS 23.304 [44].

If the Updated Remote UE Local ID IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, replace the previously provided Remote UE Local ID, if available in the UE context, with the received value.

If the Uu RLC Channel To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4].

If the Uu RLC Channel To Be Modified List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4].

If the Uu RLC Channel To Be Release List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the Uu Relay RLC channels in the list.

If the PC5 RLC Channel To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the PC5 Relay RLC channel configurations for a L2 U2N Remote UE, U2N Relay UE, a L2 U2U Remote UE or a L2 U2U Relay UE. If the F1AP-IDs are associated with a U2N Relay UE, the PC5 RLC Channel to be Setup Item IEs IE shall include the Remote UE Local ID and correspondingly, the PC5 RLC Channel Setup Item IEs IE and the PC5 RLC Channel Failed to be Setup Item IE in the UE CONTEXT MODIFICATION RESPONSE message shall include the Remote UE Local ID IE.

If the PC5 RLC Channel To Be Modified List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the PC5 Relay RLC channel configurations for a L2 U2N Remote UE , U2N Relay UE, a L2 U2U Remote UE or a L2 U2U Relay UE. If the F1AP-IDs are associated with a U2N Relay UE, the PC5 RLC Channel to be Modified Item IEs IE shall include the Remote UE Local ID IE and correspondingly, the PC5 RLC Channel Modified Item IEs IE and the PC5 RLC Channel Failed to be Modified Item IEs IE in the UE CONTEXT MODIFICATION RESPONSE message shall include the Remote UE Local ID IE.

If the PC5 RLC Channel To Be Release List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the PC5 Relay RLC channels in the list. If the F1AP-IDs are associated with a U2N Relay UE, the PC5 RLC Channel to be Released Item IEs IE shall include the Remote UE Local ID IE.

If the Path Switch Configuration IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it to configure the path switch from direct path to indirect path as specified in TS 38.401 [4].

If the MUSIM-GapConfig IE is contained in the CU to DU RRC Information IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, decide to use this IE for MUSIM gap configuration or select another one based on the received UEAssistanceInformation IE. If gNB-DU selects a different MUSIM gap configuration from received UEAssistanceInformation IE, then it shall include the selected MUSIM gap information to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message.

If MUSIM-GapConfig IE is not contained in the CU to DU RRC Information IE, then gNB-DU shall, if supported, send the selected MUSIM gap configuration based on the received UEAssistanceInformation IE, to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message. When MUSIM-GapConfig IE is received, the gNB-CU should use this value.

If the gNB-DU UE Slice Maximum Bit Rate List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported,

-	store and replace the previously provided gNB-DU UE Slice Maximum Bit Rate List, if any, with the new received gNB-DU UE Slice Maximum Bit Rate List;

-	use the received gNB-DU UE Slice Maximum Bit Rate List for the uplink traffic policing for each concerned slice as specified in TS 23.501 [21].

If the Multicast MBS Session Setup List IE or the Multicast MBS Session Remove List IE or both IEs are contained in the UE CONTEXT MODIFICATION REQUEST message the gNB-DU shall, if supported, store and use the information for configuring MBS Session Resources, if applicable.

If the UE Multicast MRB To Be Setup at Modify List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for configuring MBS Session Resources, if applicable, and shall include the Multicast F1-U Context Reference CU IE, if available, in the UE CONTEXT MODIFICATION RESPONSE message. And if the MBS PTP Retransmission Tunnel Required IE is included in the UE Multicast MRB to Be Setup at Modify Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Retransmission F1-U tunnel.

If the MBS PTP Forwarding Tunnel Required Information IE is included in the UE Multicast MRB to Be Setup at Modify Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Forwarding F1-U tunnel.

If the Management Based MDT PLMN Modification List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, overwrite any previously stored Management Based MDT PLMN List information in the UE context and use the received information to determine subsequent selection of the UE for management based MDT defined in TS 32.422 [29].

If the Dedicated SI Delivery Indication IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, take it into account for the system information delivery to the UE as described in TS 38.331 [8].

If the PDU Set QoS Parameters IE is included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and use it as specified in TS 23.501 [21].

If the ECN Marking or Congestion Information Reporting Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it accordingly for the specific DRB. If the ECN Marking or Congestion Information Reporting Status IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it to deduce if ECN marking or congestion information reporting is active or not active.

If the InterFrequencyConfig-NoGap IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the ul-GapFR2-Config IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the TwoPHRModeMCG IE or the TwoPHRModeSCG IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use this value as described in TS 38.331 [8].

If the MBSInterestIndication IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

If the ncd-SSB-RedCapInitialBWP-SDT IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the Network Controlled Repeater Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its authorization information for the UE accordingly. If the Network Controlled Repeater Authorized IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing as a Network Controlled Repeater.

If the LTM Indicator IE set to "true" is contained in the LTM Information Modfiy IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT MODIFICATION RESPONSE message.

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include the Reference Configuration Information IE in the LTM Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message to provide lower layer configuration for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the requested LTM candidate cell.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Request for RACH Configuration IE set to "true" is contained in the Early Sync Information Request IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for early TA acquisition, and include the Early UL Sync Configuration and/or Early UL Sync Configuration for SUL IE  in the UE CONTEXT MODIFICATION RESPONSE message.

If the Early Sync Information List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the LTM Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the gNB-DU.

If the LTM Cells to be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the configured candidate cells in the list.

If the Complete Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete configuration.

If the Direct Path Addition IE is contained in the Path Addition Information IE which is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns the direct path addition for the included SpCell ID IE as specified in TS 38.401 [4] and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the Indirect Path Addition IE is contained in the Path Addition Information IE, the gNB-DU shall, if supported, consider that the request concerns the indirect path addition for the MP Remote UE using PC5 link and use it as specified in TS 38.401 [4]. If the N3C Indirect Path Addition IE is contained in the Path Addition Information IE, the gNB-DU shall, if supported, consider that the request concerns the indirect path addition for the MP Remote UE using N3C and use it as specified in TS 38.401 [4].

If the S-NSSAI IE is included within the DRB to Be Modified Item IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store the corresponding information and replace any existing information.

If the S-CPAC Lower Layer Reference Config Request IE set to "true" is contained in the Conditional Intra-DU Mobility Information IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, provide the lower layer configuration in the Reference Configuration Information IE in the S-CPAC Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message for the gNB-CU to generate the S-CPAC reference configuration.

If the Complete Configuration Indicator IE set to "complete" is contained in the S-CPAC Configuration IE included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the S-CPAC candidate configuration is a complete configuration.

If the musim-CandidateBandList IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for temporary capability restriction.

If the DL LBT Failure Information Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the gNB-CU has requested the DL LBT failure information of the UE in the target cell in case of a failure.

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

#### 8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the UE context can be successfully performed, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. If the Conditional Intra-DU Mobility Information IE was included in the UE CONTEXT MODIFICATION REQUEST message and set to "CHO-initiation", the gNB-DU shall include the received SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT MODIFICATION REQUEST message, it shall reply with the UE CONTEXT MODIFICATION FAILURE message.

If the Conditional Intra-DU Mobility Information IE was included and set to "CHO-initiation" or "CHO-replace" or if the LTM Indicator IE was included, but the SpCell ID IE was not included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

If the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

#### 8.3.4.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT MODIFICATION REQUEST message and is set to the value "delay critical" but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT MODIFICATION REQUEST message were not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore those non-associated candidate cells.

If more than one of the following IEs, i.e., the Uplink TxDirectCurrentList Information IE or the Uplink TxDirectCurrentTwoCarrierList Information IE or the Uplink TxDirectCurrentMoreCarrierList Information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a logical error.

If one or more LTM cells in the LTM Cells To Be Released List IE included in the UE CONTEXT MODIFICATION REQUEST message were not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore those non-associated LTM cells.

<<<<<< END OF CHANGE >>>>