3GPP TSG-RAN WG3 Meeting #123bis	R3-24XXXX

Changsha, China, 15-19 Apr 2024

Agenda item:	11.3

Source:	Samsung, Ericsson, Huawei, Telecom Italia, ZTE, Nokia, CATT

Title:	(TP to TR 38.743) AI/ML enabled CCO

Document for:	Discussion & Decision

# Introduction

This paper contains the TP to show the agreement of the AI/ML enabled CCO.

# Text proposal to TR 38.743

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.2.1	Use case description

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

Besides, CCO decision could also be implemented via AL/ML.

### 4.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.