3GPP TSG-RAN WG3 Meeting #123	R3-240869

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	9.1.1.1.

Source:-Huawei (moderator)

Title:	Summary of Offline Discussion on SONMDT2_SPRSHR

Document for:	Approval

# Introduction

CB: # SONMDT2_SPRSHR

- Single stage2 CR for TS37.340: take all the stage2 CRs above into account

- CR for TS38.423 with reference change? Change to TS38.300?

- Possible LS to RAN2?

(moderator - HW)

Summary of offline disc R3-240869

# For the Chairman’s Notes

Propose the following:

R3-240506 and R3-240617 merged to R3-241068

R3-240416 rev in R3-241068 – agreed

(new) R3-241064, draftCR, TS 38.300 v18.0.0, Rel-18, Cat. F– agreed

(new) R3-241065, CR1243 r, TS 38.423 v18.0.0, Rel-18, Cat. F – agreed

# Discussion

## SPR available indication

| R3-240506 | Correction on Successful PSCell Change Report for SONMDT Enhancements (CMCC) | draftCR E///: stage3 is clear enough CATT, Nok: Fine with the first change, no need for the second change, the third change seems also not needed Lenovo: The third change is not needed |
| --- | --- | --- |

Q: is the new text on SPR available indication needed:

If the SPR available indication via SN RRCReconfigComplete is received by SN, SN should inform MN that an SPR is available at the UE via S-NODE MODIFICATION REQUIRED message.

| Company | Comment |
| --- | --- |
| Huawei | OK to add – details may be revised later |
| Nokia | Ok to add the above new text. |
| Samsung | OK to add. “if supported” may need to be added to the new sentence. |
| Lenovo | ok |
| ZTE | OK |

Conclusion: This is added to CR for 37.340

## SN mobility info

| R3-240617 | SPR SHR and MRO corrections (Huawei, Deutsche Telekom) | Discussion E///: P1 is ok CATT: Ok to change the reference |
| --- | --- | --- |
| R3-240414 | Correction to 38.423 for SON features enhancement (CATT) | CR1169r, TS 38.423 v18.0.0, Rel-18, Cat. F |
| R3-240413 | Correction to 37.340 for CPAC of SON feature (CATT) | draftCR Nok: The suitable PSCell has not been agreed - Capture the only change: CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases for wrong candidate PSCell list selection: - Add Nok, ZTE as co-source Rev in R3-240876 Endorsed unseen |

38.423 reference to 38.300 for SN mobility info, but there is no such definition there. The proposal is to add a short description in 37.340 and update the references in 38.423. The proposals in R3-240617 is used as the starting point for the discussion, but similar changes can be found in . R3-240414 and R3-240413.

- . If it is SN-initiated PSCell change/CPC, the SN may make the SN mobility information available to the MN which can later send it back to the SN when the failure occurs.

| Company | Comment |
| --- | --- |
| Huawei | OK to add – details may be revised later |
| Nokia | We agree on adding a short description in 37.340 and update the references in 38.423 based on Huawei proposal in R3-240617. CATT proposal in R3-240413 is too long and not needed. |
| Samsung | OK to add – details may be revised later |
| Lenovo | OK to add – details may be revised later |
| ZTE | OK to add. |

Conclusion: This is added to CR for 37.340 and 38.423

## Alignment of definition and section title in 37.340

Q: should we align the terminology and section headers as follows:

SPR	Successful PSCell Addition/Change Report

10.18 Self-optimisation for PSCell addition/change

10.18.4 into Successful PSCell addition/change Report.

| Company | Comment |
| --- | --- |
| Huawei | OK to add – details may be revised later |
| Nokia | OK based on Huawei proposal in R3-240617 |
| Samsung | OK to add – details may be revised later |
| Lenovo | OK to add – details may be revised later |
| ZTE | OK to add. |

Conclusion: This is added to CR for 37.340

## PSCell change failure in TS 38.300

| R3-240416 | Correction to 37.470 for SPR optimization (CATT, Huawei, Ericsson) | draftCR noted Whether the second change on T310 and T312 is needed? |
| --- | --- | --- |

Q: should the st2 text in 38.300 be simplified and section title updated?

- 15.5.2.6	PSCell addition/change failure

. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [21].

| Company | Comment |
| --- | --- |
| Huawei | OK to add – details may be revised later |
| Nokia | OK based on Huawei proposal in R3-240617 |
| Samsung | Not needed as we commented online. |
| Lenovo | Title update is ok, but st2 text can be kept. |

Conclusion: This is added to CR for 38.300

## Capture SPR triggers in stage2?

| R3-240416 | Correction to 37.470 for SPR optimization (CATT, Huawei, Ericsson) | draftCR noted Whether the second change on T310 and T312 is needed? |
| --- | --- | --- |

Q: should the SPR triggers be captured in 37.470.

- For SN-initiated PSCell change/CPC, the source SN decides the T310/T312 triggers for SPR and is responsible for SPR related optimizations e.g., to optimize PSCell change/CPC configuration and/or associated mobility thresholds and/or adjust SPR T310/T312 triggers and/or adjust T310/T312 timer values.

- For MN-initiated PSCell change/CPC, the MN decides the T310/T312 triggers for SPR. MN may optimize PSCell change/CPC configuration and/or associated mobility thresholds and/or adjust T310/T312 SPR triggers. Source SN may optimize lower layer issues e.g., adjust T310/T312 timer values.

| Company | Comment |
| --- | --- |
| Huawei | OK to add – details may be revised later |
| Nokia | OK to add – details may be revised later |
| Samsung | The trigger doesn’t bring any failure or potential failure. It is the implementation issue whether to optimize the trigger. No need to capture this in specification. |
| Lenovo | Agree with SS |

Conclusion: No consensus to capture this

## Revise st2 text on SPR

The question here is whether the text in 38.300 should be revised as follows as proposed in R3-240207 (Lenovo):

- successful PSCell addition/change, the UE may collect

| Company | Comment |
| --- | --- |
| Nokia | OK if it is in accordance with RAN2 |
| Lenovo | Ok, the original text is added by RAN3, so revision which is aligned with RAN2 should also be done by RAN3. |

Conclusion: This is agreeable and will be in R3-240872

## Additional stage2 text on correlation

| R3-240752 | Correction on inter-RAT SHR for correlation function for TS38.300 (ZTE) | draftCR Move to 9.1.1.1 E///, SS: Fine, check the details HW: What’s the value? ZTE: Stage2 text is missing which is to be used as reference in other specs Lenovo, CATT: The first change is already covered both intra and inter-RAT cases |
| --- | --- | --- |

The question is whether we need additional text to describe the correlation?

| Company | Comment |
| --- | --- |
| Huawei | This is not needed. The current text is based on a long discussion and there is no need to extend. |
| Nokia | Agree with Huawei – not needed. |
| Samsung | We support to have it. For the two IEs, stage 3 refers to stage 2, but there is no any description on target C-RNTI and Time since failure in stage 2. The existing text is added in Rel-17 spec for intra-NR SHR. Details may be revised later. |
| Lenovo | Not needed, the existing text covers both inter-RAT HO and intra-NR HO. |
| ZTE | As correctly point by Samsung, the stage 2 description from stage 3 is missing. Alternatively, the Stage 3 description should be corrected. |

Conclusion: No consensus to add in stage2.

Additional topic related to the above:

The following can be removed from XnAP: If the Target cell C-RNTI IE and the Time Since Failure IE are received, NG-RAN node2 uses the information as specified in TS 38.300 [9].

Conclusion: No consensus on this

## Summary

The output will be merged in three CRs:

- 38.423: HW

- 38.300: HW

- 37.340: CATT