3GPP TSG-RAN WG3 Meeting #126	R3-24xxxx

Orlando, USA, October 18th – 22th Nov 2024

Title:	[Draft] LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT

Response to:	N/A

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Samsung (to be RAN3)

To:	RAN2

Cc:	N/A

Contact Person:

      - Name:	Lixiang Xu

      - E-mail Address:	lx.xu@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall Description:

RAN3 would like to inform RAN2 about progress in 3 topics from the Rel-19 SON/MDT WI:

(1) Regarding MRO for CHO with candidate SCG(s):

RAN3 concluded that the following information should be reported from the UE for failure reason detection and optmisation:

- The type of the first fulfilled execution condition

- Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

- The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

- Regarding SON/MDT for NTN:

On MRO for NTN, RAN3 sees benefits in including the fulfilled CHO trigger condition before RLF occurs in the RLF report, in case of CHO with “time and measurement based trigger condition” or “location and measurement based trigger condition”.

On MDT for NTN, RAN3 assumes that both of the geographical area and the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. RAN3 would also like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN.

(3) Regarding R18 Leftovers:

On SON for RA-SDT, RAN3 sees benefits in including the time from the start of RA-SDT until the reporting of the RA report. The network can use it to determine the configuration that was applied at the time of the start of RA-SDT.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 kindly asks RAN2 to take the above into consideration and update their specifications as needed.

3. Date of Next RAN3 Meetings:

TSG RAN3 Meeting #127           17-21 February 2025  		 Athens, Greece

TSG RAN3 Meeting #127bis	7-11 April 2025		           Wuhan, China