3GPP TSG-RAN WG3 Meeting #126	R3-247840

Orlando, USA, 18-22 Nov 2024

Agenda item:	10.3.2

Source:	Nokia

Title:	Summary of an unofficial offline discussion on SON for network slicing

Document for:	Discussion

# 1	Introduction

At RAN3#126, the main online discussion on SON for network slicing is scheduled for the last day and thus there will not be any time for the official offline discussion. Because of this, an unofficial offline discussion was organised to prepare for the online time. This document summarises the conclusions of this unofficial discussion.

# 2	For the meeting notes

RAN3 will ask RAN2 to consider enhancements to the logged MDT related to the slice prioritisation during reselection.

Proposed text for the LS:

RAN3 has identified a possible optimisation scenario related to the usage of NSAG. RAN3 considers that the logged MDT may be used to:

- Identify the UEs that use the prioritized reselection to monitor if the configured priorities are used when needed.

- Monitoring if the network configuration of the NSAG is optimized.

Possible topics to continue the discussion online:

- Enhancements to address slice unavailability (based on R3-247132, R3-247714)

- Slice information in UHI/MHI (based on R3-247714)

- Per-slice HO delay reporting (based on R3-247497)

- Deferred MDT reporting (based on R3-247496)

- Abnormal case when Area Scope of MDT-NR IE is not included for XNAP (based on a TP in R3-247703)

# 3	Discussion

## 3.1	Enhancements for MDT to detect idle mobility problems

In [7132,7160,7668,7703,7714] it is proposed to enhance logged MDT so that efficiency of the NSAG efficiency can be monitored.

In [7496], it is proposed to use immediate MDT to record service denials in case UE switches to RRC connected in a cell where slice is not supported.

On the other hand, in [7153], it is argued no such enhancements are needed.

RAN3 may identify a scenario to work on and if RAN2’s help is needed, RAN3 must send an LS – RAN2 has suspended any discussions on the topic.

Possible text for the LS:

RAN3 has identified a possible optimisation scenario related to the usage of NSAG. RAN3 considers that the logged MDT may be used to:

- Identify the UEs that use the prioritized reselection to monitor if the configured priorities are used when needed.

- Monitoring if the network configuration of the NSAG is optimized.

Text that is not agreeable yet:

In addition, RAN3 has identified a problem when UE switches to RRC Connected mode in a cell that does not support needed slice. Such events may also be recorded via the logged MDT.

## 3.2	Enhancements to address slice unavailability

In [7132, 7714], it is proposed to enhance SHR to monitor HOs to cells that do not have all needed slices available.

On the other hand, in [7153, 7160], enhancing SHR is considered unnecessary.

Agreement proposal 3: RAN3 will request RAN2 to enhance the SHR so that it can be generated when UE executes HO to a target cell that does not support all needed slices (FFS on the details).

## 3.3	Slice information in UHI/MHI

In [7714], it is proposed to add slice information to the history records.

Agreement proposal 4: RAN3 will add used slice information to the UHI and will request RAN2 to add the used slice information in the MHI.

## 3.4	Per-slice HO delay reporting

In [7497], it is proposed to report HO delay per slice in case of any HO.

Agreement Proposal y: RAN3 to work on network based solutions for collecting per slice user plane  interruption time during handovers

## 3.5	Deferred MDT reporting

In [7496], it is proposed to enable the RAN to provide collected MDT measurements only if certain condition (load level) is fulfilled.

Agreement Proposal z RAN3 to work on deferred (conditional) immediate MDT measurements collection.

## 3.6 Abnormal case when Area Scope of MDT-NR IE is not included for XNAP

In [7703], it is propose a TP to introduce an abnormal case for XNAP:

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE in the TRACE START message, and the Area Scope of MDT-NR IE is not included, the target NG-RAN node shall ignore the Network Slice Area Scope of MDT IE, and consider that the MDT Configuration for NR is applied to all PLMNs indicated in the MDT PLMN List described in TS 32.422 [23].

# References

[7132]	R3-247132, [TP to BL CR to 38.300, MDT] MDT solution for slice support and slice-related mobility enhancements (Nokia)

[7496]	R3-247496, On MDT enhancements for Network Slicing (Ericsson, T-Mobile US, InterDigital, Jio, Telecom Italia, Deutsche Telekom)

[7703]	R3-247703, Further consideration on SON/MDT for Slicing (ZTE Corporation)

[7153]	R3-247153, SON MDT for network slicing (Qualcomm Incorporated)

[7160]	R3-247160, SON and MDT for Network Slicing (Huawei)

[7497]	R3-247497, On SON enhancements for Network Slicing (Ericsson, InterDigital, Jio)

[7498]	R3-247498, (TP for BLCR for MDT for TS 38.413) Addition of MDT enhancements (Ericsson)

[7650]	R3-247650, Discussion on SONMDT enhancements for network slicing (China Unicom)

[7668]	R3-247668, Discussion on SONMDT for network slicing (CMCC)

[7714]	R3-247714, Network slicing for SONMDT (CATT)