3GPP TSG-RAN WG3 Meeting #126	R3-247791

Orlando, USA, 18 – 22 November 2024

Agenda item:	12.3

Source:	Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, ZTE

Title:	[TP for BL CR NR Femto 38.300] Introduction of Functional aspects of NR Femto architecture

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This is the TP for functional aspect to support NR Femto in release 19.

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 4.X.2	Functional Split

An NR Femto hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

-	An NR Femto shall only connect to a single NR Femto GW at one time, namely no NG-C Flex function shall be used at the NR Femto:

-	The NR Femto will not simultaneously connect to another NR Femto GW, or another AMF.

-	The TAC and PLMN ID used by the NR Femto shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is hosted by the NR Femto GW instead of the NR Femto. Upon reception of the GUAMI or requested NSSAI from a UE, the NR Femto shall include it in the INITIAL UE MESSAGE message.

-	NR Femtos may be deployed without network planning. An NR Femto may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location;

-	Signalling the GUAMI of the Source AMF and the Source AMF UE NGAP ID to the NR Femto GW in the NGAP PATH SWITCH REQUEST message.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>