3GPP TSG-RAN WG3 Meeting #126	R3-247791

Orlando, USA, 18 – 22 November 2024

Agenda item:	12.3

Source:	Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, ZTE

Title:	[TP for BL CR NR Femto 38.300] Introduction of Functional aspects of NR Femto architecture

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This is the TP for functional aspect to support NR Femto in release 19.

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 4.X.2	Functional Split

An NR Femto hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

NOTE: The NR Femto may be configured with the identity and/or address of an appropriate NR Femto GW.

-	An NR Femto shall only connect to a single NR Femto GW at one time:

-	The NR Femto will not simultaneously connect to another NR Femto GW, or another AMF.

NOTE: NR Femtos may be deployed without network planning. An NR Femto may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location;

-	The TAC and PLMN ID(s) used by the NR Femto shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is performed by the NR Femto GW.

-	AMF selection is based on the GUAMI, if received from the UE, provided to the NR Femto GW in the INITIAL UE MESSAGE message.

-	NR Femtos may be deployed without network planning. An NR Femto may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location;

-	At Xn handover:

-	Allowing the NR Femto GW to identify the source side termination of the source UE associated signaling connection:

-	Including the Source AMF UE NGAP ID to the NR Femto GW in the NGAP PATH SWITCH REQUEST message.

-	Allowing the NR Femto GW to identify the serving AMF at path switch:

-	Including the GUAMI of the source AMF in the PATH SWITCH REQUEST message.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>