3GPP TSG-RAN WG3 Meeting #127-bis	R3-251606

Wuhan, China, 7-11 April, 2025

Title:		(TPs to TS 38.300 38.413 BL CRs) Support of Device Locating

Agenda Item:	16.3

Source:	Huawei

Document for:	other

# 1	Introduction

The followings were achieved in last meeting:

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Command Report?

AIoT RAN node sends the Inventory Report aggregated per Reader.

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally.

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report.

In this contribution, we further discuss the support of Device Locating, and provide the corresponding TPs.

# 2	Discussion

About whether to send device location in Command Report:

As the command will be transmitted after inventory report in which the device location will always be provided, there is no need to provide device location in Command related message(s).

Proposal 1: There is no need to provide device location in Command related message(s).

About whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report:

As the selection of gNB is performed by the AIOTF before initiating Inventory Request, the AIOTF must knows which gNB the Inventoy Request will be sent to. Considering of indirect communication via the AMF, it is safer to include the Global gNB ID once in the Inventory Report Transfer IE in an INVENTORY REPORT message, and then for the device location report related IEs in the INVENTORY REPORT message, only providing Reader Index is sufficient.

Proposal 2: Include Global gNB ID once in the Inventory Report Transfer IE in an INVENTORY REPORT message, and include Reader Index in the per reader report part.

About Reader Index

There was no consensus on the maximum number of the reader index in the offline discussion last meeting, maybe the following can be considered as references:

In TS 38.423, the NR Cell Identify is defined as BIT STRING (SIZE(36)), the leftmost bits of the NR Cell Identity IE correspond to the gNB ID (defined in subclause 9.2.2.1). It means that there are 4 to 14bits left to uniquely identifies a cell within a gNB, note that 214 = 16384.

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

In TS 38.455, the TRP Iidentifier is defined as BIT STRING (SIZE(16, …))

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535 |

Proposal 3: Define the Reader Index as INTEGER BIT STRING (SIZE(16, …)).

# 3	Discussion and proposals

In this contribution, we discussed the leftovers on device location, and got the following proposals:

Proposal 1: There is no need to provide device location in Command related message(s).

Proposal 2: Include Global gNB ID once in the Inventory Report Transfer IE in an INVENTORY REPORT message, and include Reader Index in the per reader report part.

Proposal 3: Define the Reader Index as BIT STRING (SIZE(16, …)).

Based on the agreements achieved in last meeting, and the proposals above, the related TPs baseline CRs are provided in section 5 and 6.

With these, in our view, the discussion of this topic can be considered as completed.

# 4	TP to TS 38.300 BL CR

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 16.xx	Support of Ambient IoT

### 16.xx.x1	Introduction

### 16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	As specified in <reference to SA2 TS to be added> the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct communication to AIOTF) or the AMF (in case of indirect communication to AIOTF).

NOTE 1:	It is not expected a deployment will deploy both direct communication and indirect communication.

-	In this version of the specification A-IoT specific data is transported between the gNB and the A-IoT CN node takes place via the NG-C interface.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

Editor’s Note 2:	WA: Including AIoTF information in NGAP.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Editor’s Note 3:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request may include information concerning one or multiple A-IoT devices.

Editor’s Note 4:	Specification of further parameterisation of the Inventory Request message (e.g., assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result(s) from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node, to provide the inventory result(s) and from which reader(s) the result(s) is received. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report message may include inventory result received from multiple A-IoT devices.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

# 5	TP to TS 38.413 BL CR

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.xx.2	Inventory Report

#### 8.xx.2.1	General

The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the A-IoT CN node. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.2.2	Successful Operation

Figure 8.xx.2.2-1: Inventory Report

The NG-RAN node initiates the procedure by sending an INVENTORY REPORT message for the Inventory service.

#### 8.xx.2.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.aa1	Reader Index

This IE represents identifier of a Reader within a gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reader Identity | M |  | BIT STRING (SIZE(16, …)) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.x.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Global gNB ID | M |  | 9.3.1.6 |  |
| Per Reader Report List |  | 0..1 |  |  |
| >Per Reader Report Item |  | 1..< maxnoofReaders> |  |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| >>Device Report List |  | 1 |  |  |
| >>>Device Report Item |  | 1..<maxnoofDevices> |  |  |
| >>>>A-IoT NAS PDU | M |  | OCTET STRING |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 65535. |
| maxnoofDevices | Maximum no. of Devices within a reader. Value is 65535. |

-----------------Start of the Next  Change-----------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

//skip unchanged part

maxnoofCellsTSS,

maxnoofPeriodicities,

maxnoofPartiallyAllowedS-NSSAIs,

maxnoofRSPPQoSFlows,

maxnoofReaders,

maxnoofDevices

FROM NGAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

//skip unchanged part

AdditionalCancelledlocationReportingReferenceIDItem ::= SEQUENCE {

locationReportingReferenceIDToBeCancelled		LocationReportingReferenceID,

iE-Extensions		ProtocolExtensionContainer { { AdditionalCancelledlocationReportingReferenceIDItem-ExtIEs} }	OPTIONAL,

...

}

AdditionalCancelledlocationReportingReferenceIDItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-DeviceReportList ::= SEQUENCE (SIZE(1..maxnoofDevices)) OF AIoT-DeviceReportItem

AIoT-DeviceReportItem ::= SEQUENCE {

aIoT-NASPDU					OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { { AIoT-DeviceReportItem-ExtIEs} } OPTIONAL,

...

}

AIoT-DeviceReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-PerReaderReportList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-PerReaderReportItem

AIoT-PerReaderReportItem ::= SEQUENCE {

readerIndex					AIoT-ReaderIndex,

deviceReportList			AIoT-DeviceReportList,

iE-Extensions		ProtocolExtensionContainer { { AIoT-PerReaderReportItem-ExtIEs} } OPTIONAL,

...

}

AIoT-PerReaderReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-ReaderIndex ::= BIT STRING (SIZE(16, ...))

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevelARP				PriorityLevelARP,

pre-emptionCapability			Pre-emptionCapability,

pre-emptionVulnerability		Pre-emptionVulnerability,

iE-Extensions		ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

//skip unchanged part

InventoryReportTransfer ::= SEQUENCE {

globalgNB-ID				GlobalGNB-ID,

perReaderReportList			AIoT-PerReaderReportList,

iE-Extensions				ProtocolExtensionContainer { { InventoryReportTransfer-ExtIEs} }		OPTIONAL,

...

}

InventoryReportTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

//skip unchanged part

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

//skip unchanged part

maxnoofCellsinNGRANNode						INTEGER ::= 16384

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDevices								INTEGER ::= 65535

maxnoofDRBs									INTEGER ::= 32

maxnoofEmergencyAreaID						INTEGER ::= 65535

maxnoofEAIforRestart						INTEGER ::= 256

maxnoofEPLMNs								INTEGER ::= 15

//skip unchanged part

maxnoofQosFlows								INTEGER ::= 64

maxnoofQosParaSets							INTEGER ::= 8

maxnoofRANNodeinAoI							INTEGER ::= 64

maxnoofReaders								INTEGER ::= 65535

maxnoofRecommendedCells						INTEGER ::= 16

maxnoofRecommendedRANNodes					INTEGER ::= 16

-----------------End of the Changes-----------------