3GPP TSG-RAN WG3 Meeting #127bis	R3-251614

Wuhan, China, 7-11 April 2025

Agenda Item:	11.4

Source:	ZTE Corporation

Title:	Discussion on split architecture

Document for:	Discussions & Approval

1	Introduction

This contribution aligns with the objectives [1] of the Rel-19 WI on 'Enhancements for AI/ML in NG-RAN.' Based on the conclusions and progress made in Rel-18 Leftovers, as captured in TR 38.743 [2], we present our analysis of the normative work for NR-DC Mobility Optimization, Support of split architecture, and Continuous MDT Collection.

The detailed objectives of the WI are listed as follows:

- Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3]

- Support of the Leftovers in Rel-18 AI/ML for NG-RAN [RAN3]:

  - Mobility Optimization for NR-DC

  - Split architecture support for Rel-18 use cases

  - Continuous MDT collection targeting the same UE across RRC states

Note: Coordination with RAN2, SA5 when needed if any.

The aim of this work item is to specify new AI/ML-based use cases and introduce further enhancements to finalize the Rel-18 leftovers based on the conclusions captured in TR 38.743.

Regarding the support of split architecture, we have following agreements:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

In split architecture, the Energy Cost of gNB is the sum of the Energy Cost of its gNB-DUs.

Mapping rule is only configured at gNB-DU and shall be unified for all the gNB-DUs within a defined area.

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

In this contribution, we further provide our views on the impacts on UE performance metrics and measured EC over E1&F1 interfaces. Additionally, the corresponding TPs are provided.

2	Discussion

2.1 UE performance feedback over E1 and F1

Following is the conclusion on UE performance feedback:

The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need further discussion during normative phase.

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

Based on the agreements above, it can be acknowledged that the packet delay measured for UE performance feedback should be transferred from the CU-UP to the CU-CP upon request. Additionally, the packet delay reported from the DU to the CU can be supported via DDDS in the CU-DU split architecture. Therefore, it is the responsibility of the CU-UP to aggregate the various packet delay measurements.

- CU-UP aggregates packet delay measurements and reports to the CU-CP upon request.

Currently, there is no procedure defined over the E1 interface allowing the CU-CP to request specific UE performance metrics from the CU-UP. To address this gap and ensure consistency with existing mechanisms such as the Xn interface procedures, we propose introducing a new "Data Collection Procedure" on the E1 interface. This procedure will enable the CU-CP to proactively initiate requests to the CU-UP, specifying the targeted UE performance metrics, including packet delay metrics.

- Introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over the E1 interface, similar to the Xn Data Collection Procedure, to enable CU-CP to request and obtain packet delay metrics from CU-UP upon demand.

Regarding UE performance metrics—throughput, as described in TS 28.552 and TS 28.558, UE throughput is typically measured at the PDCP layer by the CU-UP, which maintains comprehensive visibility of per-UE downlink and uplink traffic bytes and rates. Although the DU can measure MAC-layer transmission data, it lacks complete visibility, especially when traffic crosses multiple RLC bearers or QoS flows. For UE performance metrics related to throughput, there is no impact on the F1 interface. Instead, utilize the newly proposed E1 Data Collection Procedure to enable the CU-CP to request and obtain throughput measurements from the CU-UP.

- Utilize the newly proposed E1 Data Collection Reporting Initiation and Data Collection Reporting procedures to enable the CU-CP to request and obtain UE throughput measurements from the CU-UP.

Regarding UE performance metrics—DL packet loss, the measurement and detection of packet loss typically occur at the PDCP layer within the CU-UP. DL packet loss can be determined either by missing sequence numbers at the PDCP layer or by not receiving ACK/NACK feedback within the expected time. Although the DU can measure MAC-layer transmission issues, the DU lacks the end-to-end packet loss visibility provided by the CU-UP.

- Utilize the newly proposed E1 Data Collection Reporting Initiation and Data Collection Reporting procedures to enable the CU-CP to request and obtain downlink packet loss metrics measured at the CU-UP.

As analysed above, CU-UP can measure and aggregate UE performance metrics, i.e., UE throughput UL/DL, Packet Delay UL/DL, and Packet Loss DL. There is no impact over F1 to support UE performance metrics in the case of split architecture.

- There is no impact on the F1 interface for UE performance metrics – UE throughput UL/DL, Packet Delay UL/DL, and Packet Loss DL.

2.2 Measured EC over F1 interface

Regarding the EC metrics, we previously agreed to introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over the F1 interface to transfer measured EC from DU to CU. However, some companies expressed the desire to revisit this agreement after receiving feedback from SA5 and observing progress regarding UE performance feedback.

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

At this meeting, we have received the reply LS [3] from SA5, indicating an update to their specification. Therefore, RAN3 needs to continue the work to enhance the F1 interface to support the transfer of EC. Upon further consideration, we have observed that the F1 interface currently only needs to transfer the measured EC. Introducing a new dedicated procedure for this limited usage seems excessive.

Hence, we propose to revert the previous agreement on introducing a new Data Collection Reporting procedure over the F1 interface for Energy Cost. Instead, utilize existing Resource Status Reporting procedures to transfer the measured Energy Cost from DU to CU.

- Reuse the existing Resource Status Reporting procedures to transfer the measured Energy Cost from DU to CU.

The encoding format of measured energy cost over F1 interface shall follow the agreement of measured energy cost over Xn interface.

- Define the Energy Cost IE over F1 interface as an INTEGER (0..10000, ...).

3	Conclusion

Following is our proposals and observations:

- CU-UP aggregates packet delay measurements and reports to the CU-CP upon request.

- Introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over the E1 interface, similar to the Xn Data Collection Procedure, to enable CU-CP to request and obtain packet delay metrics from CU-UP upon demand.

- Utilize the newly proposed E1 Data Collection Reporting Initiation and Data Collection Reporting procedures to enable the CU-CP to request and obtain UE throughput measurements from the CU-UP.

- Utilize the newly proposed E1 Data Collection Reporting Initiation and Data Collection Reporting procedures to enable the CU-CP to request and obtain downlink packet loss metrics measured at the CU-UP.

- There is no impact on the F1 interface for UE performance metrics – UE throughput UL/DL, Packet Delay UL/DL, and Packet Loss DL.

- Reuse the existing Resource Status Reporting procedures to transfer the measured Energy Cost from DU to CU.

- Define the Energy Cost IE over F1 interface as an INTEGER (0..10000, ...).

4	Reference

[1] RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC

[2] TR 38.743

[3] R3-251529, LS reply to LS on energy saving for split gNB, SA5