3GPP TSG-RAN WG3 Meeting #127bis                                                              R3-251619

Wuhan, China, April 7 – 11, 2025

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	Remaining aspects for WAB

Document for:		Discussion

# 1	Introduction

This contribution discusses remaining issues of WAB related to:

- Preventability of multi-hop WAB during WAB-MT access

- Preventability of inter-WAB-gNB Xn establishment

- Dual connectivity support by WAB-gNB

# 2	Discussion

## 2.1	Preventability of multi-hop WAB during WAB-MT network access

RAN3#126 agreed on the preventability of multi-hop WAB during WAB-MT mobility [1]:

For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

It remains to be discussed on how multi-hop WAB can be prevented during WAB-MT access. Based on offline discussion in last meeting, it would be desirable to support a solution that does not require WAB-specific enhancements to the UE stack, since such enhancement might be economically viable for a rollout of a small number of WAB-nodes.

The following alternatives can be considered:

Alternative 1: Using forbidden TAC list

The WAB-MT is configured with a list of forbidden TACs, which contains all TACs used by WAB-gNBs. The BH RAN is configured to broadcast TACs that are different from those used by WAB-gNBs.

Alternative 2: WAB-MT is handed over to a BH RAN node when accessing a WAB-gNB cell.

When the WAB-MT connects to a WAB-gNB cell, the WAB-gNB conducts a handover of the WAB-MT to a BH RAN node. The WAB-gNB identifies the WAB-MT by the WAB-specific S-NSSAI included in Msg. 5.

Both alternatives use legacy UE functionality for the WAB-MT. They all can be supported based on implementation. There is no need to define other solutions, which define a new behaviour or stage-3 enhancements for the WAB-MT.

In RAN3#126 meeting, one operator raised concerns that allocating additional identifier space for WAB would overly restrict deployments. For Alt1, it is sufficient to allocate one TAC for WAB out of 4x106 TAC space. Such allocations should not pose any constraint on actual deployments. For Alt2, no additional identifier is needed for the prevention of multi-hop WAB.

Proposal 1: Multi-hop WAB during WAB-MT access is preventable via implementation, e.g., by using one of the following alternatives:

- Alt. 1: WAB-gNB broadcasts WAB-specific TACs, which are contained in WAB-MT’s forbidden TAC list,

- Alt. 2: In case WAB-MT connects to WAB-gNB, it is handed over to BH RAN.

## 2.2	Preventability of inter-WAB-gNB Xn establishment

According to the WID for RAN’s WI on Additional Topological Enhancements for NR, RAN3 should define the support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs [2].

To prevent the Xn establishment between WAB-gNBs, a WAB-gNB needs to be made aware whether a neighbour node is a WAB-gNB or another RAN node. For this purpose, WAB-gNBs can be configured with a reserved range of PCI values, gNB-ID values, NR-CGI values and/or TAC values that are not used by the BH RAN in the relevant area of the WAB-nodes’ operation. When a WAB-gNB initiates the Xn Setup procedure toward a peer WAB-node, the peer WAB-node can identify that the request was initiated by a WAB-node since the served cells use some of these WAB-reserved parameters.

This mechanism for preventability of inter-WAB-gNB Xn establishment does not require any specification enhancement.

Proposal 2: Xn establishment between WAB-gNBs is preventable via implementation, e.g., by having the WAB-gNB reject the Xn Setup request from peer gNB that uses WAB-reserved ranges of PCIs, gNB-IDs, NR CGIs and/or TACs.

## 2.3	Dual-connectivity support by WAB-gNB

In last meeting, the issue was raised whether the WAB-gNB should be able to support NR DC [3].

WAB may be deployed to support mobile or nomadic use cases.

For nomadic use cases, when the WAB-node is stationary, it is certainly feasible for the WAB-gNB to support NR DC. Also, UEs may benefit from NR DC in such scenarios. In such use case, the WAB-gNB could support the role as MN or SN based on implementation.

In case of high WAB-node mobility, support of NR DC for UEs may not be desirable. In this case, the WAB-gNB can avoid DC by rejecting SN setup requests.

Proposal 3: WAB-gNB can support DC for UEs based on implementation.

# 3	Conclusion

This contribution discussed the aspects of WAB. The following observations and proposals have been made:

Preventability of multi-hop WAB during WAB-MT network access

Proposal 1: Multi-hop WAB during WAB-MT access is preventable via implementation, e.g., by using one of the following alternatives:

- Alt. 1: WAB-gNB broadcasts WAB-specific TACs, which are contained in WAB-MT’s forbidden TAC list,

- Alt. 2: In case WAB-MT connects to WAB-gNB, it is handed over to BH RAN.

Preventability of inter-WAB-gNB Xn establishment

Proposal 2: Xn establishment between WAB-gNBs is preventable via implementation, e.g., by having the WAB-gNB reject the Xn Setup request from peer gNB that uses WAB-reserved ranges of PCIs, gNB-IDs, NR CGIs and/or TACs.

Dual-connectivity support by WAB-gNB

Proposal 3: WAB-gNB can support DC for UEs based on implementation.

# Reference

[1]  Chair notes, 3GPP TSG RAN WG3 Meeting #126, Orlando, Florida, USA, November 2024

[2]  RP-242395, New WID on additional topological enhancements for NR, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 2024

[3]  Chair notes, 3GPP TSG RAN WG3 Meeting #127, Athens, Greece, February 2025