3GPP TSG-RAN WG3 Meeting #127bis                                                              R3-251620

Wuhan, China, April 7 – 11, 2025

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	WAB radio resource coordination

Document for:		Discussion

# 1	Introduction

This contribution discusses resource coordination between WAB access and backhaul for “in-band” operation based on the objectives in WID on Additional Topological Enhancements for NR [1].

# 2	Discussion

## 2.1	Resource coordination

The WAB-node is expected to concurrently exchange access traffic with UEs and backhaul traffic with the BH RAN. Based on the frequencies used on access and BH interfaces, multiplexing constraints need to be considered for such concurrent traffic exchange, e.g., one of the WAB-node’s radio interfaces may not be able to receive while the other radio interface is transmitting.

For out-of-band operation of access and backhaul, it is generally assumed that such multiplexing constraints can be handled via implementation and no specification enhancements are needed.

Proposal 1: When access and backhaul are operated out-of-band, multiplexing constraints can be handled via implementation and no specification enhancements are needed.

When access and backhaul are operated in-band, multiplexing constraints may have to be addressed via coordination of resource allocation.

Such multiplexing issues were extensively studied for IAB. Further, for IAB, RAN1/2/3 defined mechanisms for the coordination of radio resources between the IAB-node’s parent and child link. Since the multiplexing issues are essentially the same for WAB as for IAB, the resource coordination solutions developed for IAB can be leveraged.

For WAB, the exacerbating issue arises that the WAB-node’s access and backhaul may be supported by different PLMNs, where inter-PLMN resource coordination is not possible. Further, even when access and backhaul are supported by the same PLMN, it may be desirable to operate WAB-nodes with a legacy BH RAN, which has not been upgraded with WAB-specific enhancements. In such cases, WAB can still be supported via out-of-band operation.

Proposal 2a: RAN3 assumes that WAB deployments use out-of-band operation in case access and backhaul use different PLMNs or when the BH RAN cannot be upgraded with WAB-specific enhancements.

Proposal 2b: In-band operation is only considered for scenarios, where access and backhaul use the same PLMN and the BH RAN can be upgraded with WAB-specific enhancements.

For in-band operation, the radio resource coordination problem between a WAB-node’s access and backhaul link is analogue to that of an IAB-node whose IAB-MT and IAB-DU are connected to a different IAB-donors. Therefore, an analogue signalling solution can be used for WAB as it has been defined for IAB.

Fig. 1 shows a comparison between the IAB scenario and the corresponding WAB scenario. The following analogies can be identified between both scenarios:

- In the WAB-scenario, the BH gNB CU corresponds to the IAB-MT’s IAB-donor-CU, and the WAB-gNB-CU corresponds to the IAB-DU’s IAB-donor-CU.

- Resource coordination between WAB gNB-CU and BH gNB-CU can use an equivalent procedure as the XnAP IAB RESOURCE COORDINATION procedure.

- Resource coordination between BH gNB-CU and BH gNB-DU can reuse the F1 gNB-DU RESOURCE CONFIGURATION procedure.

- Resource coordination between WAB gNB-DU and WAB gNB-CU is out-of-scope since both functions are assumed to be collocated.

Figure 1: Resource coordination for (m)IAB and for WAB using the legacy IAB resource coordination procedures

Proposal 3a: For in-band operation, the Xn information exchange between WAB-gNB and BH gNB to follow the solution defined for IAB between F1-terminating IAB-donor and non-F1-terminating IAB-donor.

Proposal 3b: For in-band operation, the F1 information exchange between BH gNB-CU and BH gNB-DU to follow the solution defined for IAB between non-F1-terminating IAB-donor-CU and non-F1-terminating IAB-donor-DU.

Proposal 4a: For Xn information exchange between WAB-gNB-CU and BH gNB-CU, introduce a new class-1 procedure that exchanges for each WAB-gNB cell the information provided by the XnAP IAB Cell Information IE.

Proposal 4b: For F1 information exchange between BH gNB-CU and BH gNB-DU, reuse the F1AP GNB-DU RESOURCE CONFIGURATION procedure.

The XnAP IAB RESOURCE COORDINATION procedure includes the UE XnAP IDs of the IAB-MT. These identifiers are introduced in the IAB-MT’s HO PREPARATION procedure (or SN ADDITION procedure), which occurs before to the Xn IAB RESOURCE COORDINATION procedure.

For WAB, there is no equivalent UE-associated Xn exchange for the WAB-MT between WAB-gNB and WAB-MT’s BH RAN-node. The UE XnAP IDs therefore need to be introduced in the XnAP resource coordination procedure itself.

Since WAB-node and BH gNB can establish an Xn connection before they establish a BH link, it is possible to pre-emptively conduct resource coordination between both RAN nodes. The WAB-node then applies the resulting resource allocation as soon as the WAB-MT has been handed over to this BH gNB.

Proposal 5: A WAB-node and a BH-gNB can pre-emptively conduct resource coordination via Xn, i.e., before they have established a BH link. The WAB-node applies the pre-emptively established resource allocation as soon as it is connected to this BH gNB.

For IAB, the specification does not define the IAB-donors’ behaviour on how to resolve conflicting resource allocations. Since both IAB-donors are mutual peers without hierarchy, such conflict resolution has been left up to implementation. For WAB, the situation is different for the following reasons:

- The WAB-node is expected to support rather few UEs opposed to the BH gNB.

- There may be multiple WAB-nodes connecting to a BH gNB.

- The WAB-nodes may dynamically move in and out of the BH gNB’s coverage.

To ensure stable resource coordination between BH gNB and the various WAB-nodes connected, it might make sense to have the BH gNB determine the resource allocation across of its own cells and the cells of all WAB-nodes.

Proposal 6: The BH gNB to determine the resource allocation across its own cells and the cells of all WAB-nodes.

## 2.1	WAB-gNB/MT collocation discovery

The WAB-gNB/MT collocation discovery allows the WAB-MT’s BH RAN node to discover the collocation between the WAB-gNB and the WAB-MT. It addresses the same issue as the topology discovery in IAB. However, opposed to IAB, enhancements for multi-hop WAB topologies are not handled in this release. Further, even if a multi-hop WAB topology were formed, the BH RAN node would not be aware of it since it only has visibility of the first tier WAB-MT.

Based on RAN3 discussion so far, the WAB-gNB/MT collocation discovery is only needed for WAB resource coordination. To enable such resource discovery, the WAB-gNB can send an WAB-MT-specific indicator to the BH RAN-node via Xn, which the BH RAN node has assigned to this WAB-MT before. It is possible to use the WAB-MT’s C-RNTI assigned by the WAB-MT’s served cell for this purpose in combination with the WAB-MT’s served cell CGI. These indicators can be exchanged via the Xn SETUP PROCEDURE and/or the Xn RAN-node CONFIGURATION UPDATE procedure. This information should also be included in the Xn resource coordination procedure between WAB-gNB and BH gNB.

Proposal 7: For WAB-gNB/MT collocation discovery, the WAB-gNB to send the collocated WAB-MT’s C-RNTI obtained from the WAB-MT’s served cell and the served cell’s CGI to the BH RAN node via the Xn SETUP PROCEDURE and/or the Xn RAN-node CONFIGURATION UPDATE procedure.

# 3	Conclusion

This contribution discussed resource coordination between WAB access and backhaul. The following proposals have been made:

Resource coordination

Proposal 1: When access and backhaul are operated out-of-band, multiplexing constraints can be handled via implementation and no specification enhancements are needed.

Proposal 2a: RAN3 assumes that WAB deployments use out-of-band operation in case access and backhaul use different PLMNs or when the BH RAN cannot be upgraded with WAB-specific enhancements.

Proposal 2b: In-band operation is only considered for scenarios, where access and backhaul use the same PLMN and the BH RAN can be upgraded with WAB-specific enhancements.

Proposal 3a: For in-band operation, the Xn information exchange between WAB-gNB and BH gNB to follow the solution defined for IAB between F1-terminating IAB-donor and non-F1-terminating IAB-donor.

Proposal 3b: For in-band operation, the F1 information exchange between BH gNB-CU and BH gNB-DU to follow the solution defined for IAB between non-F1-terminating IAB-donor-CU and non-F1-terminating IAB-donor-DU.

Proposal 4a: For Xn information exchange between WAB-gNB-CU and BH gNB-CU, introduce a new class-1 procedure that exchanges for each WAB-gNB cell the information provided by the XnAP IAB Cell Information IE.

Proposal 4b: For F1 information exchange between BH gNB-CU and BH gNB-DU, reuse the F1AP GNB-DU RESOURCE CONFIGURATION procedure.

Proposal 5: A WAB-node and a BH-gNB can pre-emptively conduct resource coordination via Xn, i.e., before they have established a BH link. The WAB-node applies the pre-emptively established resource allocation as soon as it is connected to this BH gNB.

Proposal 6: The BH gNB to determine the resource allocation across its own cells and the cells of all WAB-nodes.

WAB-gNB/MT collocation discovery

Proposal 7: For WAB-gNB/MT collocation discovery, the WAB-gNB to send the collocated WAB-MT’s C-RNTI obtained from the WAB-MT’s served cell and the served cell’s CGI to the BH RAN node via the Xn SETUP PROCEDURE and/or the Xn RAN-node CONFIGURATION UPDATE procedure.

# Reference

[1]  RP-242395, New WID on additional topological enhancements for NR, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 2024