3GPP TSG-RAN WG3 Meeting #127bis                                                              R3-251622

Wuhan, China, April 7 – 11, 2025

Agenda item: 		12.3

Source: 		Qualcomm Incorporated

Title: 	(TP to draft BL CR to 38.300) Discussion on NR Femto node shared by PLMN and PNI-NPN

Document for:		Other

# 1	Introduction

This contribution discusses SA2’s LS on NR Femto node shared by PLMN and PNI-NPN in R3-251524/S2-2502787. The LS states:

1. Overall description SA2 discussed access control description agreed in RAN3, as captured in RAN3 BL CR to TS 38.300 (R3-247936). - The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.. Among above three options, SA2 seeks some clarifications regarding the second option. As described in TS 23.501 clause 5.30.3.4 for PNI-NPN, the AMF and NG-RAN would reject the UE if none of the CAG ID of the cell are part of the UE's Allowed CAG list. As described in TS 23.501 clause 5.18, the MOCN sharing between the PLMN and PNI-NPN is achieved by using different cell IDs with separate configuration. Therefore, SA2 would like to understand whether the second option in the RAN3 BL CR for NR Femto contradicts with the existing SA2 specifications. 2. Actions To RAN3: SA2 kindly asks RAN3 to clarify whether the second option in the RAN3 BL CR has any contradictions with the existing definitions in SA2 specifications regarding PNI-NPN and Network Sharing.

# 2	Discussion

In this LS, SA2 emphasizes that according to 23.501, a logical cell cannot be associated with a PLMN and with a PNI-NPN/CAG at the same time. If a cell were configured in this manner, a UE accessing this cell via the PLMN would be rejected since none of the CAG IDs of the cell are part of the UE’s Allowed CAG list.

The LS further emphasizes that it is possible for the Femto to support separate logical cells, one supporting a PLMN and the other supporting a PNI-NPN/CAG.

SA2’s LS is in line with TS 38.300, which states in clause 4.6:

## 4.6 Radio Access Network Sharing NG-RAN supports radio access network sharing as defined in TS 23.501 [3]. If NR access is shared, system information broadcast in a shared cell indicates a TAC and a Cell Identity for each subset of PLMNs, PNI-NPNs and SNPNs. NR access provides only one TAC and one Cell Identity per cell per PLMN, SNPN or PNI-NPN. In this version of the specification, a Cell Identity can only belong to one network type among PLMN, PNI-NPN or SNPN as defined in TS 23.501 [3]. Each Cell Identity associated with a subset of PLMNs, SNPNs or PNI-NPNs identifies its serving NG-RAN node.

RAN3’s draft BL CR to 38.300 is not in contradiction with TS 23.501 or TS 38.300. However, the CR is lacking clarity in the following aspects:

- The BL CR uses the term “cell” as a reference to a physical cell, not a logical cell.

- The BL CR does not state that a physical cell broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1 should use different NR CGIs for PLMN and NPN.

The first bullet can be addressed by explicitly referring to “physical cell”.

The second bullet can be addressed by adding the clarification that according to TS 38.300, the physical cell needs to use different NR CGIs for PLMN and PNI-NPN. Such clarification may not be strictly necessary since this constraint has already been captured in TS 38.300 but is will certainly avoid misunderstandings.

The Annex below includes a modification to the draft BL CR that addresses these two issues.

Proposal 1: RAN3 believes that the second option for access control in RAN3’s draft BL CR for NR Femto is in line with legacy specifications by clarifying the following:

- The term “cell” in this section refers to “physical cell”.

- A Femto with a physical cell broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in SIB1 needs to use different NR CGIs for PLMN and PNI-NPN.

Proposal 2: Add the above clarifications to the draft BL CR to TS 38.300 as provided in the Annex.

Proposal 3: Send Reply LS to SA2 with agreed proposal 1 and updated draft BL CR as provided in R3-25xxxx.

# 3	Conclusion

This contribution discussed the LS by SA2 on NR Femto node shared by PLMN and PNI-NPN. The following proposals have been made:

Proposal 1: RAN3 believes that the second option for access control in RAN3’s draft BL CR for NR Femto is in line with legacy specifications by clarifying the following:

- The term “cell” in this section refers to “physical cell”.

- A Femto with a physical cell broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in SIB1 needs to use different NR CGIs for PLMN and PNI-NPN.

Proposal 2: Add the above clarifications to the draft BL CR to TS 38.300 as provided in the Annex.

Proposal 3: Send Reply LS to SA2 with agreed proposal 1 and updated draft BL CR as provided in R3-25xxxx.

# Annex: TP to draft BL CR to TS 38.300

BEGIN OF CHANGES

### 4.x.4	Access Control

- Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription.

NOTE: The NR Femto node may use the CAG mechanism for PNI-NPN (see clause 16.7.4) as follows:

-	The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	The NR Femto may activate a physical cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this physical cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. The physical cell needs to use different NR CGIs for PLMN and PNI-NPN as defined in clause 4.6.

-	The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

END OF CHANGES