3GPP TSG-RAN WG3 Meeting #127bis                                                           R3-251624

Wuhan China, April 7 – 11, 2025

Agenda item: 		17.3

Source: 		Qualcomm Incorporated

Title: 	Aspects of on-demand SIB1 for NES enhancements

Document for:		Discussion

# 1	Introduction

In last meeting, RAN3 agreed on aspects related to OD SIB1:

It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity. It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx. It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

This contribution aims to make further progress on the support for on-demand SIB1.

# 2	Discussion

## 2.1	Necessary Xn signaling enhancements

In RAN3#126, RAN3 introduced the Xn UL WUS CONFIGURATION PROVISIONING procedure (naming further to be discussed). In the last two meetings, RAN3 agreed that the Xn UL WUS CONFIGURATION PROVISIONING REQUEST message can be used for the following purposes [1][2]:

- Provide Cell-A gNB with UL WUS configuration

- Request Cell-A gNB to start transmission of UL WUS configuration

RAN3 further agreed on signaling from NES gNB to Cell-A gNB with the purpose:

- Request Cell-A gNB to stop transmission of UL WUS configuration

It should also be possible for the NES gNB to request Cell-A gNB to discard an UL WUS configuration.

Proposal 1-1: In addition to provisioning an UL WUS configuration and requesting start/stop transmission of such a configuration, the NES gNB can request the Cell-A gNB to discard an UL WUS configuration.

When the NES gNB sends an UL WUS configuration to a Cell-A gNB, it needs to include the list of NES cells to which this configuration applies.

Proposal 1-2: When the NES gNB sends to Cell-A gNB an UL WUS configuration and/or the request for starting, stopping the transmission of such a configuration, the NES gNB to include the list of NES cells to which this configuration applies.

When the NES gNB sends to Cell-A gNB the request for starting or stopping the transmission of an UL WUS configuration, the cell-A gNB should respond with a list of cell-A’s where the UL WUS configuration is transmitted. This allows the NES gNB to determine whether a NES cell can support OD SIB1 mode.

Proposal 1-3: In response to the NES gNB’s request to start/stop an UL WUS configuration transmission, the Cell-A gNB should provide a list of cell-As where the UL WUS configuration is transmitted.

The cell-A gNB should also be able to send an unsolicited message to the NES gNB, where it informs the NES gNB about an update of cell-As where an UL WUS configuration is transmitted.

Proposal 1-4: The cell-A gNB can send to NES gNB an unsolicited update on the list of cell-A’s where an UL WUS configuration is transmitted.

RAN3 introduced the class-1 XnAP UL WUS CONFIGURATION PROFISIONING procedure. This procedure can be used for the following purposes:

- NES gNB provisions Cell-A gNB with an UL WUS configuration, or it requests Cell-A gNB to discard an UL WUS configuration provisioned before. Cell-A gNB confirms the acceptance of this message.

- NES gNB sends start/stop indication for an UL WUS configuration with a list of affected NES cells. NES gNB responds with a list of cell-A’s where the UL WUS configuration is transmitted.

Proposal 1-5: The XnAP UL WUS CONFIGURATION PROVISIONING REQUEST message to be used by NES gNB for provisioning an UL WUS configuration and request start/stop or discard of this configuration.

Proposal 1-6: The XnAP UL WUS CONFIGURATION PROVISIONING RESPONSE message to be used by cell-A gNB to indicate the list of cell-A’s where the transmission of an UL WUS configuration is started or stopped.

RAN3 also introduced the class-2 Xn UL WUS CONFIGURATION TRANSMISSION STATUS procedure. This procedure can be used by the by cell-A gNB for the unsolicited update of the list of cell-A’s where the transmission of an UL WUS configuration is started or stopped.

Proposal 1-7: The Xn UL WUS CONFIGURATION TRANSMISSION STATUS procedure to be used by the cell-A gNB for an unsolicited update of the cell-A’s where the transmission of an UL WUS configuration is started or stopped.

## 2.2	Necessary F1 signaling enhancements

The gNB-DU needs to inform the NES gNB-CU about the NES cells that can support OD SIB1 and those, for which NES support is revoked. This allows the NES gNB to ensure proper support by cell-A’s, and to ask cell-A gNBs to stop UL WUS configuration transmissions in case they are not needed anymore.

Proposal 2-1: The NES gNB-DU to inform the NES gNB-CU about the NES cells that can support OD SIB1 operation.

The NES gNB-CU should decide, which NES cell can conduct OD-SIB1 operation based on the cell-A’s that transmit the corresponding UL WUS configuration. It should then inform the NES gNB-DU about the NES cells that are permitted to operate in OD SIB1 mode.

Proposal 2-2: The NES gNB-CU to inform the NES gNB-DU about the NES cells, which are permitted to operate in OD SIB1 mode.

The cell-A gNB-CU should request the cell-A gNB-DU to conduct transmission of a UL WUS configurations for specific cell-A’s. It should also be able to request the cell-A gNB-DU to stop transmission of an UL WUS configuration for specific cell-A’s.

Proposal 2-3: The cell-A gNB-CU to request the cell-A gNB-DU to start or stop transmission of WUS configurations for specific cell-A’s.

The cell-A gNB-DU should inform the cell-A gNB-CU whether the transmission status of an UL WUS configuration has changed for a cell-A.

Proposal: 2-4: The cell-A gNB-DU to inform the cell-A-CU in response or unsolicited, whether the transmission status of an UL WUS configuration has changed for an cell-A.

Since any changes related to OD-SIB1 support and transmission of UL WUS configurations are expected to occur on semi-statically, all F1 signaling on NES gNB and cell-A gNB related to OD-SIB1 can reuse legacy procedures.

Proposal 2-5: All OD-SIB1 related signaling on NES gNB and cell-A gNB to reuse legacy procedures.

# 3	Conclusion

This contribution aimed to make further progress on the support for on-demand SIB1. The following observations and proposals have been made:

Xn signaling enhancements

Proposal 1-1: In addition to provisioning an UL WUS configuration and requesting start/stop transmission of such a configuration, the NES gNB can request the Cell-A gNB to discard an UL WUS configuration.

Proposal 1-2: When the NES gNB sends to Cell-A gNB an UL WUS configuration and/or the request for starting, stopping the transmission of such a configuration, the NES gNB to include the list of NES cells to which this configuration applies.

Proposal 1-3: In response to the NES gNB’s request to start/stop an UL WUS configuration transmission, the Cell-A gNB should provide a list of cell-As where the UL WUS configuration is transmitted.

Proposal 1-4: The cell-A gNB can send to NES gNB an unsolicited update on the list of cell-A’s where an UL WUS configuration is transmitted.

Proposal 1-5: The XnAP UL WUS CONFIGURATION PROVISIONING REQUEST message to be used by NES gNB for provisioning an UL WUS configuration and request start/stop or discard of this configuration.

Proposal 1-6: The XnAP UL WUS CONFIGURATION PROVISIONING RESPONSE message to be used by cell-A gNB to indicate the list of cell-A’s where the transmission of an UL WUS configuration is started or stopped.

Proposal 1-7: The Xn UL WUS CONFIGURATION TRANSMISSION STATUS procedure to be used by the cell-A gNB for an unsolicited update of the cell-A’s where the transmission of an UL WUS configuration is started or stopped.

F1 signaling enhancements

Proposal 2-1: The NES gNB-DU to inform the NES gNB-CU about the NES cells that can support OD SIB1 operation.

Proposal 2-2: The NES gNB-CU to inform the NES gNB-DU about the NES cells, which are permitted to operate in OD SIB1 mode.

Proposal 2-3: The cell-A gNB-CU to request the cell-A gNB-DU to start or stop transmission of WUS configurations for specific cell-A’s.

Proposal: 2-4: The cell-A gNB-DU to inform the cell-A gNB-CU in response or unsolicited, whether the transmission status of an UL WUS configuration has changed for an cell-A.

Proposal 2-5: All OD-SIB1 related signaling on NES gNB and cell-A gNB to reuse legacy procedures.

# Reference

[1]  Chair notes, RAN WG3 meeting #126, Orlanda, FL, USA, Nov 2024

[2]  Chair notes, RAN WG3 meeting #127, Athens, Greece, Feb 2025