3GPP TSG-RAN WG3 Meeting #127bis	R3-251627

Wuhan, China, 07-11 April, 2025

Agenda Item:		11.2

Source:		Ofinno

Title:	AI/ML enabled slicing

Document for:		Discussion and decision

# Introduction

After RAN3 #127 meeting, the following progress has been achieved on the agenda item 11.2 AI/ML enabled slicing:

Enhance the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface.

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

Introduce Slice To Report List for Data Collection IE including PLMN ID and S-NSSAI in DATA COLLECTION REQUEST message.

RAN3#126:

First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status with list of Slice in the DATA COLLECTION REQUEST message.

Introduce the new bit for Predicted slice available capacity in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message.

Introduce a Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message to report the requested predicted slice available capacity to the requesting node.

Update the Semantics Description of Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message to indicate that this IE also includes the Slice Radio Resource Status List IE.

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node.

RAN3#127:

Introduce Slice Measurement Initiation Result IE to indicate the predicted radio resource status per slice or predicted slice available capacity cannot be initiated.

Whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity? To be checked in next meeting…

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback.

Other granularity of UE performance are not excluded and to be continued based on cases ...

This contribution discusses several aspects related to UE performance feedback.

# Discussion

## 2.1	UE performance feedback per slice

RAN3 has agreed to introduce slice-specific prediction reporting to the DATA COLLECTION procedure.

In particular, Slice To Report List for Data Collection is included to the DATA COLLECTION REQUEST:

| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | - |  |
| >>>>S-NSSAI List |  |  | 1 |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | - |  |

Also, Tenth Bit for requesting the Predicted Slice Available Capacity is added to the Report Characteristics for Data Collection:

| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Available Capacity Other bits are ignored by the NG-RAN node2. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

This allows NG-RAN node1 to request NG-RAN node2 to report predicted Slice Radio Resource Status and predicted Slice Available Capacity:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. If the Slice To Report List for Data Collection IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, include the Slice Radio Resource Status Item IE.

-	the Predicted Slice Available Capacity IE, if the tenth bit, "Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1"and if

- the Slice To Report List for Data Collection IE is included for the cell, and

- the measurement object is admitted by NG-RAN node2.

This allows, for example, AI/ML in the NG-RAN node1 to make, e.g., mobility, energy saving, load balancing, CCO decisions considering slice-level granularity. After such decisions, it is beneficial for the NG-RAN node1 to get slice-level feedback from the NG-RAN node2.

One form of such feedback would be slice-level measurements from the NG-RAN node2 (e.g., Slice Available Capacity measurements and Slice Radio Resource Status List in RESOURCE STATUS UPDATE message). But it is also beneficial for the AI/ML in the NG-RAN node1 to understand how its decisions have impacted UEs handed over from the NG-RAN node1 to the NG-RAN node2. For this purpose, it is beneficial to have a possibility for the NG-RAN node1 to request the NG-RAN node2 to report the UE performance feedback per slice.

Proposal 1: Source node can request target node per slice reporting of UE performance feedback. If requested and supported, the target node can report to the source node per slice UE performance feedback.

## 2.2	Slice To Report List for Data Collection

Slice To Report List for Data Collection has been included to the DATA COLLECTION REQUEST:

| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | - |  |
| >>>>S-NSSAI List |  |  | 1 |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | - |  |

Slice Measurement Initiation Result has been included to the DATA COLLECTION RESPONSE:

#### 9.2.3.x1	Slice Measurement Initiation Result

This IE indicates the list of measurement objects that failed to be initiated per slice.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Measurement Initiation Result List |  | 1 |  |  |
| >Slice Measurement Initiation Result Item |  | 1..< maxnoofBPLMNs > |  |  |
| >>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >>S-NSSAI Measurement Initiation Result List |  | 1 |  |  |
| >>> S-NSSAI Measurement Initiation Result Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>> S-NSSAI Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the slice. |
| >>>>> S-NSSAI Measurement Failure Cause Item |  | 1 .. <maxFailedSliceMeasObjects> |  |  |
| >>>>> S-NSSAI Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Slice Available Capacity Other bits are ignored by the NG-RAN node1. |
| >>>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. |

A UE may not use all slices indicated in the Slice To Report List for Data Collection. As a result, NG-RAN node2 (target node) may not be able to report measured UE performance for all slices from the Slice To Report List for Data Collection.

Both DATA COLLECTION REQUEST and DATA COLLECTION RESPONSE messages do not use UE-associated signalling. The Slice Measurement Initiation Result parameter cannot be used to indicate measurement initiation results for a particular UE, because there is no UE ID in both DATA COLLECTION REQUEST and DATA COLLECTION RESPONSE messages.

There is a need for a new mechanism in RAN3 for a NG-RAN node2 (target node) to indicate to a NG-RAN node1 (source node) UE performance initiation results for a particular UE.

One possible solution is as follows.

The NG-RAN node2 (target node) reports the NG-RAN node1 (source node) UE performance feedback per slice only for those slices from the Slice To Report List for Data Collection, that are applicable for the UE handed over from the NG-RAN node1 to the NG-RAN node2. The NG-RAN node2 signals the list of applicable slices inside Slice UE Performance when reporting the UE performance feedback per slice.

This may be enhanced by adding cause values for those of the applicable slices for which the measurement results are not available (either per slice or per UE performance parameter). This will allow the NG-RAN node1 (source node) to understand why the measurements are not available.

This may be further enhanced as follows. The NG-RAN node1 (source node) explicitly signals to the NG-RAN node2 (target node), a UE-associated list of slices for reporting the UE performance feedback. For example, this may be included in the same HANDOVER REQUEST message and S-NODE ADDITION REQUEST message that initiates UE performance feedback reporting by the Data Collection ID IE. This will allow the NG-RAN node1 (source node) to request the UE performance measurement results for a part of slice applicable for the UE.

Proposal 2: The NG-RAN node2 (target node) reports the NG-RAN node1 (source node) UE performance feedback per slice only for those slices from the Slice To Report List for Data Collection, that are applicable for the UE handed over from the NG-RAN node1 to the NG-RAN node2. The NG-RAN node2 signals the list of applicable slices inside Slice UE Performance when reporting the UE performance feedback per slice.

Proposal 3: Cause values are added for those of the applicable slices for which the measurement results are not available (either per slice or per UE performance parameter).

Proposal 4: The NG-RAN node1 (source node) explicitly signals to the NG-RAN node2 (target node), a UE-associated list of slices for reporting the UE performance feedback. For example, this may be included in the same HANDOVER REQUEST message and S-NODE ADDITION REQUEST message that initiates UE performance feedback reporting by the Data Collection ID IE.

# Proposals

Proposal 1: Source node can request target node per slice reporting of UE performance feedback. If requested and supported, the target node can report to the source node per slice UE performance feedback.

Proposal 2: The NG-RAN node2 (target node) reports the NG-RAN node1 (source node) UE performance feedback per slice only for those slices from the Slice To Report List for Data Collection, that are applicable for the UE handed over from the NG-RAN node1 to the NG-RAN node2. The NG-RAN node2 signals the list of applicable slices inside Slice UE Performance when reporting the UE performance feedback per slice.

Proposal 3: Cause values are added for those of the applicable slices for which the measurement results are not available (either per slice or per UE performance parameter).

Proposal 4: The NG-RAN node1 (source node) explicitly signals to the NG-RAN node2 (target node), a UE-associated list of slices for reporting the UE performance feedback. For example, this may be included in the same HANDOVER REQUEST message and S-NODE ADDITION REQUEST message that initiates UE performance feedback reporting by the Data Collection ID IE.

# Reference

[1] R3-251501, RAN3#127bis Meeting Agenda

[2] RS-251502, RAN3#127 Meeting Report

[3] R3-251544, (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN