3GPP TSG-RAN WG3 Meeting #127bis	R3-251629

Wuhan, China, 7-11 April 2025

Agenda Item:		13.2

Source:		Ofinno

Title:	Data Forwarding on Inter-CU LTM in DC Scenario

Document for:		Discussion and Decision

# Introduction

In RAN3 #127 meeting, the following progress has been achieved on the agenda item 13.2, i.e., support for Inter-CU LTM NR-DC:

RAN3#127:

RAN3 agreed the following scenarios to support LTM with NR-DC:

1.SN initiated inter-CU SCG LTM without MCG changes (high priority)

2.Inter-CU MCG LTM without SN release

3.Inter-CU MCG LTM with SN release

4.Inter-CU MCG LTM with SN addition

For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

The MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

This contribution discusses the data forwarding issues on NR-DC LTM.

# Discussion

In Rel-17/18, we introduced several new IEs (e.g., CHO MR-DC Indicator, CHO MR-DC Early Data Forwarding Indicator, CPC Data Forwarding indicator) in XN-U ADDRESS INDICATION message as shown in the table below. The motivation was to trigger early data forwarding for CHO MR-DC or CPC scenarios.

#### 9.1.1.11	XN-U ADDRESS INDICATION

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| … |  |  |  |  |  |  |
| CHO MR-DC Indicator | O |  | ENUMERATED (true, ..., coordination-only) | Indicating that the XN-U ADDRESS INDICATION message is for Conditional Handover, as specified in TS 37.340 [8]. | YES | reject |
| CHO MR-DC Early Data Forwarding Indicator | O |  | ENUMERATED (stop, ...) |  | YES | ignore |
| CPC Data Forwarding indicator | O |  | ENUMERATED (triggered, early data transmission stop, ..., coordination-only) | Indicating that the XN-U ADDRESS INDICATION message is for a Conditional PSCell Change. | YES | reject |
| … |  |  |  |  |  |  |
| … |  |  |  |  |  |  |
| … |  |  |  |  |  |  |

In this release, early data forwarding is also supported for Inter-CU/gNB LTM as shown step 9a [4].

FIG. 1 Part of signalling procedure for inter-gNB LTM (Figure 9.2.3.5.X-1 in [4])

Similar to Rel-17/18 CHO MR-DC or CPC scenarios, there are two scenarios for Inter-CU LTM, which were agreed to support in last meeting.

The first scenario is about inter-MN change while the source secondary node is finally released as shown in FIG. 2.

FIG. 2 Part of signalling procedure for inter-CU LTM (MR-DC case).

The second scenario is about inter-SN change while the master node is not changed as shown in FIG. 3.

FIG. 3 Part of signalling procedure for inter-SN LTM (PSCell change case).

In order to support early data forwarding for the scenarios showed in FIG. 2 and FIG. 3, XN-U ADDRESS INDICATION message should be enhanced. One possible solution is to introduce two indicators, for example, one LTM MR-DC data forwarding indicator and one LTM PSCell change data forwarding indicator. The other solution is to change the stage 3 descriptions to cover the LTM MR-DC and LTM PSCell change scenarios.

Based on the analysis above, the following proposal is suggested to RAN3:

Proposal: To enhance XN-U ADDRESS INDICATION message to cover the LTM MR-DC and LTM PSCell change scenarios.

- Option 1: Introduce two indicators: LTM MR-DC data forwarding indicator and LTM PSCell change data forwarding indicator

- Option 2: to change the stage 3 descriptions, e.g., semantics and text

# Proposals

Proposal: To enhance XN-U ADDRESS INDICATION message to cover the LTM MR-DC and LTM PSCell change scenarios.

- Option 1: Introduce two indicators: LTM MR-DC data forwarding indicator and LTM PSCell change data forwarding indicator

- Option 2: to change the stage 3 descriptions, e.g., semantics and text

# Reference

[1] R3-250001, RAN3#127 Meeting Agenda

[2] R3-250002, RAN3#126 Meeting Report

[3] R3-247824, Summary of offline discussion on Mobility Enh

[4] R3-250060, (BL CR to 38.300) Support for Inter-CU LTM