3GPP TSG-RAN WG3 Meeting #127bis                                                               R3-251635

Wuhan, China, 7th – 11th, April 2025

Source:	CATT

Title:	On energy saving indicator from CN to RAN

Agenda Item:	8.3

Document for:	Discussion and Decision

1 Introduction

In the last RAN3 meeting[1], the LS from SA2 on energy saving indicator from CN to RAN was received. In the LS, RAN3 was requested to feedback the consideration on the indicator:

As part of the Rel-19 EnergySys work SA2 approved the attached 23.501CR5739. SA2 would like RAN2 and RAN3 to provide feedback on the Energy Saving Indicator that may be provided from AMF to NG-RAN.

This issue has been discussed during the last RAN3 meeting, however, no consensus was made. As a result, it is planned to be discussed in RAN3#127bis:

There is no consensus in RAN3 on the usage of this energy saving indication in this meeting, making the decision in next meeting and provide reply LS to SA2. To be discussed in AI8.3 in RAN3#127bis.

This contribution will provide our view on the energy saving indicator from CN to RAN.

2 Background

According to the incoming LS[2], the 23.501 CR[3] was approved by SA2 in Rel-19 EnergySys as below:

S2-2413019: ### 5.X.Y Subscription information aspects The Subscription data for a UE may include an Energy Saving indicator that the UE is subject to network energy saving operation. The AMF receives the Energy Saving indicator in the UE Subscription data. The AMF forwards the Energy Saving indicator to the NG-RAN and to the PCF. Editor’s Note: The indicator is expected to select an energy saving strategy for the UE in the system. Whether there are multiple values of the indicator is FFS. Editor’s Note: Whether the indicator is provided to the NG-RAN and how it is used in the NG-RAN is FFS. Editor’s Note: Whether the Energy Saving indicator is also stored in the UDR is FFS. ### 5.X.Z Policy control aspects During AM policy association establishment procedure, the AMF sends to the PCF the Energy Saving indicator as described in clause 5.x.y. Based on the Energy Saving indicator the PCF may make policy decisions reusing the existing parameters based on the operator policy. Editor’s Note: A notification for the energy related information, if any, should not be provided from the UDM/UDR. Whether the notification for the energy related information from the EIF is provided to the PCF and what are the notification details, are FFS.

It can be observed that following mechanism is approved by SA2:

- An Energy Saving indicator in subscription data for UE is sent to the AMF and the AMF will forward the indicator to PCF.

- This per-UE indicator is to assist the network energy saving strategy. The PCF may make the policy decisions for the UE considering this indicator.

And it is FFS whether the indicator is provided to the NG-RAN and how it is used by NG-RAN.

3 Discussion

In our understanding, two possible cases can be considered for the need of the per-UE energy saving indicator from CN to RAN:

- Case 1: For a particular treatment by RAN to the UE with the indicator

For example, the energy saving indicator would mean that serving to UE can be deprioritized compared to other UEs, if network energy saving is required.

- Case 2: For indicating capability (or willingness) of the UE on supporting RAN level NES technique.

Because RAN should know whether the UE supports the NES technique, otherwise RAN cannot decide to apply the NES technique to UE.

On the need of particular treatment to UE with this indicator

According to TS 23.503[4], if dynamic PCC is deployed, the PCF provides authorized QoS for a service data flow and other network control regarding SDF detection, gating, QoS and charging towards the SMF. The SMF is responsible for the enforcement of the policy related to SDF detection, authorized QoS, charging, gating, traffic usage reporting, packet routing and forwarding and traffic steering. The SMF controls the policy enforcement which includes the binding of SDFs to QoS Flows and the interaction with the CHF. And it is the SMF to interact with the UPF(s), the RAN and the UE to achieve the appropriate treatment of the traffic.

According to TS 23.501, the PCF uses the local preconfigured policy or UE subscription information to generate policy rules and install them on the SMF including the QoS parameters for handling packets. The SMF then generates and installs information (e.g., PDR/URR) on UPF to enable the QoS enforcement. The SMF also generates the information including the QoS parameters and provide them to NG-RAN in the PDU session management procedures. The NG-RAN node will allocate the radio link resources to all UEs’ traffic considering the QoS parameters based on RAN’s RRM algorithm.

With the energy-saving indicator delivered to PCF, we understand that the PCF can make the policy rules for UE taking the indicator into account. Namely, if a UE’s indicator is provided, the PCF would generate new policy rules for this UE compared to the policy rules made in case the indicator is not provided, which will result in the change of requirement for policy enforcement. If the new policy leads to a new QoS-related policy, the change of QoS enforcement is required. Per legacy procedures on QoS enforcement, SMF can send the QoS parameters to UPF and RAN, and the UPF and RAN can treat this UE accordingly.

5G QoS parameters transmitted to RAN include:

- Session-AMBR

- For each QoS flow, the QoS profile shall include the parameters:

-	5G QoS Identifier (5QI)

-	Allocation and Retention Priority (ARP)

-	For each GBR QoS Flow only, the QoS profile shall also include the QoS parameters:

-	Guaranteed Flow Bit Rate (GFBR) - UL and DL

-	Maximum Flow Bit Rate (MFBR) - UL and DL

TS 23.501 #### 5.7.1.5 QoS Flow mapping The SMF performs the binding of PCC rules to QoS Flows based on the QoS and service requirements (as defined in TS 23.503 [45]). The SMF assigns the QFI for a new QoS Flow and derives its QoS profile, corresponding UPF instructions and QoS Rule(s) from the PCC rule(s) bound to the QoS Flow and other information provided by the PCF. When applicable, the SMF provides the following information for the QoS Flow to the (R)AN: - QFI; - QoS profile as described in clause 5.7.1.2. - optionally, Alternative QoS Profile(s) as described in clause 5.7.1.2a; For each PCC rule bound to a QoS Flow, the SMF provides the following information to the UPF enabling classification, bandwidth enforcement and marking of User Plane traffic (the details are described in clause 5.8): - a DL PDR containing the DL part of the SDF template; - an UL PDR containing the UL part of the SDF template; NOTE 1: If a DL PDR for an bidirectional SDF is associated with a QoS Flow other than the one associated with the default QoS rule and the UE has not received any instruction to use this QoS Flow for the SDF in uplink direction (i.e. neither a corresponding QoS rule is sent to the UE nor the Reflective QoS Indication is set in the PCC rule), it means that the UL PDR for the same SDF has to be associated with the QoS Flow associated with the default QoS rule. - the PDR precedence value (see clause 5.7.1.9) for both PDRs is set to the precedence value of the PCC rule; - QoS related information (e.g. MBR for an SDF, GFBR and MFBR for a GBR QoS Flow) as described in clause 5.8.2; - the corresponding packet marking information (e.g. the QFI, the transport level packet marking value (e.g. the DSCP value of the outer IP header); - optionally, the Reflective QoS Indication is included in the QER associated with the DL PDR (as described in clause 5.7.5.3).

It can be observed that, given dynamic PCC is deployed, the PCF can produce new policy leading to the change of QoS parameters (probably a QoS degradation) for UEs based on the energy saving indicator, which makes the particular treatment to UEs with the indicator possible. Whether and when to consider the energy saving indicator in PCF for policy producing is up to the operator, e.g., the necessity of saving network energy would trigger the PCF to consider UE’s energy saving indicator. Furthermore, whether the QoS related policy is impacted by the indicator can also up to the operator, e.g., if the RAN level energy saving is required, PCF should consider this indicator when making the QoS related policy for UE. Therefore, a separate indicator sent from CN to RAN for Case 1 is not needed.

Observation 1: The indicator delivered to PCF can make the PCF produce QoS related policy considering this indicator, that particular treatment by RAN to the UEs with this indicator can be achieved. Separate indicator from CN to RAN for particular treatment to such UEs is not needed.

On need of indicating the UE supporting NES

We can divide the NES techniques into two categories, one category of NES techniques does not need cooperation of UE, namely, RAN can apply the NES techniques of this category without UE’s support, and another category requires cooperation of UE.

Category 1: Not need support of UE

- Cell deactivation and re-activation

- SSB level deactivation and re-activation

- Paging optimization in limited SSBs

Category 2: Need support of UE

Category 2 can further be divided into two groups. Category 2.1 applies to Idle/Inactive mode UEs, while category 2.2 applies to Connected mode UEs. Therefore, RAN can operate Category 2.1 without knowing the information on whether the UE supports the technique, while RAN has to be aware of that information before operating such technique for Category 2.2. Note that PRACH adaptation can be applied to both Idle/Inactive mode and connected mode, so it belongs to both Category 2.1 and Category 2.2.

Category 2.1: Apply to Idle/Inactive mode, RAN does not need to know whether UE supports the technique

- On-demand SIB1 (R19)

- Paging adaptation (R19)

- PRACH adaptation (R19)

Category 2.2: Apply to Connected mode, RAN has to know whether UE supports the technique

- Cell DTX/DRX

- CHO for NES

- SSB-less SCell

- Spatial and power domain adaptation

- On-demand SSB (R19)

- SSB adaptation (R19)

- PRACH adaptation (R19)

It can be observed that RAN needs to be aware of the information on UE’s support for Category 2.2. But other techniques do not require RAN to obtain the information of UE’s support, RAN can be preconfigured to operate one or several NES techniques by the operator.

For Category 2.2, RAN can obtain UE capability information via RRC message. In Rel-18, different indicators for indicating the UE’s capability of each Category 2.2 technique were captured in RRC spec. Also, the UE capabilities with respect to Rel-19 NES techniques of Category 2.2 will be captured in RRC spec in Rel-19. So, a separate indicator sent from CN to RAN to indicate the UE’s capability is not needed.

Let’s further analyze whether it’s necessary to let RAN know the UE’s willingness to utilize a NES technique for Category 2.2. The consideration is that RAN should be allowed by UE for operating the NES technique because RAN needs UE’s help. If that’s the case, RAN considers both UE’s capability and UE’s willingness according to subscription data to apply a NES technique. In our understanding, applying such NES techniques indeed need UE’s support and impose certain complexity to UE. Basically, UE has to receive or send the extra signalling, e.g., for configuring or (de-)activating those NES techniques. However, the principle of Rel-18/19 NES techniques is that the network energy saving should be achieved without costing UE’s power significantly, the amount of extra signalling is assumed limited, that handling such signalling will not consume much power or lead to a big burden to UE. What’s more, the service quality to UE’s traffic is not expected to be impacted due to applying those NES techniques, given the QoS is determined. Thus, we don’t think it is necessary for UE to allow RAN applying the NES techniques, it’s up to RAN whether to apply NES technique to a UE if only the UE has the capability.

Observation 2: A separate indicator from CN to RAN for indicating UE supporting NES is not necessary.

From Observation 1 and 2, we believe the indicator provision from CN to NG-RAN is not necessary. But RAN3 can further discuss this issue if SA2 has more progress on the necessity of the indicator. We propose RAN3 to make the conclusion and send a reply LS to SA2.

Proposal 1: RAN3 reply to SA2 that the indicator provision from CN to NG-RAN is not necessary from RAN3 perspective, but further discussion is not precluded if SA2 has more progress on this issue.

3 Conclusion

In the previous sections we made the following observations and proposals:

Observation 1: The indicator delivered to PCF can make the PCF produce QoS related policy considering this indicator, that particular treatment by RAN to the UEs with this indicator can be achieved. Separate indicator from CN to RAN for particular treatment to such UEs is not needed.

Observation 2: A separate indicator from CN to RAN for indicating UE supporting NES is not necessary.

Proposal 1: RAN3 reply to SA2 that the indicator provision from CN to NG-RAN is not necessary from RAN3 perspective, but further discussion is not precluded if SA2 has more progress on this issue.

4 Reference

- RAN3 Chair’s Notes, RAN3#127

- R3-250024(S2-2413034), LS on energy saving indication from CN to RAN, SA2, RAN3#127

- S2-2413019, KI#2: UE subscription and policy control for energy efficiency and energy saving, Lenovo, vivo, ZTE, Samsung, NEC, Huawei, China Mobile, MediaTek Inc., China Unicom, TOYOTA, China Telecom, CATT, Deutsche Telekom, LG Electronics, SA2#166

- 3GPP TS 23.503, Policy and charging control framework for the 5G System, stage-2, v19.2.0 (2024-12)