3GPP TSG-RAN WG3 #125bis	R3-251645

Wuhan, China, 7th – 11th April 2025

Title:	[Draft] LS on LP-WUS subgrouping

Response to:

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	CATT [To be RAN3]

To:	SA3

Cc:	RAN2, SA2

Contact Person:

#### Name:	LU Yang

E-mail Address:	luyang (at) catt (dot) cn

1. Overall Description:

According to the current TS 38.304 where LP-WUS is not supported yet, the UE_ID based subgrouping for PEI is determined by the 15 bits (truncated) 5G-S-TMSI. So, up to 15 LSBs of 5G-S-TMSI is delivered over F1 and Xn interfaces now for deriving the PEI subgroup ID.

However, for UE_ID based subgrouping for LP-WUS, additional 5 LSB bits (i.e., 20 bits in total) of 5G-S-TMSI need to be delivered over F1 and Xn interfaces for RAN node to derive the LP-WUS subgroup ID.

RAN3, however, is not sure whether there is any potential issue if the truncated 5G-S-TMSI over F1/Xn is extended to 20 bits.

## 2. Actions:

To SA2 and SA3:

ACTION: RAN3 respectfully asks SA3 to take above information into account, and feedback if there is any concern.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #128	19th – 23rd May 2025	Malta, MT

TSG-RAN3 Meeting #129	25th – 29th Aug 2025	India (TBC), IN