3GPP TSG-RAN WG3 Meeting #127bis	R3-251648

Wuhan, China, 7 – 11 Avril, 2025

Agenda Item:	11.3

Source:	Nokia, FiberCop, Jio Platforms (JPL)

Title:	(TP for BLCR to TS 38.473) Further discussion on energy efficient AI/ML-based CCO issue prediction in split architecture

Document for:	Text Proposal

# 1	Introduction

At RAN3#126 the following Working Assumption (WA) was turned into agreement:

Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1.

This agreement confirms the use of the same signalling procedure, i.e. the gNB-CU Configuration Update procedure, to convey detected (legacy) CCO issue and predicted CCO issue from the gNB-CU to the gNB-DU. In this paper we further discuss the need to provide additional coordination between the gNB-DU and the gNB-CU in order to enable energy efficient AI/ML-based CCO issue prediction in split architecture, and propose a way to do this taking into account the agreements for F1AP that were made at last RAN3 meeting.

# 2	Discussion

3GPP RAN WGs have worked on mechanisms to reduce the energy consumption in the RAN and in UEs since Rel-10 at least. The effort has essentially focused on energy consumption linked to radio transmission and reception, which corresponds to the major part of RAN energy consumption. The radio will continue to be the main energy consumer also after introduction of AI/ML in the NG-RAN, however we believe that energy efficiency also in other entities than the gNB-DU should be taken into account whenever the standard can make a difference.

Standards support of AI/ML-based NG-RAN use cases are introduced in Rel-18 and Rel-19 in order to enable AI/ML model operation in NG-RAN nodes. This will represent clear enhancements for different aspects of the network operation, including in terms of energy saving. Still, AI/ML-based CCO requires, in order to be operational, an AI/ML model generating predicted CCO issue which is trained and tested for the cell (including cell neighbourhood) to which the CCO issue prediction applies. The inference phase may require real-time measurements from own cell and will possibly require prediction information from other AI/ML models, when such can be provided. TR 38.743 indicates input information to include Measured/Predicted radio resource status from local node and neighbour node. However, it is reasonable to assume that the CCO inference phase additionally requires e.g. predicted UE trajectory information for served UEs.

Of course, the 3GPP standard is AI/ML model agnostic and main responsibility for energy efficient AI/ML algorithms lies on implementions that will make choices relative to Life Cycle Management (e.g. training in OAM vs. training in the gNB) and e.g. dimensioning of the AI/ML algorithms so that energy consumption for AI/ML model training and inference is kept at the lowest possible level.

However, also as part of 3GPP’s work on signalling enhancements to support CCO issue predictions it is important to take sustainability aspects into account. The standardized legacy CCO signalling framework for split architecture doesn’t provide any mechanism to select the cells for which CCO issue detection is done, and this could be justified by the fact that the gNB-CU will anyway monitor the overall cell performance, cases of failure (e.g. radio link failure due to coverage holes) and cases of reduced UE performance (e.g. cell edge capacity) for the purpose of performance monitoring and KPI reporting to the OAM. Therefore, this is done not necessarily for the purpose of CCO alone, but taking into account the increased importance of sustainable solutions in all domains we don’t exclude that a solution for energy efficient CCO issue prediction could also apply to legacy CCO issue detection.

And concerning AI/ML-based predictions, it is in any case not energy efficient to predict CCO issues for which DU does not have alternative coverage configurations.

Observation 1: It is not energy efficient to predict CCO issues for which DU does not have alternative coverage configurations.

The gNB-DU should therefore be able to indicate to the gNB-CU the cells for which the gNB-DU would be able to change the coverage state in case a CCO issue is predicted. This could depend on antenna-related capabilities (e.g. whether an Active Antenna System is used for the cell), which would be relatively static information, but could also depend on available coverage configurations from the OAM or on the immediate capability or capacity of the gNB-DU to calculate and enforce a new coverage state for given cells. Making such cell information available in the gNB-CU would help to avoid wasted computing efforts and corresponding wasted energy consumption for CCO prediction for cells for which no CCO action can be taken. RAN3 could as well consider applying this information also for legacy CCO detection.

Proposal 1: The gNB-CU receives information about the cells for which the gNB-DU can change the coverage state, so that it can limit CCO issue prediction to those cells.

How to encode this information on the F1 interface should be left to RAN3’s discussion, but a possible solution could be to use an enumerated value to express that CCO issue prediction is requested or not requested. This would avoid sending capability-related information over F1.

Proposal 2: RAN3 to define detailed encoding about the cells for which the gNB-DU can change the coverage state, which could be e.g. an enumerated value to express that CCO issue prediction is requested or not requested.

A further step in terms of optimization would be that the gNB-DU informs the gNB-CU about the available coverage configurations. E.g. an extensive choice of alternative coverage configurations may be available for some cell, and a more limited choice of alternative coverage configurations may be available for other cells. The gNB-DU may also have configured information about whether these available coverage configurations are suitable for solving coverage issues or cell edge capacity issues. By receiving such information, the gNB-CU may optimize data collection, model training and model inference in order to ensure that energy and computing capacity is consumed in accordance with the corrective actions available in the gNB-DU.

Proposal 3: The gNB-DU informs the gNB-CU about available coverage configurations for its served cells.

RAN3#126 agreed to provide the predicted CCO issue information within a new F1AP Predicted CCO Assistance Information IE sent in the gNB-CU Configuration Update procedure. It therefore seems suitable to provide the information outlined in Proposal 2 and 3 in the gNB-DU Configuration Update procedure, e.g. as cell level information.  This is illustrated in Figure 1.

Figure 1: Example of the gNB-DU informing the gNB-CU about its possibilities in terms of coverage change modification for its served cells.

Based on this, our proposal is that the gNB-DU provides the discussed assistance information for CCO in the F1AP Served Cell Information IE which is included in the F1 SETUP REQUEST and GNB-DU CONFIGURATION UPDATE messages.

Proposal 4: The CCO assistance information discussed above is included in the F1AP Served Cell Information IE.

# 3	Conclusion

We have made the following observation and proposals:

Observation 1: It is not energy efficient to predict CCO issues for which DU does not have alternative coverage configurations.

Proposal 1: The gNB-CU receives information about the cells for which the gNB-DU can change the coverage state, so that it can limit CCO issue prediction to those cells.

Proposal 2: RAN3 to define detailed encoding about the cells for which the gNB-DU can change the coverage state, which could be e.g. an enumerated value to express that CCO issue prediction is requested or not requested.

Proposal 3: The gNB-DU informs the gNB-CU about available coverage configurations for its served cells.

Proposal 4: The CCO assistance information discussed above is included in the F1AP Served Cell Information IE.

A corresponding F1AP TP is included in annex of this paper.

# Annex	- TP for BLCR to TS 38.473

The TP is based on R3-247929.

<<< start of changes >>>

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

The gNB-DU initiates the procedure by sending a F1 SETUP REQUEST message including the appropriate data to the gNB-CU. The gNB-CU responds with a F1 SETUP RESPONSE message including the appropriate data.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the F1 interface is operational and other F1 messages may be exchanged.

If the F1 SETUP REQUEST message contains the gNB-DU Name IE, the gNB-CU may use this IE as a human readable name of the gNB-DU. If the F1 SETUP REQUEST message contains the Extended gNB-DU Name IE, the gNB-CU may use this IE as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if included.

<<<<SKIP UNRELATED PART>>>>

If the XR Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the CCO Assistance Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization.

#### 8.2.3.3	Unsuccessful Operation

Figure 8.2.3.3-1: F1 Setup procedure: Unsuccessful Operation

If the gNB-CU cannot accept the setup, it should respond with a F1 SETUP FAILURE and appropriate cause value.

If the F1 SETUP FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the F1 setup towards the same gNB-CU.

#### 8.2.3.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

<<<<SKIP UNRELATED PART>>>>

If the Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization and network energy saving. If the Coverage Modification Cause IE is set to the "network energy saving", gNB-CU may consider those deactivated SSB beams are due to network energy saving.

If the Cells for SON IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update this information and behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the RedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the eRedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU. If the Extended gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if also included.

If the RRC Terminating IAB-Donor Related Info IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider that the BAP address indicated by the Mobile IAB-MT BAP Address IE is assigned by the gNB-CU of the RRC-terminating IAB-donor indicated  by the RRC Terminating IAB-Donor gNB-ID IE, and it shall use this  BAP address and gNB ID for the subsequent IAB Transport Migration Management procedure towards the RRC-terminating IAB-donor of the mobile IAB-node as needed, as specified in TS 38.423 [28].

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization.

If the CCO Assistance Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization.

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: gNB-DU Configuration Update procedure: Unsuccessful Operation

If the gNB-CU cannot accept the update, it shall respond with a GNB-DU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-DU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the GNB-DU CONFIGURATION UPDATE message towards the same gNB-CU.

#### 8.2.4.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| <<<<SKIP UNRELATED PART>>>> |  |  |  |  |  |  |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [8]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | Ignore |
| CCO Assistance Information | O |  | 9.3.1.C |  | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.C	CCO Assistance Information

This IE includes CCO assistance information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CCO Issue Prediction Request Information | M |  | ENUMERATED (requested, not requested, …) |  |
| Available Cell Coverage Configurations List |  | 0..1 |  |  |
| >Available Cell Coverage Configurations Item |  | 1.. <maxnooAvailCellCoverageConfigurations> |  |  |
| >>Cell Coverage State | M |  | INTEGER (0..63, …) |  |
| >>>Available SSB Coverage Configurations List |  | 0..1 |  |  |
| >>>>Available SSB Coverage Configurations Item |  | 1..< maxnooAvailSSBCoverageConfigurations> |  |  |
| >>>>>SSB Coverage State | M |  | INTEGER (0..15, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnooAvailCellCoverageConfigurations | Maximum no. of available cell coverage configurations. Value is 64. |
| maxnooAvailSSBCoverageConfigurations | Maximum no. of available SSB coverage configurations. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

<<< start of changes >>>

ASN.1 to be added

<<< end of changes >>>