3GPP TSG-RAN WG3 Meeting #127-bis	R3-251679

Wuhan, China, 7th – 11st April 2025

Agenda Item:		11.4

Source:		NEC

Title:			Measurement Continuity for continuous MDT collection

Document for:		Discussion and decision

# Introduction

In last RAN3 meeting, one LS to SA5 [1] was agreed to ask SA5 to provide guidance on possible solutions for Continuous MDT.

In this contribution, we further discusses the open issues related to measurement continuity problem that do not depend on SA5’s reply.

# Discussion

Regarding continuous MDT collection, two problems for continuous MDT collection, as below, were identified in SI and captured in the TR38.743 [1].

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

For Problem B trace correlation, RAN3 prefers to use a temporary identifier, that can be available at the TCE and is not revealing the UE identity, to enable correlation of the management-based MDT reports.

## 2.1	Measurement continuity across RRC states

We already agreed that “Continuous collection of MDT traces is beneficial only for AI/ML training in OAM”. Training data is the initial dataset used to train machine learning algorithms on how to make predictions. In order to increase the precision of a ML model, the process of training an ML model needs large volume of input data and a rather long training time. The provision of training data is not time critical.

As in legacy, immediate MDT can be configured to UE for measurements in RRC Connected, while logged MDT in – RRC Idle and RRC Inactive. Measurement continuity in RRC Connected or RRC Idle/RRC Inactive can be achieved through existing MDT mechanism.

Observation 1: Measurement continuity during the same RRC state can be guaranteed in legacy way.

Regarding to the gap during RRC state transition, it is a extremely short period of time comparing to the long training time and the measurements during the transition can be negligible.

Besides, the collected raw data may still need to be processed and translated into usable information for training by data processing technology, which can also handle the sparse missing data issue, due to e.g. RRC state transition or other reasons.

In addition, three type of information, i.e. UE location information, UE measurement report and UE Mobility History Information, can be collected from UE for AI/ML. In order to limit the impact on UE power consumption and processing, continuous collection and logging of UE measurements should as much as possible rely on the measurements that are already supported in UE and so we only consider to continuous collect the above data. In this case, only UE location information may need to be considered for measurement continuity across RRC states, but this information is most likely keep unchanged in a extremely short time.

Observation 2: Measurement discontinuity during RRC state transition is extreme short and can be negligible.

Based on the above analysis, measurement continuity during the same RRC state can be guaranteed in legacy way and measurement discontinuity during RRC state transition can be negligible, therefore we do not think this issue needs a dedicated solution.

Proposal 1: Measurement continuity issue is negligible for continuous MDT collection and a dedicated solution is not needed.

## 2.2	Measurement Continuity during RRC_Connected mobility

As mentioned in the LS[1], RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility.

As we discussed during offline in last meeting, this issue can be addressed by sending a continuous MDT collection indication from source gNB to target gNB during handover preparation procedure, e.g. in HANDOVER REQUEST message. This indication informs the target gNB that this UE still needs to be selected for continuous MDT collection.

Proposal 2: A continuous MDT collection indication can be added in HANDOVER REQUEST message to support measurement continuity during handover.

# Proposals

In this contribution, we provide our further analysis on continuous MDT collection for AI/ML and the following observation and proposals were drawn from the analysis.

Observation 1: Measurement continuity during the same RRC state can be guaranteed in legacy way.

Observation 2: Measurement discontinuity during RRC state transition is extreme short and can be negligible.

Proposal 1: Measurement continuity issue is negligible for continuous MDT collection and a dedicated solution is not needed.

Proposal 2: A continuous MDT collection indication can be added in HANDOVER REQUEST message to support measurement continuity during handover.

# Reference

[1] R3-250891, LS on Continuous MDT