3GPP TSG-RAN WG3 Meeting #127-bis	R3-251681

Wuhan, China, 7th – 11st April 2025

Agenda Item:		11.4

Source:		NEC

Title:	(TP to BLCR TS38.473) Split architecture support

Document for:		Discussion and decision

# Introduction

In previous RAN3 meetings, the following agreements were achieved:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

In split architecture, the Energy Cost of gNB is the sum of the Energy Cost of its gNB-DUs.

Mapping rule is only configured at gNB-DU and shall be unified for all the gNB-DUs within a defined area.

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

In this contribution, we provide our considerations on the stage 3 details of the solution.

# Discussion

As agreed, Energy Cost EC is measured in gNB-DU and reported to gNB-CU, and then gNB-CU calculates the sum of the EC of its gNB-DUs to generate the EC of gNB.

Since we also agreed to introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU, the existing related XnAP procedures can be used as baseline for F1AP messages design.

As measured EC and measured UE performance were agreed to be transferred from gNB-DU to gNB-CU(-CP), below we analyse the existing Data Collection and Reporting messages one by one.

## 2.1	Measured EC

In the DATA COLLECTION REQUEST message, the following IE needs to be included:

  - Report Characteristics for Data Collection: only one bit for measured EC

  - Reporting Periodicity for Data Collection

  - Requested Prediction Time

Since EC is a node level measurement, DATA COLLECTION RESPONSE message needs to be updated accordingly to include only Node Measurement Initiation Result List.

In the DATA COLLECTION UPDATE message, only Node Associated Info Result for EC is included.

TP to TS38.473, mirroring the analysis above, are available in Annex.

Proposal 1: To agree the TP to BLCR TS38.473 on measured EC transfer from gNB-DU to gNB-CU in Annex.

## 2.2	Measured UE performance

According to TS28.558, for split gNB, the UE performance are measured at different nodes as below:

  - Average delay UL/DL: gNB-DU and gNB-CU-UP

  - UL Packet loss: gNB-CU-UP

  - DL Packet loss: gNB-DU

  - Average UL/DL UE throughput: gNB-DU

  - DL/UL data volume: gNB-CU

Observation: DL/UL data volume is already available at gNB-CU.

Considering AI/ML model is located in the gNB-CU(-CP) in case of split architecture, gNB-DU and gNB-CU-UP need to send their measured UE performance to gNB-CU-CP.

In last meeting, we already agreed that “the delay in DU side is aggregated in the CU-UP in the case of CP-UP split architecture”, therefore average delay UL/DL measured in gNB-DU needs to be sent to gNB-CU-UP vis F1-U interface, and then gNB-CU-UP sends the aggregated delay to gNB-CU-CP over E1 interface.

For the sake of simplicity, we suggest to use gNB-CU-UP based solution also for DL Packet loss measured in gNB-DU.

In summary, gNB-DU sends the following measured UE performance to gNB-CU-UP over F1-U interface:

  - Average delay UL/DL

  - DL Packet loss

  - Average UL/DL UE throughput

And after aggregation in gNB-CU-UP, gNB-CU-UP sends all UE performance, including Average delay UL/DL, Packet loss UL/DL and Average UE throughput UL/DL, to gNB-CU-CP over E1 interface.

Proposal 2: gNB-DU sends the following measured UE performance to gNB-CU-UP over F1 interface:

  - Average delay UL/DL

  - DL Packet loss

  - Average UL/DL UE throughput

Proposal 3: gNB-CU-UP sends the following measured UE performance to gNB-CU-CP over F1 interface:

  - Average delay UL/DL

  - UL/DL Packet loss

  - Average UL/DL UE throughput

# Proposals

In this contribution, we provide our general consideration on split architecture support and the following proposals and observation are made.

Proposal 1: To agree the TP to BLCR TS38.473 on measured EC transfer from gNB-DU to gNB-CU in Annex.

Observation: DL/UL data volume is already available at gNB-CU.

Proposal 2: gNB-DU sends the following measured UE performance to gNB-CU-UP over F1 interface:

  - Average delay UL/DL

  - DL Packet loss

  - Average UL/DL UE throughput

Proposal 3: gNB-CU-UP sends the following measured UE performance to gNB-CU-CP over F1 interface:

  - Average delay UL/DL

  - UL/DL Packet loss

  - Average UL/DL UE throughput

# Annex TP to BLCR TS38.473

<Start of 1st change>

### 8.x	Data Collection and Reporting procedures

### 8.x.1	Data Collection Reporting Initiation

#### 8.x.1.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.x.1.2	Successful Operation

Figure 8.x.1.2-1: Data Collection Reporting Initiation, successful operation

gNB-CU initiates the procedure by sending the DATA COLLECTION REQUEST message to gNB-DU to start information reporting or to stop information reporting. Upon receipt, gNB-DU:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If gNB-DU is capable of providing all of the requested information, it shall initiate the information reporting as requested by gNB-CU and respond with the DATA COLLECTION RESPONSE message.

If gNB-DU is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The gNB-DU shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The gNB-DU shall take it into account when generating the requested predicted information.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the gNB-DU shall take it into account for the configuration of UE performance collection and reporting.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects gNB-DU performs measurements or predictions on. gNB-DU shall include in the DATA COLLECTION UPDATE message:

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-DU.

#### 8.x.1.3	Unsuccessful Operation

Figure 8.x.1.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, gNB-DU shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.x.1.4	Abnormal Conditions

For the same Measurement ID, if the initiating gNB-CU does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the gNB-CU may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-DU, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the gNB-DU Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the gNB-DU shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the gNB-CU Measurement ID IE corresponding to an existing on-going Data Collection reporting, then gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

### 8.x.2	Data Collection Reporting

#### 8.x.2.1	General

This procedure is initiated by an gNB-DU to report information accepted by the gNB-DU following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.x.2.2	Successful Operation

Figure 8.x.2.2-1: Data Collection Reporting, successful operation

gNB-DU shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

#### 8.x.2.3	Unsuccessful Operation

Not applicable.

#### 8.x.2.4	Abnormal Conditions

Void.

<Start of 2nd change>

#### 9.2.y	Data Collection and Reporting Messages

#### 9.2.y.1	DATA COLLECTION REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested information reporting according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-DU is requested to report. First Bit = Energy Cost Other bits are ignored by the gNB-DU. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Performance Collection Configuration | O |  | FFS |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

#### 9.2.y.2	DATA COLLECTION RESPONSE

This message is sent by gNB-DU to gNB-CU to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the gNB-DU. First Bit = Energy Cost, Other bits are ignored by the gNB-CU. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

#### 9.2.y.3	DATA COLLECTION FAILURE

This message is sent by the gNB-DU to gNB-CU to indicate that for all of the requested objects the reporting cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.2.y.4	DATA COLLECTION UPDATE

This message is sent by gNB-DU to gNB-CU to report the requested information.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | reject |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | gNB-CU UE AP ID 9.3.1.4 |  | – |  |
| >>UE Performance | O |  | FFS |  | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

<End of change>