3GPP TSG-RAN WG3 #127bis	R3-251690

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	22.2

Source:	NEC

Title:	(TP to BLCR for TS38.423 on multi-hop relay) Discussion on multi-hop relay

Document for:	Discussion, Decision

Introduction

During RAN2 meeting, L2 multi-hop U2N relay has been discussed for several meetings and several agreements have been made. So, in this contribution, we will discuss some RAN3 related issue based on current RAN2 progress.

Discussion

In Rel-18, when U2N remote UE is performing handover from source gNB towards target gNB (for both inter-gNB direct to indirect path switch and indirect to indirect path switch), source gNB will provide candidate relay UE list associated to the corresponding U2N remote UE towards target gNB, target gNB shall be based on this information to perform final U2N relay UE selection for the U2N remote UE, where it is shown in below:

1. The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) and Uu measurements to the source gNB, after it measures/discovers the candidate L2 U2N Relay UE(s): - The L2 U2N Remote UE filters the appropriate L2 U2N Relay UE(s) according to relay selection criteria before reporting. The L2 U2N Remote UE shall report only the L2 U2N Relay UE candidate(s) that fulfil the higher layer criteria; - The reporting includes at least a L2 U2N Relay UE ID, a L2 U2N Relay UE's serving cell ID, and a sidelink measurement quantity information. SD-RSRP is used as sidelink measurement quantity. 2. The source gNB decides to trigger path switch for the L2 U2N Remote UE onto indirect path of the target gNB, based on MeasurementReport and RRM information. 3. The source gNB sends a HANDOVER REQUEST message to the target gNB to prepare the path switch at the target side. The HANDOVER REQUEST message includes Remote UE L2 ID and a list of candidate target Relay UE IDs belonging to one cell of the target gNB. 4. Admission Control may be performed by the target gNB. 5. The target gNB selects one target Relay UE from the list of candidate Relay UEs provided by the source gNB, and sends the RRCReconfiguration message to L2 U2N Relay UE for relaying configuration, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu Relay RLC channel and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration. 6. The target gNB sends the HANDOVER REQUEST ACKNOWLEDGE message to the source gNB, which contains RRC configuration for L2 U2N Remote UE at the target side. 7. The source gNB sends the RRCReconfiguration message to the L2 U2N Remote UE, which includes at least the L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated Uu end-to-end radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the direct path after reception of the RRCReconfiguration message from the source gNB. 8. The source gNB sends the SN STATUS TRANSFER message to the target gNB to convey the uplink PDCP SN receiver status and the downlink PDCP SN transmitter status of the L2 U2N Remote UE's DRBs for which PDCP status preservation applies (i.e. for RLC AM). 9. The L2 U2N Remote UE establishes PC5 connection with L2 U2N Relay UE. 10. The L2 U2N Remote UE sends the RRCReconfigurationComplete message to target gNB via the L2 U2N Relay UE. 11. The data path is switched from direct path to indirect path between the L2 U2N Remote UE and the target gNB via the target L2 U2N Relay UE. 12. The target gNB sends the UE CONTEXT RELEASE message to inform the source gNB about the success of the path switch.

In Rel-19, for inter-gNB direct to indirect path switch or indirect to indirect path switch, since U2N intermediate relay UE is introduced between U2N remote UE and U2N last relay UE, one U2N remote UE can be switched to either a U2N intermediate relay UE if it is a multi-hop U2N relay link, or directly to a U2N last relay UE if it is a single hop U2N relay link. Therefore, it is possible that source gNB shall provide a candidate list for either U2N intermediate relay UE or U2N last relay UE, to the target gNB within HANDOVER REQUEST message.

Source gNB shall provide a candidate list for either U2N intermediate relay UE or U2N last relay UE for the path switch of one U2N remote UE.

Furthermore, if source gNB provide a candidate list for U2N intermediate relay UE, target gNB should also have the knowledge on which last relay UE that each intermediate relay UE is connected to (for 3 hops multi-hop relay), therefore, source gNB should also provide the information of the connected U2N last relay UE for each candidate U2N intermediate relay UE towards target gNB within HANDOVER REQUEST message.

If source gNB provide a candidate list for U2N intermediate relay UE, it should also provide the information of the connected U2N last relay UE for each candidate U2N intermediate relay UE.

It is suggested to adopt the corresponding TP1 in the reference section.

Conclusion

In this contribution, we have primarily discussed further enhancement at RAN3 for U2N multi-hop relay. A brunch of proposals has been made as following:

Proposal 1	Source gNB shall provide a candidate list for either U2N intermediate relay UE or U2N last relay UE for the path switch of one U2N remote UE.

Proposal 2	If source gNB provide a candidate list for U2N intermediate relay UE, it should also provide the information of the connected U2N last relay UE for each candidate U2N intermediate relay UE.

Proposal 3	It is suggested to adopt the corresponding TP1 in the reference section.

Reference

TP1 TS38.423

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

- 9.2.3.188	Candidate Relay UE Info List

This IE contains the identity of the candidate relay UE(s) when the source NG-RAN decides to switch the UE to an indirect path at the target NG-RAN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Candidate Relay UE Info Item |  | 1..<maxnoofCandidateRelayUEs> |  |  |
| >Candidate Relay UE ID | M |  | BIT STRING(SIZE(24)) | Includes the SL-SourceIdentity IE for the candidate relay UE as defined in TS 38.331 [10]. |
| >Candidate U2N intermediate Relay UE ID | M |  | BIT STRING(SIZE(24)) | Includes the SL-SourceIdentity IE for the candidate U2N intermediate relay UE as defined in TS 38.331[10] |
| >Associated U2N Relay UE ID | M |  | BIT STRING(SIZE(24)) | Includes the SL-SourceIdentity IE for the associated U2N Last relay UE ID to each of the candidate U2N intermediate relay UE as defined in TS 38.331[10] |

| Range bound | Explanation |
| --- | --- |
| maxnoofCandidateRelayUEs | Maximum number of candidate relay UE(s). The value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

- 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

Skip unchanged part

#### -- C

CAG-Identifier	::= BIT STRING (SIZE (32))

CandidateRelayUEInfoList ::= SEQUENCE (SIZE(1..maxnoofCandidateRelayUEs)) OF CandidateRelayUEInfoItem

CandidateRelayUEInfoItem ::= SEQUENCE {

candidateRelayUEID		BIT STRING(SIZE(24)),

candidateIntermediateRelayUEID	BIT STRING(SIZE(24)),

associatedRelayUEID		BIT STRING(SIZE(24)),

iE-Extensions			ProtocolExtensionContainer { { CandidateRelayUEInfoItem-ExtIEs } }		OPTIONAL,

...

}

CandidateRelayUEInfoItem-ExtIEs	XNAP-PROTOCOL-EXTENSION ::= {

...

}