3GPP TSG-RAN WG3 Meeting #127b	 R3-251693

Wuhan, China, April 7-11, 2025

Agenda Item:	18.2

Source: 	NEC

Title:  	Discussion on LP-WUS UE-ID

Document for:	Discussion and Decision

- 1 Introduction

In last RAN3#127 meeting, the following agreements are made:

RAN3 will follow RAN2 conclusion on the terminology of “LP-WUS” and wait for the LS from RAN2 to SA2.

For the following issues, including Extended UE Identity Index、The maximum value of CN subgroup ID、Emergency call back、the terminology of “LP-WUS”、the LP-WUS node capability need more inputs from other WGs.

In last RAN2#128 meeting, the following agreement related to LP-WUS subgroup is made:

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

In this contribution, we further discuss how to design UE identify index value.

- 2 Discussion

- 2.1 UE Identity Index Value

The IE – (extended) UE Identity Index Value is used by the target NG-RAN node to calculate the Paging Frame and Paging Occasion for either normal DRX or eDRX, and calculate UE_ID based PEI subgroup ID. And it represents UE_ID in TS 38.304.

Before discussion, let’s take a look at the paging formula:

The PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

UE_ID:

If the UE operates in eDRX as specified in clause 7.4:

- 5G-S-TMSI mod 4096

else:

- 5G-S-TMSI mod 1024

It can be seen that for normal DRX scenarios, the UE_ID could be [0 ~ 1023], which means 10 bits are needed to represent UE_ID. For CN paging, the gNB can acquire 5G-S-TMSI from NGAP PAGING message and directly calculate UE_ID, while for RAN paging, the anchor gNB is not aware of UE’s 5G-S-TMSI (I-RNTI is used instead), in order to calculate paging occasion, the anchor gNB need to acquire UE_ID from Core Network Assistance Information for RRC INACTIVE sent by CN and this message includes 10 bit UE Identity Index Value. When it comes to RAN paging exchange between anchor gNB and other gNB, i.e., XnAP RAN PAGING message, the anchor gNB also need to inform other gNBs of UE_ID (i.e., UE Identity Index Value). Therefore this is the usage of UE Identity Index Value. It is also noted that the current UE_ID could be [0 ~ 8191] for PEI UE-ID based subgroup and could be [0 ~ 32767] for eDRX case, which means more bits are needed to represent UE_ID, that is reason why Extended UE Identity Index Value (16 bits) is introduced.

Back to LP-WUS discussion, RAN2 now has already designed UE-ID based LP-WUS subgroup formula which is shown below:

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where

-	UE_ID is related to 5G-S-TMSI,

-	N is the number of total paging frames in one DRX cycle,

-	Ns is the number of the PO for a PF,

-	Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

-	subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Even though RAN2 has not made a final conclusion about UE-ID calculation, but it almost should be 5G-S-MTSI mode X, and FFS value X. One purpose of using UE-ID is to make sure that all UEs with different UE-ID can be evenly separated into different PF/PO/subgroup. To fully calculate UE-ID, the value of UE-ID should not be less than [N*Ns*Np* subgroupsNumForUEID], currently the max N is 256 without eDRX, Ns is 4 and Np is 8. As for subgroupsNumForUEID for LP-WUS, even though the RAN1 modified it to 31, we still consider it as 32 due to the fact that one common group still exists. Therefore the max UE-ID without eDRX case should be [256*4*8*32] - 1, i.e., 262144 - 1, and X should be 262144 (18 bits). Furthermore, if eDRX is supported, max N is extended to 1024 due to the extended paging cycle T, therefore X should be 1048576 (20 bits).

No matter whether eDRX will be supported or not with LP-WUS simultaneously, at least 18 bits are needed for calculating subgroup ID. The current extended UE identity index value (i.e., 16 bits) is not enough for calculating LP-WUS subgroup ID. So that RAN3 can firstly confirm that new type of extended UE identity index value should be introduced.

However, the current situation is that RAN2 still did not make conclusion on the UE-ID calculation, for example, what is the value of X, and whether eDRX is supported or not, we need more information from RAN2.

Proposal-1: new type of extended UE identity index value should be introduced. FFS how many bits are needed pending on RAN2 discussion (e.g., what is value X, whether to support eDRX).

- 3 Conclusions

Proposal-1: new type of extended UE identity index value should be introduced. FFS how many bits are needed pending on RAN2 discussion (e.g., what is value X, whether to support eDRX).

- 4 References