3GPP TSG-RAN WG3 #127bis	R3-251695

Wuhan, China, April 7th – 11th, 2025

Source:				NEC

Agenda item: 	10.2. MRO Enhancements

Title: 	(TP to BLCR of 38.473) Discussion on MRO for LTM

Document for:				Discussion and decision

- Introduction

During previous RAN3 meetings, MRO enhancements for LTM were discussed, and the following agreements were made:

RAN3#123 bis: Work on scenarios of near failure LTM. Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell. RAN3#124: MRO for LTM: RAN3 to prioritize MCG LTM over SCG LTM. RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study. For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization. For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization. RACH information in LTM: RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM. RAN3#125: MRO for LTM: Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover. FFS to enhance failure scenario. Take the existing detection mechanism descriptions as baseline. For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure. CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure. FFS for additional information needed. RAN3#125 bis: For TP to BL CR to TS 38.300: - Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions. - For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM. - Change“successful handover" definition, as proposed in R3-245230. For TP to BL CR to TS 38.401: - Capture wrong LTM prepared cell selection. Add failure type to F1AP ACCESS AND MOBILITY INDICATION message. CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell. RAN3#126: For TP to BL CR to TS 38.473: - In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI. For TP to BL CR to TS 38.401: - For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM. - For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell. RAN3#127 CU can adjust the candidate Cell list for LTM based on UHI. DU should be able to resolve LTM ping-pong based on information over F1. Network based solution is used for LTM candidate cell list. Network based or UE based solution for TA acquisition type?

In this contribution, we would like to provide our further consideration on MRO for LTM.

- Discussion

2.1 MRO enhancement in case of RLF-report absent

There may be cases that the RLF-report is not available at the CU. For example, it is not availabe because the UE does not have the RLF-report, i.e., before being reported to the network, the content of the RLF-report associated with the LTM failure is replaced by a following connection failure. For another example, the RLF-report cannot be obtained by the gNB-CU due to network congestion, and finally discarded by the UE after 48 hours. For the case that RLF-report is not available for the gNB-CU, the gNB-CU can still identify the happening of LTM failure event:

- Too late LTM:  after the configuring of LTM configuration to the UE, if the gNB-CU finds that LTM cell switch is not triggered by the gNB-DU, and the UE re-establishes/re-connectes to another cell, it can identify the happening of too late LTM cell switch.

- Too early LTM: if the gNB-CU finds that a LTM cell switch is triggered by the gNB-DU, the UE does not access to the target LTM cell or soon after the UE accesses to the target LTM cell, and the UE re-establishes/re-connects to the source cell of the LTM cell switch procedure, the gNB-DU can identify the happening of too early LTM cell switch.

- LTM to wrong cell:  if the gNB-CU finds that a LTM cell switch is triggered by the gNB-DU, the UE does not access to the target LTM cell or soon after the UE accesses to the target LTM cell, and the UE re-establishes/re-connects to a cell which is neither the source cell nor the target cell of the LTM cell switch procedure, the gNB-DU can identify the happening of LTM cell switch to wrong cell.

Similar to Xn based procedures which have already been supported for Layer 3 based mobility, for example by Handover Report message and Failure Indication message, the the source gNB can be provided with handover failure information with RLF-report optionally presented, the CU can also indicates to DU the failure information of LTM cell switch without RLF report.

For LTM failure wihout RLF-report, in addition to the failure type (i.e., too late LTM, too early LTM and LTM to wrong cell) and UE ID (including gNB-DU F1AP ID and source C-RNTI), some basic information can also be provided to the gNB-DU by the gNB-CU, for example the ID of the source cell/failed cell of the gNB-DU, target cell of the LTM cell switch procedure, and ID of the re-established/re-connected cell.

Proposal 1: When RLF-report is absent, CU can indicate to DU the following LTM failure information:

- Failure type (i.e., too late LTM, too early LTM and LTM to wrong cell)

- UE ID (gNB-DU F1AP ID, source C-RNTI)

- ID of the source/failed cell of the gNB-DU, and ID of target LTM cell

- ID of re-established/re-connected cell

2.2 Ping-pong avoidance for LTM

At the last RAN3 meeting, it was agreed that DU should be able to resolve LTM ping-pong based on information over F1. And it is not clear what kind of information is sent from the CU to the DU. One straightforward Option is that the CU forwards the lastest UHI to the target DU in each CU-DU Cell Switch Notification message. And an althernative option is that CU identitfies the happening of ping-pong cell swich and indicate information related to ping-pong cell switch to the DU. In our understanding, both option works. However, for the second option, RAN3 needs to spend additional effort discussing the exact information related to the ping-poing, and that the ping-pong already happened cannot be avoided. While for the first option, the existing IE can be reused directly, and the ping-pong cell switch can be avoided from the beginning.

Proposal 2: Introduce UE History Information in CU-DU Cell Switch Notification message

2.3 Cell switch information for LTM

In conventional L3 handover procedure, the source gNB provides to the target gNB Handover Request message including a mobility information, which enable later analysis of the conditions that led to a wrong HO. Upon reception of the mobility information, the target gNB shall, if supported, store this information. And when problem of the related HO procedure is detected, for example a too late HO or HO to wrong cell happens for the HO associated with the mobility information, the target gNB may send to the source gNB a Handover Report message including the mobility information.

Simiarly, for LTM, a “cell switch information” which identifies paramenters and conditions of the cell switch procedure can be sent from the source DU to the CU, such that when problem of the LTM cell switch procedure is detected, the CU can send to the source DU the Access and Mobility Indication message including the “cell switch information” associated with the LTM cell switch procedure. With the cell switch information, the source DU can further analysis the condition that led to the wrong LTM cell switch.

Proposal 3: Introdcue “Cell Switch Information” in DU-CU Cell Switch Notification message and Access and Mobility Indication message.

# 3. Conclusion

In this paper, we provide our further consideration on MRO for LTM, and have the following proposals:

Proposal 1: When RLF-report is absent, CU can indicate to DU the following LTM failure information:

- Failure type (i.e., too late LTM, too early LTM and LTM to wrong cell)

- UE ID (gNB-DU F1AP ID, source C-RNTI)

- ID of the source/failed cell of the gNB-DU, and ID of target LTM cell

- ID of re-established/re-connected cell.

- ID of re-established/re-connected cell.

Proposal 2: Introduce UE History Information in CU-DU Cell Switch Notification message

Proposal 3: Introdcue “Cell Switch Information” in DU-CU Cell Switch Notification message and Access and Mobility Indication message.

Annex	TP for LTM for 38.473

### 8.3.9	DU-CU Cell Switch Notification

#### 8.3.9.1	General

The purpose of the DU-CU Cell Switch Notification procedure is to enable the gNB-DU to inform the gNB-CU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-DU to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: DU-CU Cell Switch Notification procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU CELL SWITCH NOTIFICATION message.

Upon reception of the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

Interaction with CU-DU Cell Switch Notification procedure:

If the LTM Cell Switch Information IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

If the TA Information List IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message to transfer the TA value(s).

If the Cell Switch Information IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, store this information.

8.3.9.3	Unsuccessful Operation

Not applicable.

8.3.9.4	Abnormal Conditions

Not applicable.

### 8.3.10	CU-DU Cell Switch Notification

#### 8.3.10.1	General

The purpose of the CU-DU Cell Switch Notification procedure is to enable the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

#### 8.3.10.2	Successful Operation

Figure 8.3.10.2-1: CU-DU Cell Switch Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CELL SWITCH NOTIFICATION message.

Upon reception of the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the LTM Cell Switch Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the TA Information List IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the Tag ID Pointer IE is also included, the gNB-DU shall, if supported, consider it to determine the TAG corresponding to the received TA value.

If the UE History Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it for subsequent LTM cell switch initiation.

#### 8.3.10.3	Unsuccessful Operation

Not applicable.

#### 8.3.10.4	Abnormal Conditions

Not applicable.

---------------------------------------------------------------next change-----------------------------------------------------------------

8.11.1	Access and Mobility Indication

8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the LTM Failure information IE, the gNB-DU may take it into account for optimization of LTM cell switch related paramenters.

8.11.1.3	Abnormal Conditions

Not applicable.

---------------------------------------------------------------next change-----------------------------------------------------------------

#### 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| Cell Switch Information | O |  | BIT STRING (SIZE (32)) | Information related to the cell switch; the source gNB-DU node provides it in order to enable later analysis of the conditions that led to a wrong cell switch. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| UE History Information | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

---------------------------------------------------------------next change-------------------------------------------------------------------

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| >>Cell Switch Information | O |  | BIT STRING (SIZE (32)) | Information provided in the DU-CU CELL SWITCH NOTIFICATION message from the source gNB-DU. | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| LTM Failure information | O |  | 9.3.1.z | Includes information of a LTM cell switch failure event. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

---------------------------------------------------------------next change-------------------------------------------------------------------

9.3	Information Element Definitions

9.3.1	Radio Network Layer Related IEs

[skip]

#### 9.3.1.x	UE History Information

The UE History Information IE contains information about cells that a UE has been served by in active state prior to the target cell. The overall mechanism is described in TS 36.300 [20].

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 38.413 [3].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list |
| >Last Visited Cell Information | M |  | 9.3.1.y |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |

#### 9.3.1.y	Last Visited Cell Information

The Last Visited Cell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Last Visited Cell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Last Visited NG-RAN Cell Information | M |  | OCTET STRING | Defined in TS 38.413 [3]. |

#### 9.3.1.z	LTM failure Information

The LTM failure Information IE contains information about a LTM failure event.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Assistant Identifier | M |  | gNB-DU UE F1AP ID 9.3.1.5 |  |
| C-RNTI | M |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU |
| LTM failure type | M |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  |
| Source cell CGI | M |  | NR CGI 9.3.1.12 | NR CGI of source cell of the LTM cell switch procedure or failed cell of the gNB-DU. |
| Target cell CGI | O |  | NR CGI 9.3.1.12 | NR CGI of target cell for LTM cell switch procedure |
| Re-establishment cell CGI | O |  | NR CGI 9.3.1.12 | NR CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the LTM failure |
| Cell Switch Information | O |  | BIT STRING (SIZE (32)) | Information provided in the DU-CU CELL SWITCH NOTIFICATION message from the source gNB-DU. |