3GPP RAN WG3 Meeting #127-bis		R3-251713

Wuhan, China, 7th – 11th April 2025

Source:	CATT

Title:	Clarification on MBS broadcast service area

Agenda Item:	14.2

Document for:	Discussion and Decision

# 1. Introduction

For MBS broadcast service topic, the NR NTN Phase 3 WID [1] content is as follows:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

The last RAN3 meeting has already agreed that encode the Intended MBS service area as OCT STRING which refers to RAN2 spec. In this contribution, we will continue to discuss the issues related to MBS broadcast service topic.

# 2. Discussion

The previous RAN3 meeting has agreed that the intended service area provided to the gNB can be Mapped Cell ID(s), TAI(s), or geographical service area information and send LS to SA2, RAN2 and CT4. However, some companies still have differing interpretations regarding the relationships between Cells, TAIs, and intended MBS service areas (ISAs), which require further clarification. In our opinion, any combination of these three IE is technically feasible, including ISAs alone. Cells, TAIs, and ISAs can exist independently and stand alone. If the AMF only provides TAI list, the gNB can broadcast the geographical fixed areas of the TAI(s) to the UE using legacy procedure or translate into the RAN2 format. If the AMF only provides mapped cell ID list, the gNB needs to translate the geographical fixed areas of mapped cell ID(s) into the RAN2 format. And when the AMF only provides ISAs, the gNB can decode the ISA to determine the Uu cell. There is no problem for gNB to decode the OCTET STRING of ISAs, depending on gNB implementation.

Proposal 1: There is no problem for a gNB to decode the OCTET STRING of Intended Service Area (ISA), depending on the implementation.

Proposal 2: For NR NTN MBS broadcast, using ISA list only is allowed, any combination of Cell list, TAI list, and ISA list should be allowed.

And when Cells, TAIs, and ISAs are used in combination, the MBS broadcast service area should be the union of their coverage ranges. If TAI, Cell, and ISA are provided to the gNB in any combination, the gNB could translate the above geographical fixed areas of MBS service areas into the RAN2 format, which is a set of circles or a set of polygons. And the picture below shows a typical example of the MBS service area formed by the combination of Cells and ISAs. The MBS service area of the satellite is the union of the following four geographically fixed areas: Cell 1 and Cell 2 of the TAC 1, ISA 1and ISA 2 of the TAC2.

Figure 1 MBS Service Area of NTN

Proposal 3: When Cells, TAIs, and ISAs are used in combination, the MBS broadcast service area should be the union of their coverage ranges.

SA2 replied LS in last RAN3 meeting [2], and the description of MBS Service Area in TS 23.247 CR#0372 is as follows:

Specifically for NR NTN, an MBS service area in the 5GC and towards NG-RAN may be denoted with an NR NTN Intended Service Area as defined in TS 38.413 [15] in combination with a TAI list and/or cell ID list, for instance to denote an area smaller than a cell. The NR NTN Intended Service Area is only used in NG-RAN to further restrict the MBS service area denoted by TAIs and/or cell IDs as defined in TS 38.300 [9].

The previous RAN2 meeting has already agreed that the intended service area can cover the area of more than one NTN cells (or portions thereof). The understanding of SA2 is that the MBS Service Area provided by NR NTN Intended Service Area should be combined with the TAI list and/or the cell ID list. The understanding of the relationship between Cell TAI and ISA by SA2 needs to align with RAN3.

Proposal 4: SA2 understanding of the relationship between Cell, TAI and ISA should be align with RAN3. It is proposed to agree the related draft reply LS in appendix.

# 3. Conclusion

According to the analysis in section 2, the following observations and proposals are provided:

Proposal 1: There is no problem for a gNB to decode the OCTET STRING of Intended Service Area (ISA), depending on the implementation.

Proposal 2: For NR NTN MBS broadcast, using ISA list only is allowed, any combination of Cell list, TAI list, and ISA list should be allowed.

Proposal 3: When Cells, TAIs, and ISAs are used in combination, the MBS broadcast service area should be the union of their coverage ranges.

Proposal 4: SA2 understanding of the relationship between Cell, TAI and ISA should be align with RAN3. It is proposed to agree the related draft reply LS in appendix.

# 4. Reference

- RP-240775 Rel-19 Moderator WID NR NTN phase 3RAN3_126_agenda_20241122_EOM1

- R3-250072 Reply LS on Supporting MBS broadcast service for NR NTN

- Draft Reply LS to SA2 (S2-2501329/R3-250072)

Title:	[DRAFT] Reply LS on Supporting MBS broadcast service for NR NTN

Response to:	LS (S2-2501329/R3-250072) on < Reply LS on Supporting MBS broadcast service for NR NTN >

Release:	Rel-19

Work Item:	NR_NTN_enh

Source:	CATT [to become RAN3]

To:	SA2

Cc:

Contact Person:	Jiancheng Sun

sunjiancheng@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 thanks SA2 for the reply LS on Supporting MBS broadcast service for NR NTN. RAN3 needs to clarify the following issues:

- Cells, TAIs, and ISAs can exist independently and stand alone. When the AMF only provides ISA(s), the gNB can decode the ISA(s) to determine the Uu cell for broadcast.

- Any combination of Cell list, TAI list, and intended MBS service area (ISA) list should be allowed. When Cells, TAIs, and ISAs are used in combination, the MBS broadcast service area should be the union of their coverage ranges.

2. Actions:

To SA2 group:

ACTION: 	RAN3 kindly asks SA2 to take the above information into account.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #128	19-23 May 2025			Malta, MT

TSG-RAN3 Meeting #129			25-29 Aug 2025			India, IN