3GPP TSG-RAN WG3 Meeting #127-bis                           	  R3-251715

Wuhan, CN, 7th – 11th April, 2025

Source:	CATT

Title:	(TP to BL CR for TS38.300) A-IoT protocol stack

Agenda Item:	16.2

Document for:	Discussion and Decision

- Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signallings, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved(WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In the last RAN3 meeting, we discussed the architecture aspects to support AIoT Topology 1, corresponding agreements and working assumptions are listed below:

RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1.

WA: Take Option1 (Including AIoTF information in NGAP).

WA: The AIoT RAN node may only provide the AIoT radio.

In this contribution, we will further discuss the architecture aspects on support of Ambient IoT, especially the protocol stack on support of A-IoT. Base on the discussion, we will provide the observations and proposals accordingly.

- Discussion

In the last RAN3 meeting, we discussed the architecture of AIoT for Topology 1. It’s agreed that both direct and indirect path between AIoT RAN and AIoTF are supported, and it’s discussed the protocol stack and achieved the WA as below:

- RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1.

- WA: Take Option1 (Including AIoTF information in NGAP).

To our understanding, both options (AIoTF Information in NGAP vs. NRPPa like new protocol layer over NGAP) are workable. Option 2 (new protocol layer) is better to minimize the impact to AMF, especially for the indirect path deployment. However, majority of companies preferred to go for the option 1, the information not associated with AMF is transferred via AIoT Information Transfer (like SMF container). To make progress, we’re ok to turn the WA to the agreement, and further discuss which information should be included in the container, and which information should be the NGAP IE.

Proposal 1: Turn the “WA: Take Option1 (Including AIoTF information in NGAP).” to the agreement.

In the last RAN3 meeting, we also discussed whether an AIoT RAN node provides AIoT Radio only, or provides AIoT Radio and legacy NR Uu. The WA is:

- WA: The AIoT RAN node may only provide the AIoT radio.

Our understanding is AIoT RAN node may upgrade from a legacy gNB, which could support both NR Uu and A-IoT Radio, it may be a new RAN node only for AIoT services, which only support AIoT Radio. As above, we do not see any limitation to the implementation. Therefore, the WA could be turned to the agreement.

Proposal 2: Turn the “WA: The AIoT RAN node may only provide the AIoT radio.” to the agreement.

Base on the discussion above, the protocol stack to support A-IoT Topology 1 could be illustrated with the figure below:

Figure 1. Protocol stack for A-IoT Topology 1

In this figure, A-IoT CN could be AIoTF for direct communication case, or AMF + AIoTF for indirection communication case. It’s beneficial to capture the figure of protocol stack in the stage 2 for easy reading and better understanding.

Proposal 3: Beneficial to capture the figure of protocol stack for A-IoT Topology 1.

Due to the architecture and protocol stack RAN3 discussed may have some impact the core network, it’s better to send a liaison to SA2 to indicate our progress and decision. Therefore, SA2 could further consider the impact to AIoTF and AMF accordingly in their normative work.

Proposal 4: Send LS to SA2 to indicate RAN3 progress on AIoT architecture and protocol stack, drat LS is provided in the Annex.

Base on the discussion above, we provide the stage 2 TP for TS 38.300, in the section 5.

Proposal 5: Discuss and agree the TP to TS38.300 on AIoT system architecture aspects.

- Conclusion

Based on the discussion in section 2, the following proposals are provided:

Proposal 1: Turn the “WA: Take Option1 (Including AIoTF information in NGAP).” to the agreement.

Proposal 2: Turn the “WA: The AIoT RAN node may only provide the AIoT radio.” to the agreement.

Proposal 3: Beneficial to capture the figure of protocol stack for A-IoT Topology 1.

Proposal 4: Send LS to SA2 to indicate RAN3 progress on AIoT architecture and protocol stack, drat LS is provided in the Annex.

Proposal 5: Discuss and agree the TP to TS38.300 on AIoT system architecture aspects.

- Reference

- TR 38.769 TR 38.769 Study on solutions for ambient IoT (Internet of Things)

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- TR 23.700-13-110 Study on Architecture support of Ambient power-enabled Internet of Things

- R3-251563 (BL CR to 38.300) Introduction of Ambient IoT

- TP to BL CR for TS 38.300

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

## 16.xx	Support of Ambient IoT

### 16.xx.x1	Introduction

### 16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	As specified in <reference to SA2 TS to be added> the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct communication to AIOTF) or the AMF (in case of indirect communication to AIOTF).

NOTE 1:	It is not expected a deployment will deploy both direct communication and indirect communication.

-	In this version of the specification A-IoT specific data is transported between the gNB and the A-IoT CN node takes place via the NG-C interface, where AIoTF information is transferred transparently via AMF in case of indirect communication to AIoTF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

As above, the protocol stack on support of A-IoT Topology 1 is illustrated with the figure below:

Figure 16.xx - 1

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>

- Draft LS to SA2 on Architecture aspects

Title:	[Draft] LS on Architecture aspects on support of AIoT topology 1

Response to:	-

Release:	Rel-19

Work Item:	Ambient_IoT_Solutions

Source:	CATT (to be RAN3)

To:	SA2

Cc: 	SA4, RAN2

Contact Person:

Name:	Jiancheng Sun

E-mail Address:	sunjiancheng@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

On Solutions for Ambient IoT (Internet of Things) in NR, RAN3 discussed the architecture and protocol stack considering both direct and indirect communication between AIoT RAN and AIoTF are supported.

RAN3 agreed to specify AIoT related procedures in the NGAP, where the AIoTF related information is transferred transparently by AMF (similar to SMF container) for indirect communication case.

RAN3 would like SA2 to take it into account in the specification work.

2. Actions:

To SA2:

ACTION: RAN3 kindly asks SA2 to take above into consideration for their specification work.

3. Date of Next RAN3 Meetings:

RAN3#128	19th – 23rd May 2025				Malta, MT

RAN3#129	25th  – 29th August 2025			India(TBC), IN