3GPP TSG-RAN WG3 #127-bis                                         R3-251716

Wuhan, CN, 7th – 11th Apr, 2025

Source:	CATT

Title:	(TP to BL CR for TS38.413) A-IoT inventory and command procedures

Agenda Item:	16.2

Document for:	Decision

- Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signallings, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved(WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In the last RAN3 meeting, we discussed the inventory and command procedure for AIoT Topology 1, corresponding agreements and working assumptions are listed below:

WA: Introduce a NGAP Class 1 Inventory Request procedure.

Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE.

Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE.

Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE.

Introduce a NGAP Class 2 Inventory Report procedure.

Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE.

The above Transfer IEs are transparent to AMF in case of indirect communication.

WA: Introduce a NGAP Class 1 Command Request procedure

Introduce COMMAND REQUEST message, which includes Command Request Transfer IE.

Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE.

Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE.

The above Transfer IEs are transparent to AMF in case of indirect communication.

In this contribution, we will further discuss the inventory and command procedures and signaling impact on support of Ambient IoT. Base on the discussion, we will provide the proposals accordingly.

- Discussion

2.1. Inventory procedure

For signalling message for A-IoT Inventory procedure, there are inventory request, inventory response and inventory report message captured in candidate procedures.

For AIoT Inventory request message, the following information was discussed in last RAN3 meeting:

FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

For correlation identifier, it is also mentioned in SA2 and RAN2 as below:

- RAN2 Agreements

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

In S2-2502364[1] which is agreed in SA2, the following is captured:

- NOTE Y: The AIoT responses aggregated from the AIoT devices share the same correlation identifier.

From RAN3 perspective, Correlation Identifier could be used per AIOTF to identify certain inventory/command procedure.

But among different AIOTFs, only Correlation Identifier is not sufficient to identify a procedure, then, AIOTF Identifier is also needed in case of the same Correlation Identifier used by different AIOTF.

Since Correlation Identifier is only used in RAN and AIoTF, it should be included in Inventory Request Transfer IE within INVENTORY REQUEST message. To correlate INVENTORY RESPONSE/FAILURE and INVENTORY REPORT message, Correlation Identifier and AIOTF Identifier should also be included in these messages.

Proposal 1: Correlation Identifier should be included in Inventory Request Transfer IE within INVENTORY REQUEST, INVENTORY RESPONSE/FAILURE and INVENTORY REPORT messages.

AIOTF Identifier may be used by AMF for routing in case of indirect communication with AIOTF, thus, the AIOTF Identifier should be included in the INVENTORY REQUEST, INVENTORY RESPONSE/FAILURE and INVENTORY REPORT messages, outside the Transfer container.

Proposal 2: AIOTF Identifier should be included in INVENTORY REQUEST, INVENTORY RESPONSE/FAILURE and INVENTORY REPORT, which is used by RAN and AMF for routing with correct AIoTF.

In S2-2502364[3] which is agreed in SA2, the following is captured:

f. The AIOTF may provide the AIoT aggregation assistance information to the AIoT RAN. Aggregation assistance information is used to assist in the aggregation of AIoT responses received from multiple AIoT devices. The AIOTF may determine the aggregation assistance information based on operator policy, considering information provided from the AF, which consists of: - Time interval: The fixed time interval for which AIoT RAN collects AIoT Device responses before AIoT RAN reports the aggregated AIoT responses to AIOTF. This may potentially happen multiple times until the AIoT RAN completes the request operation. If the AF has provided a time interval, then the AIOTF should signal a time interval to AIOT RAN that is equal or shorter than the time interval received from the AF.

SA2 suggest that Time interval may be provided to AIOT RAN. With Time interval IE, AIOT RAN should send response message (including inventory report and command response message) within this period of time. In case of time limit exceeded, the operation should be considered as completed. To align with SA2, Time interval should be included in INVENTORY REQUEST message, in the Inventory Request Transfer IE.

Proposal 3: It is proposed to include Time interval in the Inventory Request Transfer IE of the INVENTORY REQUEST message.

In S2-2502364[3], SA2 ask RAN3 to send a complete message to AIOTF as below:

NOTE Z:	AIOTF is expected to receive an end indication from AIoT RAN, signaling the completion of reporting for a specific correlation ID.

When AIOTF triggers inventory to multiple devices, whether AIoT RAN could detect the completion of inventory procedure is up to RAN1 and RAN2. If needed, we can further work on that according to the LS.

As for the completion of inventory procedure, whether there is inventory complete message from CN to AIoT RAN needs consideration. The Time interval IE in INVENTORY REQUEST message may implicitly indicate the completion from CN. So, for CN, whether there is a completion of inventory procedure and how to indicate the completion may need clarification from SA2.

Proposal 4: whether to introduce AIoT Inventory Complete message from RAN to CN or from CN to RAN is pending to the other WGs (RAN1, RAN2 and SA2).

2.2 Command procedure

For signalling message for A-IoT command procedure in Topology 1, there are command request and command response captured in candidate procedures.

For command report message, the following FFS is captured:

FFS: The need for a Command Report procedure?

In case of writing device, the result of action “write” is included in command response message.

In case of reading device, the “data” as the result of read operation could also be include in command response message.

So, it is unnecessary to introduce Command Report procedure.

Proposal 5: No need to introduce Command Report procedure.

As for Correlation Identifier and AIOTF Identifier, Command related messages were discussed.

FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

For the same reason as inventory procedure, Correlation Identifier and AIOTF Identifier could be used to identify a procedure.

Proposal 6: Both Correlation Identifier and AIOTF Identifier are needed to identify certain command procedure.

As for High-level procedure, following agreement was achieved in last RAN2 meeting.

Agreements

From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

Only command procedure after inventory procedure is supported in RAN2. So, in inventory procedure, an indication may be needed to indicate whether there is a subsequent command procedure. For only inventory case, AIoT RAN could release device context. While for inventory and command case, it may be needed for AIoT RAN to keep device context and resource for following command procedure.

Proposal 7: For Inventory and Command, an indicator should be introduced in Inventory Request for AIoT RAN keeping device context and resource for the following command procedure.

For per device or multiple devices command procedure, the open issue is captured below:

FFS: Command request procedure is a per device procedure or to be used for multiple devices?

We support per device command procedure for the following reasons:

- The AIoT command information is encapsulated as NAS PDU. For NAS PDU, it is per device design due to security consideration. So, it is not reasonable to send the same NAS PDU to multiple devices.

- For command after inventory procedure, after receiving per device inventory report message, per device command procedure could be triggered at once. This solution is simple and quick.

Proposal 8: It is proposed to only introduce per device command request procedure, not support a single command for a group of devices, or multiple devices.

To correlate inventory and the following command procedure, Correlation Identifier and AIOTF Identifier could also be used. But in case of multiple devices inventory and the following per device triggered command procedure, only Correlation Identifier and AIOTF Identifier is not sufficient to correlate per device command procedure. The following FFS is captured in last RAN3 meeting:

FFS: Device association between gNB and AIOTF is used in Command procedure.

Correlation Identifier and AIOTF Identifier could be used to correlate inventory and command procedure. Within command procedure, per device association may be needed.

There is an example of introducing per device association. In inventory report message, besides Correlation Identifier and AIOTF Identifier, NGAP device ID as device association is sent to AIOTF. In the following command request message, NGAP device ID is included to indicate the same device. With this information, AIoT RAN could use the previous kept resource for command procedure.

Proposal 10: It is proposed to introduce device association between gNB and AIOTF for inventory and command procedure. For example, NGAP device ID in introduced to indicate certain device.

- Conclusions

In this contribution, we mainly discussed A-IoT Procedures and Information exchange between A-IoT CN and A-IoT RAN. Based on the discussion above, the following proposals are provided:

Inventory procedure：

Proposal 1: Correlation Identifier should be included in Inventory Request Transfer IE within INVENTORY REQUEST, INVENTORY RESPONSE/FAILURE and INVENTORY REPORT messages.

Proposal 2: AIOTF Identifier should be included in INVENTORY REQUEST, INVENTORY RESPONSE/FAILURE and INVENTORY REPORT, which is used by RAN and AMF for routing with correct AIoTF.

Proposal 3: It is proposed to include Time interval in the Inventory Request Transfer IE of the INVENTORY REQUEST message.

Proposal 4: whether to introduce AIoT Inventory Complete message from RAN to CN or from CN to RAN is pending to the other WGs (RAN1, RAN2 and SA2).

Command procedure：

Proposal 5: No need to introduce Command Report procedure.

Proposal 6: Both Correlation Identifier and AIOTF Identifier are needed to identify certain command procedure.

Proposal 7: For Inventory and Command, an indicator should be introduced in Inventory Request for AIoT RAN keeping device context and resource for the following command procedure.

Proposal 8: It is proposed to only introduce per device command request procedure, not support a single command for a group of devices, or multiple devices.

Proposal 9: It is proposed to introduce device association between gNB and AIOTF for inventory and command procedure. For example, NGAP device ID in introduced to indicate certain device.

- Reference

[1] TR 38.769 Study on solutions for ambient IoT (Internet of Things)

[2] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] S2-2502364” Conclusions for KI#1 on the aggregation aspects” SA WG2 Meeting #167

- TP to BL CR for TS38.413

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

### 9.2.x		A-IoT Messages

Editor’s Note: The A-IoT messages described in the section are FFS and the A-IoT messages are carried over NGAP based on RAN3 working assumption.

#### 9.2.x.1	INVENTORY REQUEST

This message is sent by the A-IoT CN node to trigger the NG-RAN node to perform inventory over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Inventory Request Transfer | M |  | OCTET STRING | Containing the Inventory Request Transfer IE specified in subclause 9.3.x.1 | YES | reject |
| AIOTF Identification | M |  | FFS |  | YES | reject |

#### 9.2.x.2	INVENTORY RESPONSE

This message is sent by the NG-RAN node to report the successful outcome of the request from the INVENTORY REQUEST message.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Inventory Response Transfer | M |  | OCTET STRING | Containing the Inventory Response Transfer IE specified in subclause 9.3.x.2 | YES | reject |
| AIOTF Identification | M |  | FFS |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.3	INVENTORY FAILURE

This message is sent by the NG-RAN node to report the unsuccessful outcome of the request from the INVENTORY REQUEST message.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Inventory Failure Transfer | M |  | OCTET STRING | Containing the Inventory Failure Transfer IE specified in subclause 9.3.x.3 | YES | ignore |
| AIOTF Identification | M |  | FFS |  | YES | reject |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.4	INVENTORY REPORT

This message is sent by the NG-RAN node to provide the inventory report to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Inventory Report Transfer | M |  | OCTET STRING | Containing the Inventory Report Transfer IE specified in subclause 9.3.x.4 | YES | reject |
| AIOTF Identification | M |  | FFS |  | YES | reject |

#### 9.2.x.5	COMMAND REQUEST

This message is sent by the A-IoT CN node to the gNB to trigger the command transmission over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Command Request Transfer | M |  | OCTET STRING | Containing the Command Request Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| AIOTF Identification | M |  | FFS |  | YES | reject |

9.2.x.6	COMMAND RESPONSE

This message is sent by the NG-RAN node to provide the command response to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Command Response Transfer | M |  | OCTET STRING | Containing the Command Response Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| AIOTF Identification | M |  | FFS |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.7	COMMAND FAILURE

This message is sent by the NG-RAN node to report the unsuccessful transmission of the command to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Command Failure Transfer | M |  | OCTET STRING | Containing the Command Failure Transfer IE specified in subclause 9.3.6.5. | YES | ignore |
| AIOTF Identification | M |  | FFS |  | YES | reject |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

-----------------Start of the Next  Change-----------------

9.3.x	AIOTF Related IEs

9.3.x.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation Identification | M |  | FFS |  |
| Estimated Number of devices | O |  | INTEGER (0..FFS) |  |
| CHOICE Inventory Area | O |  |  |  |
| >TAI |  |  |  |  |
| >>TA List |  | 1..<maxnoofTAIforAIoT> |  |  |
| >>>TA I | M |  | 9.3.3.11 |  |
| >Reader |  |  |  |  |
| >>Reader List |  | 1..<maxnoofReader> |  |  |
| >>>Reader ID | M |  | INTEGER (1..65536,…) |  |
| NAS-PDU | O |  | OCTET STRING |  |
| Time interval | O |  | FFS |  |
| Command indication | O |  | FFS |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforAIoT | Maximum no. of TAI subject for AIoT. Value is 65536. |
| maxnoofReader | Maximum no. of Readers served by an AIoT RAN. Value is 65536. |

9.3.x.2	Inventory Response Transfer

This IE provides the inventory response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation Identification | M |  | FFS |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

9.3.x.3	Inventory Failure Transfer

This IE provides the inventory failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation Identification | M |  | FFS |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

9.3.x.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation Identification | M |  | FFS |  |
| Reader ID | M |  | INTEGER (1..65536,…) |  |

9.3.x.5	Command Request Transfer

This IE provides the command request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation Identification | M |  | FFS |  |
| NAS-PDU | O |  | OCTET STRING |  |

9.3.x.6	Command Response Transfer

This IE provides the command response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation Identification | M |  | FFS |  |
| NAS-PDU | O |  | OCTET STRING |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

9.3.x.7	Command Failure Transfer

This IE provides the command failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation Identification | M |  | FFS |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>