3GPP TSG-RAN WG3 Meeting #127-bis                           	  R3-257120

Wuhan, CN, 7th – 11th April, 2025

Source:	CATT

Title:	(TP to BL CRs) Support of gNB-based AI positioning

Agenda Item:	20.2

Document for:	Discussion and Decision

1 Introduction

In the last RAN3 meeting, we initially discussed the potential RAN3 impact to support AI/ML assisted Positioning, i.e. the Case 3a. According to the discussion [1], some agreements and open issues as below:

The existing LCS framework is re-used for AI/ML positioning. The AI/ML model (inference and training functions) is at the gNB. LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. FFS if LMF should be aware that the reported measurements are AI/ML generated. Check progress in RAN1 and solution discussion… RAN3#127 Agreements: Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement. For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner. For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs. WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB. To be continued: Stage 2 (TS 38.305) text proposals for case 3a data collection. Companies are encouraged to consolidate solutions, if possible.

In this contribution, we will discuss the potential RAN3 impact to support AI/ML assisted positioning, taking the RAN1 progress/agreements into account. Base on the discussion, we will provide corresponding observations and proposals.

2 Discussion

2.2 Training data collection

In the previous RAN3 meetings, we discussed the procedures of training data collection for gNB based AI positioning (case 3a), corresponding agreements and open issues are listed below:

For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner. For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs. WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB. To be continued: Stage 2 (TS 38.305) text proposals for case 3a data collection. Companies are encouraged to consolidate solutions, if possible.

The potential solutions for data training as summarized in the SoD [2] are listed below:

- Solution 1: gNB triggers AI/ML data collection for a specific UE via a UE-associated NGAP procedure.

- Solution 2: gNB triggers AI/ML data collection for a specific UE via an NRPPa message.

- Solution 3: LMF triggers AI/ML data collection and provides both Part A and Part B to the gNB in a new NRPPA message.

- Solution 4: LMF triggers AI/ML data collection and provides Part B to the gNB in the MEASUREMENT REQUEST message.

- Solution 5: gNB requests Part B in the MEASUREMENT RESPONSE and LMF subsequently provides Part B in a new NRPPa message.

To make life easier, we’ll not discuss the above solutions one by one. Let’s discuss whether and how to take both of the opportunistic solution and proactive solution in following sections. Let’s give a simple definition to the opportunistic solution and proactive solution, as below:

- Opportunistic solution: collect training data on behalf of the on-going Positioning Measurements.

- Proactive solution: gNB proactively collect training data, large number of UEs.

In opportunistic solution, the ongoing positioning measurement procedures are reused, the measurement results are collected as part of the training data. This is a kind of optimization by reuse the ongoing positioning procedures. However, the amount of UE labels may be far from enough for data collection if only taking the UEs under positioning session into account.

Observation 1: The amount of UE labels may be far from enough for data collection if only taking the UEs under positioning session into account.

In contrast, Proactive solution is more suitable for collecting large amount of UEs’ training data. Therefore, it’s beneficial to support both Opportunistic solution and Proactive solution for data collection.

Proposal 1: Beneficial to support both Opportunistic solution and Proactive solution for data collection for gNB based model (case 3a).

Support of Proactive solution:

In the last meeting, we discussed and agreed that both served and non-served UEs could be selected for data collection, as below:

For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs.

According to the previous discussion, the key issue need to be decided first is which node is responsible for label UE selection. From gNB perspective, it knows the served UEs, but it does not know the UEs served by its neighbour gNBs.

Observation 2: If gNB is responsible to select UEs for data collection, it could only select its served UEs, not able to select the UEs from the other gNBs (e.g. neighbour gNBs).

In existing specification, gNB and AMF have the context of the connected UEs, but LMF has no knowledge about it. Thus, if UEs are selected by gNB, the gNB needs to indicate AMF the data collection request and provide the selected UEs to AMF, the AMF initiates positioning services for the selected UEs towards LMF.

Observation 3: No UE context in LMF, If gNB is responsible to select UEs for data collection, AMF should be involved, gNB triggers AI/ML data collection for a specific UE via a UE-associated NGAP procedure.

As above, it’s not proper to select UEs for data collection in the gNB, it’s more appreciate to select UE labels in 5GC. Then, let’s discuss how to select UE labels in 5GC. Two options:

Option 1: AMF select UE labels

- gNB requests for data collection towards AMF via NGAP, AMF selects UEs (UEs served by the gNB and probably other gNBs) and initiates Positioning procedures towards LMF.

Option 2: LMF select UE labels (may need coordination with AMF)

- gNB requests for data collection towards LMF via NRPPa, LMF selects UEs and initiates Positioning procedures towards the gNB. How the UEs are selected are up to LMF, maybe according to the configuration, or coordination with AMF is needed.

Considering the specification impact, it’s better to avoid the NGAP impact for AI Positioning, thus, the option 2 is preferred. If agreed, we can send the LS to SA2 to further consider the impact to 5GC, e.g. coordination between AMF, LMF and other functions within 5GC.

Observation 4: Beneficial to avoid NGAP impact for positioning features, including AI Positioning.

Proposal 2: For Proactive solution, gNB triggers AI/ML data collection request towards LMF via a new NRPPa signalling, LMF selects UE labels (with probably coordination with AMF) and initiates Positioning measurement accordingly.

In the last RAN3 meeting, we agreed that LMF starts a NRPPa transaction, the gNB determines that data collection is needed.

Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement.

If there’s no NRPPa transaction between gNB and LMF, it’s not possible for gNB to request for training data towards LMF. Following the agreement, it’s better to introduce a new class 2 message from LMF to gNB to initiate the NRPPa transaction, e.g. named as DATA COLLECTION NOTIFICATION.

If there’re already some ongoing NRPPa transactions between gNB and LMF, this indication may be used to indicate gNB the LMF is ready for data collection, however, it’s not a mandatory procedure in this case.

Proposal 3: For Proactive solution, to initiate an NRPPa transaction between LMF and gNB, a new class 2 NRPPa signalling need to be introduced, e.g. DATA COLLECTION NOTIFICATION.

For Proactive solution, gNB requests for the data collection towards LMF when needed, which may include the TRP IDs, the intended number of UE labels in the request message. LMF decides the UE labels and triggers the Positioning measurement towards the UE labels. There’s a working assumption the last meeting:

WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB.

On top of the WA, let’s further discuss when and how to provide the Part B from LMF to gNB. We understand this is pending to how Part B is generated in LMF.

- If part B is proactively obtained by LMF before triggering the Measurement Request, Part B could be included in the Measurement Request. In this case, gNB could obtain the Part A internally via the Positioning measurement and associate Part A and Part B as a full set of training data.

- If Part B is generated after LMF get the measurement report from the gNB, a new NRPPa message seems needed for provision of such kind of info to the gNB. Furthermore, we should consider how to associate the Part A and Part B. As part A is already reported to LMF as part of measurement report, we understand easiest way is to provide both Part A and Part B in the new signalling from LMF to gNB. With that, legacy NRPPa Measurement Procedure could be reused without any enhancement.

Proposal 4: For Proactive solution, if Part B is proactively obtained by LMF before triggering the Measurement Request, Part B could be included in the Measurement Request, gNB could obtain the Part A internally via the Positioning measurement and associate Part A and Part B as a full set of training data.

Proposal 4bis: For Proactive solution, if Part B is generated after LMF get the measurement report from the gNB, the easiest way is to provide both Part A and Part B in a new signalling from LMF to gNB, e.g. DATA COLLECTION UPDATE. (No impact to the existing measurement procedures)

For Case 3a, the AI/ML model is deployed at gNB side and the channel measurement entity is also gNB, gNB may request for different information for training, thus, some assistance info should be provided to LMF when request for the training data, e.g. TRP IDs, expected amount of labels, expected measurement type/result (e.g. UL-RTOA, Multi-RTT).

Proposal 5: For Proactive solution, gNB should provide some assistance information to LMF when request for training data, e.g. TRP IDs, expected amount of labels, expected measurement type/result (e.g. UL-RTOA, Multi-RTT).

Moreover, since the AI/ML model is deployed at gNB side, gNB may have its preferred UL SRS configuration for training data collection. However, currently, only LMF has the capability of requesting preferred SRS transmission characteristics, e.g., the density and periodicity of SRS resources. During the request process, LMF does not have prior knowledge about gNB-side model, i.e., LMF does not know the SRS configuration most suitable for gNB-side models, no matter from consistency or measurement perspective. To tackle this issue, gNB can provide its preferred SRS configuration to LMF in advance. In this way, LMF can better negotiate with different TRPs to determine the SRS configuration most suitable for gNB side models. Specifically, the report can be performed in explicit (request specific SRS transmission characteristics) or implicit manner (via associated ID).

Proposal 6: For data collection in Case 3a, it’s beneficial for gNB to provide the preferred UL SRS characteristics/configuration for training data collection to LMF.

On support of opportunistic solution:

For opportunistic solution, both gNB and LMF could decide to take the ongoing positioning measurement into account for data collection.

If UE label is decided by the gNB, e.g. take the ongoing positioning measurement into account, gNB needs to request for Part B in the Measurement Response/Report. LMF could use a new NRPPa signalling to provide Part B to the gNB, same signalling could be used for both opportunistic solution and Proactive solution, e.g. DATA COLLECTION UPDATE as shown in the figure above.

If UE label is decided by LMF, LMF indicates gNB the Positioning measurement procedure is to be used for data collection, then the gNB could store the Part A accordingly. On the provision of the Part B, same analysis as the proactive solution, refer to the Proposal 4 and 4bis above.

It’s proposed to have a unified solution for opportunistic solution and proactive solution, which means selection of the UE labels is left to LMF.

Proposal 7: For opportunistic solution, LMF could indicate gNB UE is selected for data collection in Measurement Request.

Proposal 8: For opportunistic solution, same way as Proactive solution (refer to P4, P4bis) could be used to provide Part B from LMF to gNB.

Base on the discussion above, the overall procedure for Proactive data collection is shown in the figure below:

Figure 1. Proactive Data Collection Procedures

1. LMF initiates the Data Collection Notification procedure towards the gNBs which holds the AI/ML positioning model to establish the NRPPa transaction.

Note: This step could be ignored if NRPPa transaction(s) has already been established between the gNB and LMF for other purposes.

2. When gNB decides to train the model for AI/ML based positioning, it may request for the training data from LMF.

3. The gNB sends the Data Collection Request towards the LMF, with some assistance information, e.g. expected amount of UE labels, expected data type.

4. LMF confirms the requirements of the gNB and replies with the response message.

5. LMF decides the UE labels and initiates proper positioning procedures to collect the training data via the Positioning Information Transfer and Measurement procedures.

6. When the training data is collected, LMF provides the training data to the gNB via the Data Collection Update.

Corresponding stage 2 and stage 3 TPs to support data collection for gNB-based AI positioning are provided in the Annex. RAN3 is requested to discuss and agree the TPs.

Proposal 9: Discuss and agree the stage 2 and stage 3 TPs for proactive data collection as provided in the Annex.

2.3 Inference

For Model Input/Report for inference, there’s a common understanding that:

- Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used.

We assume NRPPa Measurement procedure could be reused to support the purpose of training data collection, inference and monitoring for Case 3a. If any other information needs to be reported from gNB to LMF is pending to RAN1.

Proposal 10: In Case 3a, existing measurement procedure could be reused for training data collection, inference and monitoring. If any other information needs to be reported from gNB to LMF is pending to RAN1.

There’s an open issue that:

- FFS if LMF should be aware that the reported measurements are AI/ML generated.

Not sure about the purpose of such kind of indication, and what’s the expected behaviors of LMF if reported measurements are AI/ML generated or not. As we’ve already agreed that the performance monitoring for the case 3a is performed in the gNB, gNB may start or stop using the AI model for positioning measurement, not necessary to inform the LMF the reported measurements are AI/ML generated or not.

Proposal 11: For Case 3a, no need to indicate LMF the reported measurements are AI/ML generated or not.

2.4 Performance Monitoring

For performance monitoring, it’s already agreed that:

- RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

In RAN1 agreements, it’s also noted that:

- Note: Exact method to perform monitoring metric calculation is up to implementation.

- Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

How to perform performance monitoring, if any other information need to be provided to NG-RAN to be further discussed, taking RAN1 progress into account.

Proposal 12: How to perform performance monitoring in NG-RAN to be further discussed, taking RAN1 progress into account.

3. Conclusion

In this contribution, discussed the RAN3 impact to support AI/ML assisted positioning for AI assisted Positioning Accuracy enhancements. Based on the discussion above, the following proposals are provided:

Observation 1: The amount of UE labels may be far from enough for data collection if only taking the UEs under positioning session into account.

Proposal 1: Beneficial to support both Opportunistic solution and Proactive solution for data collection for gNB based model (case 3a).

Observation 2: If gNB is responsible to select UEs for data collection, it could only select its served UEs, not able to select the UEs from the other gNBs (e.g. neighbour gNBs).

Observation 3: No UE context in LMF, If gNB is responsible to select UEs for data collection, AMF should be involved, gNB triggers AI/ML data collection for a specific UE via a UE-associated NGAP procedure.

Observation 4: Beneficial to avoid NGAP impact for positioning features, including AI Positioning.

Proposal 2: For Proactive solution, gNB triggers AI/ML data collection request towards LMF via a new NRPPa signalling, LMF selects UE labels (with probably coordination with AMF) and initiates Positioning measurement accordingly.

Proposal 3: For Proactive solution, to initiate an NRPPa transaction between LMF and gNB, a new class 2 NRPPa signalling need to be introduced, e.g. DATA COLLECTION NOTIFICATION.

Proposal 4: For Proactive solution, if Part B is proactively obtained by LMF before triggering the Measurement Request, Part B could be included in the Measurement Request, gNB could obtain the Part A internally via the Positioning measurement and associate Part A and Part B as a full set of training data.

Proposal 4bis: For Proactive solution, if Part B is generated after LMF get the measurement report from the gNB, the easiest way is to provide both Part A and Part B in a new signalling from LMF to gNB, e.g. DATA COLLECTION UPDATE. (No impact to the existing measurement procedures)

Proposal 5: For Proactive solution, gNB should provide some assistance information to LMF when request for training data, e.g. TRP IDs, expected amount of labels, expected measurement type/result (e.g. UL-RTOA, Multi-RTT).

Proposal 6: For data collection in Case 3a, it’s beneficial for gNB to provide the preferred UL SRS characteristics/configuration for training data collection to LMF.

Proposal 7: For opportunistic solution, LMF could indicate gNB UE is selected for data collection in Measurement Request.

Proposal 8: For opportunistic solution, same way as Proactive solution (refer to P4, P4bis) could be used to provide Part B from LMF to gNB.

Proposal 9: Discuss and agree the stage 2 and stage 3 TPs for proactive data collection as provided in the Annex.

Proposal 10: In Case 3a, existing measurement procedure could be reused for training data collection, inference and monitoring. If any other information needs to be reported from gNB to LMF is pending to RAN1.

Proposal 11: For Case 3a, no need to indicate LMF the reported measurements are AI/ML generated or not.

Proposal 12: How to perform performance monitoring in NG-RAN to be further discussed, taking RAN1 progress into account.

4. Reference

- RAN3#127 Chair Notes

- RAN1#116/#116bis/#117/#118/118bis/#119/120 Chair’s Notes

Annex 1. (TP to BL CR for TS38.305) support of AI assisted Positioning

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 7.x	Procedures for Data Collection Information Transfer

### 7.x.1	General

To support AI/ML based positioning with gNB-side model, data collection information is used to enable the data training in gNB-side AI model.

### 7.x.2	Data Collection Information Transfer Procedures

Figure 7.x.2-1 shows the Data Collection Information Transfer Procedure.

Figure 7.x.2-1 Data Collection Information Transfer Procedures

1. LMF initiates the Data Collection Notification procedure towards the gNBs which holds the AI/ML positioning model to establish the NRPPa transaction.

Note: This step could be ignored if NRPPa transaction(s) has already been established between the gNB and LMF for other purposes.

2. When gNB decides to train the model for AI/ML based positioning, it may request for the training data from LMF.

3. The gNB sends the Data Collection Request towards the LMF, with some assistance information, e.g. expected amount of UE labels, expected data type.

4. LMF confirms the requirements of the gNB and replies with the response message.

5. LMF decides the UE labels and initiates proper positioning procedures to collect the training data via the Positioning Information Transfer and Measurement procedures.

6. When the training data is collected, LMF provides the training data to the gNB via the Data Collection Update.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

Annex 2. (TP to BL CR for TS38.413) support of AI assisted Positioning

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.10	NRPPa Transport Procedures

### 8.10.1	General

The purpose of the NRPPa Transport procedures is to carry NRPPa signalling (defined in TS 38.455 [19]) between the NG-RAN node and the LMF over the NG interface.

The Downlink UE Associated NRPPa Transport procedure and the Uplink UE Associated NRPPa Transport procedure use UE-associated signalling. The UE-associated signalling is used to support E-CID Location Information Transfer, Positioning Information Transfer, Measurement Preconfiguration Information Transfer, and Reporting of General Error Situations due to reception of an NRPPa message that utilized UE-associated signalling.

The Downlink Non UE Associated NRPPa Transport procedure and the Uplink Non UE Associated NRPPa Transport procedure use non-UE associated signalling. The non-UE associated signalling is used to support OTDOA Information Transfer, Assistance Information Transfer, TRP Information Transfer, Measurement Information Transfer, PRS Information Transfer, Area-specific Information Transfer, Data Collection Information Transfer and Reporting of General Error Situations due to reception of an NRPPa message that utilized non-UE associated signalling.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

Annex 3. (TP to BL CR for TS38.455) support of AI assisted Positioning

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer. This function allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

- Data Collection Information Transfer. This function allows the gNB to request for training data information for AI based positioning with gNB-side model.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |
| Area-specific SRS Information Transfer | SRS Information Reservation Notification |
| Data Collection Information Transfer | a) Data Collection Notification b) Data Collection Initiation c) Data Collection Reporting |

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

8	NRPPa procedures

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| Data Collection Information Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Data Collection Notification | DATA COLLECTION NOTIFICATION |
| Data Collection Update | DATA COLLECTION UPDATE |

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.X	Data Collection Information Transfer

### 8.X.1	Data Collection Notification

#### 8.X.1.1	General

The Data Collection Notification procedure allows the LMF to indicate the NG-RAN node that it’s ready to prepare the data collection information for gNB-based AI model.

#### 8.X.1.2	Successful Operation

Figure 8.5.1.2.1: Data Collection Notification procedure. Successful operation.

The LMF initiates the procedure by sending a DATA COLLECTION NOTIFICATION message to the NG-RAN node. Which allows the NG-RAN node to request for AI Assisted Positioning Information towards the LMF for AI assisted positioning where AI model is gNB-based.

#### 8.X.1.3	Unsuccessful Operation

Not applicable.

#### 8.X.1.4	Abnormal Conditions

Void.

### 8.X.2	Data Collection Initiation

#### 8.X.2.1	General

The Data Collection Information Initiation procedure allows the NG-RAN to request positioning information from the LMF for data collection.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2.1: Data Collection Information Exchange procedure. Successful operation.

The NG-RAN initiates the procedure by sending a DATA COLLECTION REQUEST message to the LMF.

If the Data Collection Request IE is set to "start", the LMF shall reply with a DATA COLLECTION RESPONSE message and shall initiate the Data Collection Information Update procedure including the requested positioning information in the area of indicated in the TRP Area list IE.

If the Desired amount of UE labels IE is included, the LMF shall, if supported, take it into account when preparing for the UE labels and provide corresponding information to the gNB.

If the UE location required IE is included, the LMF shall, if supported, provide the location of the UE labels to the gNB in the DATA COLLECTION UPDATE message.

If TRP Measurement Quantities IE is included, the LMF shall, if supported, perform corresponding positioning measurement and provide corresponding results to the gNB in the DATA COLLECTION UPDATE message.

If Requested SRS Transmission Characteristics IE is included, the LMF shall, if supported, take it into account when performing Positioning Information Exchange and Measurement procedures for data collection.

If the Periodicity IE is included in the Requested Positioning Information Item IE, the LMF shall, if supported, consider is as the periodicity of reporting the requested Positioning information.

#### 8.X.2.3	Unsuccessful Operation

Figure 8.X.2.3.1: Data Collection Information Exchange procedure. Unsuccessful operation.

If the LMF cannot provide any of the requested positioning information as requested in the DATA COLLECTION REQUEST message, it shall respond with a DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.x.2.4	Abnormal Conditions

Void.

### 8.X.3	Data Collection Update

#### 8.X.3.1	General

The Data Collection Update procedure allows the LMF to provide positioning information to the NG-RAN for data collection.

#### 8.X.3.2	Successful Operation

Figure 8.X.3.2.1: Data Collection Update procedure.

The LMF initiates the procedure by sending a DATA COLLECTION UPDATE message to the NG-RAN node. The DATA COLLECTION UPDATE message contains the requested positioning information for data collection at NG-RAN.

#### 8.X.3.3	Unsuccessful Operation

Not applicable.

#### 8.X.3.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.1.X	Messages for Data Collection Information Transfer Procedures

#### 9.1.X.1	DATA COLLECTION NOTIFICATION

This message is used to indicate the NG-RAN node that data collection is allowed for gNB-based AI model.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |

#### 9.1.X.2	DATA COLLECTION REQUEST

This message is initiated to request positioning information for data collection.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Data Collection Request | M |  | ENUMERATED(start, stop, …) |  | YES | reject |
| TRP Data Collection List |  | 1 |  |  | YES | ignore |
| >TRP Data Collection Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| Desired amount of UE labels | O |  | INTEGER (1..65536, …) |  | YES | ignore |
| UE location required | O |  | ENUMERATED(true,…) |  | YES | ignore |
| TRP Measurement Quantities |  | 0 .. <maxnoPosMeas> |  |  | YES | ignore |
| >TRP Measurement Type | O |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP) |  |  |  |
| >Periodicity | O |  | ENUMERATED(160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.X.3	DATA COLLECTION RESPONSE

This message is sent to confirm start reporting of the requested positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.X.4	DATA COLLECTION FAILURE

This message is sent to report failure of initiating data collection of positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.X.5	DATA COLLECTION UPDATE

This message is sent to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| UE label Information list |  | 0..1 |  |  |  |  |
| >UE Label Information Item |  | 1..<maxnoofUELabels> |  |  |  |  |
| >>Location Information | M |  | OCTET STRING | Location of the UE label, as defined in TS 37.355 [14]. | - |  |
| >>Measurement Result | O | FFS |  | Including Part A and Part B. |  |  |
| >>Time Stamp | O |  | 9.2.42 | Location time stamp. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUELabels | Maximum no. of UE labels that can be included within one message. Value is 65536. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>