3GPP TSG-RAN WG3 Meeting #127bis	R3-251763

Wuhan, China, 7 – 11 April 2025

Agenda item:	16.2

Source:	Xiaomi

Title:	(TP for TS 38.413) Support of A-IoT

Document for:	Other

# 1	Introduction

As discussed in R3-251762, the following new procedures are proposed:

- introduce a new AIoT NG setup procedure between gNB and AOTF.

-  introduce a class 1 procedure for reader information exchange.

Introduce the following new Information in the AIoT related IEs

| Inventory request transfer | Inventory response transfer | Inventory report transfer |
| --- | --- | --- |
| Correlation ID Reader Selection Info (O) Device Identification (O) Aggregation Information (O) Service Type (O) Resource Allocation Assistance Information (O) | Correlation ID Reader ID (O) UL Device NAS Container List (O) End indication (O) | Correlation ID Reader ID (O) UL Device NAS Container List (O) End indication (O) |

| Command request transfer | Command response transfer |
| --- | --- |
| Correlation ID RAN Device ID DL Device NAS container No further communication indication (O) | Correlation ID RAN Device ID UL Device NAS container Device context released (O) |

| Inventory failure transfer | Command failure transfer |
| --- | --- |
| Correlation ID | Correlation ID |

# 2	TP to TS 38.413

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |
| Broadcast Session Transport | BROADCAST SESSION TRANSPORT REQUEST | BROADCAST SESSION TRANSPORT RESPONSE | BROADCAST SESSION TRANSPORT FAILURE |
| Inventory Request | INVENTORY REQUEST | INVENTORY RESPONSE | INVENTORY FAILURE |
| Command Request | COMMAND REQUEST | COMMAND RESPONSE | COMMAND FAILURE |
| AIoT NG Setup | AIOT NG SETUP REQUEST | AIOT NG SETUP RESPONSE | AIOT NG SETUP FAILURE |
| Reader Information Exchange | READER INFORMATION REQUEST | READER INFORMATION RESPONS | READER INFORMATION FAILURE |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.7.x	AIoT NG Setup

#### 8.7.x.1	General

The purpose of the AIoT NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AIOTF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling. This procedure applies only if the NG-RAN node is a gNB.

#### 8.7.x.2	Successful Operation

Figure 8.7.1.2-1: AIoT NG setup: successful operation

The NG-RAN node initiates the procedure by sending an AIOT NG SETUP REQUEST message including the appropriate data to the AIOTF. The AIOTF node responds with an AIOT NG SETUP RESPONSE message including the appropriate data.

#### 8.7.x.3	Unsuccessful Operation

Figure 8.7.1.3-1: AIoT NG setup: unsuccessful operation

If the AIOTF node cannot accept the setup, it should respond with an AIOT NG SETUP FAILURE message and appropriate cause value.

#### 8.7.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.xx.4	Reader Information Exchange

#### 8.xx.4.1	General

The purpose of the Reader Information Exchange procedure is to allow the AIoT CN node to request the NG-RAN node to provide information for readers hosted by the NG-RAN node. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.4.2	Successful Operation

Figure 8.xx.4.2-1: Reader Information Exchange: successful operation

The AIoT CN node initiates the procedure by sending a READER INFORMATION REQUEST message. The NG-RAN node responds with a READER INFORMATION RESPONSE message.

#### 8.xx.4.3	Unsuccessful Operation

Figure 8.xx.4.3-1: Reader Information Exchange: unsuccessful operation

If the NG-RAN node cannot provide any of the requested information for any Reader, it shall consider the procedure as failed and reply with the READER INFORMATION FAILURE message.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.2.x.8	READER INFORMATION REQUEST

This message is sent by the A-IoT CN node to the gNB to request reader information in the gNB.

Direction: A-IoT CN node ® NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Reader Information Request Transfer | M |  | OCTET STRING | Containing the Reader Information Request Transfer IE specified in subclause 9.3.x.8. | YES | reject |

#### 9.2.x.9	READER INFORMATION RESPONSE

This message is sent by the NG-RAN node to provide the reader information response to the A-IoT CN node.

Direction: NG-RAN node ® A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Reader Information Response Transfer | M |  | OCTET STRING | Containing the Reader Information Response Transfer IE specified in subclause 9.3.x.9. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.10	READER INFORMATION FAILURE

This message is sent by the NG-RAN node to report the unsuccessful providing of the reader information to the A-IoT CN node.

Direction: NG-RAN node ® A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Reader Information Failure Transfer | M |  | OCTET STRING | Containing the Reader Information Failure Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.x	AIOTF Related IEs

9.3.x.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation ID | M |  | FFS |  |
| Reader Selection Information | O |  | FFS |  |
| Device Identification | O |  | FFS |  |
| Service Type | O |  | ENUMERATED (Inventory and command, ...) |  |
| Approximate Number of AIoT Devices | O |  | FFS |  |
| Expected D2R Message Size | O |  | FFS |  |
| Time Interval for Response Aggregation | O |  | FFS |  |

9.3.x.2	Inventory Response Transfer

This IE provides the inventory response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |
| Correlation ID | M |  | FFS |  |
| Reader ID | M |  | FFS |  |
| Aggregated Device NAS Containers | O |  | FFS |  |
| End Indication | O |  | ENUMERATED (True, ...) |  |

9.3.x.3	Inventory Failure Transfer

This IE provides the inventory failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |
| Correlation ID | M |  | FFS |  |

9.3.x.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation ID | M |  | FFS |  |
| Reader ID | M |  | FFS |  |
| Aggregated Device NAS Containers | O |  | FFS |  |
| End Indication | O |  | ENUMERATED (True, ...) |  |

9.3.x.5	Command Request Transfer

This IE provide the command request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation ID | M |  | FFS |  |
| Reader ID | M |  | FFS |  |
| RAN Device ID | M |  | FFS |  |
| DL Device NAS Container | M |  | FFS |  |

9.3.x.6	Command Response Transfer

This IE provides the command response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |
| Correlation ID | M |  | FFS |  |
| RAN Device ID | M |  | FFS |  |
| UL Device NAS Container | O |  | FFS |  |

9.3.x.7	Command Failure Transfer

This IE provides the command failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |
| Correlation ID | M |  | FFS |  |

9.3.x.8	Reader Information Request Transfer

This IE provide the reader information request from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS |  |  |  |  |

9.3.x.9	Reader Information Response Transfer

This IE provides the requested reader information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>