3GPP TSG-RAN3 Meeting #127bis	R3-251772

Wuhan, China, April 07-11, 2025

Agenda Item:	14.3

Source: 	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Xiaomi, CATT

Title: 	TP for TS 38.300 on NR NTN Regenerative Payload Feeder Link Switch Over

Document for:	Discussion/Decision

# Introduction

This is a TP to TS 38.300 to capture the below agreement on Feeder Link switch over for NR NTN Regenerative Payload.

The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch

Clause 16.14.4.3 in TS 38.300 mentions about Feeder link switch over for Transparent payload as it says FLSO between 2 gNBs.

TS 38.300 Clause 16.14.4.3 “The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed.”

The above text is applicable for Transparent payload as the FLSO happens between 2 gNBs as shown below.

But for a Regenerative payload FLSO without AMF change, the gNB and the AMF remains intact during FLSO as shown below. The gNB and AMF remains the same before and after FLSO and HO is not necessary. Hence this needs to be mentioned clearly in TS 38.300 in Clause 16.14.4.3.

It is clear for a reader to have different paragraphs for Regenerative and Transparent payload, as some text in the existing paragraph is not required for Regenerative payload.

Hence, similar text is needed for FLSO for Regenerative Payload to mention that NTN control function decides the FLSO time and to incorporate the above RAN3 agreement. “In Regenerative NTN payload, the NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The migration of transport path from old NTN GW to new NTN GW is performed by existing mechanism, e.g. multiple TNLA association. “

Conclusion 1: Add “For Transparent Payload” to the text in Clause 16.14.4.3 as “ For Transparent Payload the NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed.”

Conclusion 2: Add text for FLSO for Regenerative Payload to reflect the above agreement as “In Regenerative NTN payload, the NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The migration of transport path from old NTN GW to new NTN GW is performed by existing mechanism, e.g. multiple TNLA association. “

# TP for TS 38.300

16.14.4	Switchover

16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for a specific NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN.

16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

16.14.4.3	Procedures

For Transparent Payload, the NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switch over is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

In Regenerative NTN payload, the NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The change of transport path from old NTN GW to new NTN GW is performed by existing mechanism, e.g. multiple TNLA association.