3GPP TSG-RAN3 Meeting #127bis	R3-251785

Wuhan, China, April 07-11, 2025

Agenda Item:	11.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on remaining issues for NG-RAN AI/ML for Network Slicing

Document for:	Discussion/Decision

# Background

In RAN3#127 we discussed briefly on UE performance feedback reporting, and it was agreed to UE performance at Slice level granularity can be reported and the definition of the UE performance can be referred in SA5 spec.

RAN3#127 UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases ... Whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity? To be checked in next meeting…

In this paper we discuss our views on the remaining open issues on UE performance reporting.

# Discussion

## Slice based UE Performance

In the last meeting it was agreed to report UE performance at slice level granularity as feedback and other granularities are not precluded.

TS 28.552 only details how average UE throughput can be calculated in kbits/s. It does not detail how UE throughput is averaged at slice level or QoS level. It just says sub counters at NSSAI, per QoS level etc can be reported. The problem of aggregation remains the same for both UE performance at slice level and at QoS level.

TS 28.552 ##### 5.1.1.3.1 Average DL UE throughput in gNB a) This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC) and subcounters per supported S-NSSAI, and subcounters per PLMN ID, and subcounters per BWP. In the case of per BWP, the UE data volume refers to the total volume scheduled for each Active BWP with same bandwith except UEs with activated supplemental aggregated carrier(s).

Since the aggregation technique is not standardised for any sub counter be it NSSAI or per QoS, it is not possible for the receiver of the measurement to know how the values are aggregated. Hence it will  not be like-to-like comparison of UE throughput at both sender and receiver.

There are 2 options possible -

- TS 28.552 or TS 38.423 provides clarification on how the UE performance metrics are aggregated per NSSAI or per QoS flow etc

- Report UE performance metrics at lowest granularity per DRB or group of QoS Flows and the receiving node can aggregate the metrics based on its internal implementation.

We are fine with both the options. However easiest option will be option 2 – “to report at per DRB or group of QoS flow level” as this reporting will be needed for NR-DC use case as well. Any future use case can receive the UE performance metrics at lowest granularity possible and aggregate as per its needs.

Since TS 28.552 does not specify aggregation technique for aggregating UE performance metrics at sub counter level (NSSAI, QoS, PDU level etc), sender and receiver may not have the common aggregation technique and comparison of UE performance metrics across both the nodes may be flawed.

As a compromised approach the reporting node can have flexibility to report either DRB level UE performance or slice level UE performance. If DRB level UE performance is reported the receiving node can aggregate, it to slice level UE performance. Sample IE structure is shown below –

9.2.3.x2	UE performance

This IE indicates the UE performance at per slice or per DRB level.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Slice UE performance List |  | 0..1 |  |  |
| > Slice UE performance Item |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 |  |
| >S-NSSAI UE performance List |  | 1 |  |  |
| >>S-NSSAI UE performance Item |  | 1..<maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>UE Performance | O |  | 9.2.3.179 |  |
| DRB UE performance List |  | 0..1 |  |  |
| > DRB UE performance Item |  | 1..< maxnoofDRBs > |  |  |
| >>DRB ID | M |  | 9.2.3.33 |  |
| >>Slice ID | O |  | 9.2.3.21 |  |
| >>Cell ID | O |  | 9.2.2.7 |  |
| >>UE Performance | O |  | 9.2.3.179 |  |

To provide flexibility to the reporting node to report UE performance at either slice level or DRB level (group of QoS level) as a compromised approach. DRB level UE performance reporting will anyways be needed for AI/ML based NR-DC use case.

## Slice Available Capacity

Predicted slice available capacity was agreed to be introduced for AI/ML based Slice optimisation. Interpretation of Slice available capacity value is dependent on Composite available Capacity group IE which is used in Composite available capacity. Two options are possible.

- Report Predicted Composite available Capacity along with Predicted Slice available capacity.

- Report Cell capacity class value along with Predicted Slice available capacity

We prefer to report predicted composite available capacity along with predicted Slice available capacity. This will provide additional information to the node on cell capacity and slice capacity ratio, instead of reporting the Cell Capacity class value.

From TS 38.423 9.2.2.55 Slice Available Capacity The Slice Available Capacity IE indicates the amount of resources per network slice that are available per cell relative to the total NG-RAN resources per cell. The Slice Available Capacity Value Downlink IE and the Slice Available Capacity Value Uplink IE can be weighted according to the ratio of the corresponding cell capacity class values contained in the Composite Available Capacity Group IE, if available.

Report Predicted Composite available capacity along with Predicted Slice available capacity as the predicted composite available capacity can provide additional information on cell capacity along with slice capacity.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	Since TS 28.552 does not specify aggregation technique for aggregating UE performance metrics at sub counter level (NSSAI, QoS, PDU level etc), sender and receiver may not have the common aggregation technique and comparison of UE performance metrics across both the nodes may be flawed.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	To provide flexibility to the reporting node to report UE performance at either slice level or DRB level (group of QoS level) as a compromised approach. DRB level UE performance reporting will anyways be needed for AI/ML based NR-DC use case.

Proposal 2:	Report Predicted Composite available capacity along with Predicted Slice available capacity as the predicted composite available capacity can provide additional information on cell capacity along with slice capacity.