3GPP TSG-RAN WG3 Meeting #127bis	R3-251811

# Wuhan, China, 7– 11 April 2025

Agenda item:	14.2

Source: 	Samsung

Title: 	Remaining issue on support broadcast service for NR NTN

Document for:	Discussion and decision

Introduction

Last meeting further discussed the support of MBS broadcast service for NR NTN, and the agreement and open issues are captured as follows,

Encode the Intended MBS service area as OCT STRING which refers to RAN2 spec.

Work on the stage2/3 details.

In this contribution, we further discuss these open issues.

Discussion

Last meeting continued to discuss to support broadcast service for NR NTN, and the main progress is to agree to encode the intended MBS service area as OCTET STRING as follows,

9.3.1.x	Intended Service Area Information

This IE provides information on the Intended Service Area for MBS Broadcast Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Intended Service Area Information | M |  | OCTET STRING | Encoded in the same format as the IntendedServiceAreaInfo IE as defined in TS 38.331 [18] |

During the discussion, some company questioned whether the gNB is able to decode the OCTET STRING, and our understanding is that a gNB is able to decode the OCTET STRING. For example, in XnAP HANDOVER REQUEST message, the source node sends the RRC Context IE which contains the HandoverPreparationInformation message to the target node, and the IE type of RRC Context is also OCTET STRING, which is given as follows,

| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| --- | --- | --- | --- | --- | --- | --- |

Everyone knows that the HandoverPreparationInformation message needs to be decoded by the target node in order for the target node to prepare the Handover Command sent to the UE. Therefore, similarly, a gNB is totally capable of decoding the OCTET STRING, and agreeing to encode the Intended MBS service area as OCTET STRING does not impact the gNB to understand the content of the intended MBS service area.

Another open issue discussed during offline is that how to handle the case when only the Intended Service Area list is provided without cell or TAI information in the MBS Service Area Information IE. Some company pointed out that in case there are two cells which are totally overlapped, and the MBS service is only intended to provide to one of the cells, then the gNB is not able to distinguish on which cell to transmit the MBS service.

However, we need to bear in mind that we’ve already agreed that the geographical area corresponding to a mapped cell is configured to the Core Network via OAM. So the CN knows two cells are totally overlapped if such situation happens. As a result, if the CN wants to transmit the MBS service over only one of the cells, it makes no sense for the CN to just send the Intended Service Area information without TAI or cell information to the gNB.

As a result, we do not identify any issue on the overlapped cell cases, if only the Intended Service Area Information is provided and the cells are overlapped, the gNB will understand that both cells will transmit the MBS service.

As a summary, our understanding is that the discussion on support broadcast service for NR NTN can be completed. And further discussion is only driven by incoming LSes.

Proposal 1: No issue has been identified for overlapped cell case by using the existing signalling.

Conclusion

In this paper, we provide discussion on support MBS broadcast service for NR NTN. The following proposals are provided,

Proposal 1: No issue has been identified for overlapped cell case by using the existing signalling.

References