3GPP TSG-RAN WG3 Meeting #127bis	R3-251812

# Wuhan, China, 7– 11 April 2025

Agenda item:	14.3

Source: 	Samsung

Title: 	Discussion on support of regenerative payload for NR NTN

Document for:	Discussion and decision

Introduction

Last meeting further discussed the support of regenerative payload for NR NTN, but the discussion mainly focuses on the reply LS to SA2, and the following discussion is not carried out last meeting,

Inactive UE support:

Option 1, UE context is forwarded to the next satellites

Option 2, UE context is kept on a fixed node (on-ground gNB or AMF)

Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

Whether to support NG suspend/Resume?

Continue the discussion on above open issues…

In this contribution, we further discuss the support of regenerative payload.

Discussion

Last several RAN3 meetings continued to discuss the NG interface management. And the following open issue is captured,

RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection?

And another alternative is to introduce new NG suspend/resume procedures.

According to the discussion during last several RAN3 meetings, the proponents of NG suspend/resume mentioned that the introduction of NG suspend/resume will save transmission energy when recovering the connection between gNB and AMF since less information is expected to be transmitted compared to NG setup as well as the following up RAN/AMF Configuration Update procedures.

On the other hand, the introduction of NG suspend/resume will require much more storage on NG interface management related information for gNB on board, especially after several orbits, the gNB on board may need to store the NG interface related information with every AMF along the orbits. In addition, there’s also a concern that whether the original suspended information is still valid when the gNB on board connects back to a specific AMF.

In our understanding, considering that the supported TA list of an on-board gNB could be provided to AMF by OAM, the main information that is needed to be stored at the gNB onboard is the TNL association related information, which actually does not require too much storage space, even if the gNB onboard needs to maintain the interface related information with every AMF along the orbits.

In addition, we observe that the addition/update/removal of the TNLA is initiated by the AMF, and both AMF and gNB is aware of the ephemeris of the regenerative payload as well as the supported TA information for a gNB on board. As a consequence, we think it is also worth considering whether to introduce suspend/resume indicators to AMF Configuration Update procedure instead.

And we can immediately see a benefit when we introduce a resume indicator to AMF CONFIGURATION UPDATE message that it will potentially save the number of interface management messages to be exchanged when the feeder link recovers. More specifically,

- Reusing the NG Setup approach to recover the feeder link: Needs one Class 1 NG Setup procedure + one Class 1 AMF Configuration Update procedure if there are TNLAs that are needed to be added/updated/removed.

- Adding resume indicator in RAN CONFIGURATION UPDATE message approach to recover the feeder link: Needs one Class 1 RAN Configuration Update procedure + one Class 1 AMF Configuration Update procedure if there are TNLAs that are needed to be added/updated/removed.

- Our proposed approach: Only needs one Class 1 AMF Configuration Update procedure to add/update/remove TNLAs and recover the feeder link at the same time.

We think it is the simplest approach we can identify.

Proposal 1: Introduce Suspend/Resume indicators in AMF CONFIGURATION UPDATE message, to support NG suspend/resume.

Another open issue is on the support of RRC Inactive UEs.

For the options,

Option 1, UE context is forwarded to the next satellites

Option 2, UE context is kept on a fixed node (on-ground gNB or AMF)

Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

Remember that we’ve already agreed the following agreement in RAN3,

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

So basically, the current mechanism has already been workable without supporting any of the options on the table for enhancement.

Firstly, we think Option 1 is not a good choice, since it introduces too much signalling overhead over Xn/NG interface, especially for those NGSO satellites.

Secondly, for NGSO satellites, the Option 3 does not work very well especially when the feeder link between the anchor gNB and the CN on ground is unavailable; under such condition, the context retrieve will still be failed so the benefit looks limited to introduce extra NG context retrieve procedure only.

Thirdly, for Option 2, the solution to keep UE context on an on-ground gNB will introduce other open issues including e.g. the need to define a special on-ground gNB to store the UE context, the forwarding mechanism, etc., which introduces too much spec impact. As a result, compared to keeping the UE context on an on-ground gNB, keeping the UE context on AMF looks more reasonable. On the other hand, the feasibility to keep the UE context on AMF needs coordination with SA2, and the primary observation is that storing all the RAN side UE context (including RRC context and NG context for all the RRC inactive UEs) will require an extra huge amount of storage space from the CN.

So we prefer not to make any enhancements to support RRC inactive UEs in R19, considering the cost brought by each option.

Proposal 2: Do not make any enhancement to support RRC inactive UEs in Rel-19.

The next open issue is about the location based CHO, which receives many attention by companies.

In the contributions submitted to last meeting, companies proposed several solutions for XnAP signalling,

Option 1: Add parameters related to condEventD1 in XnAP signalling

Option 2: Add coarse location info in XnAP signalling

In our understanding, since we have introduced time based CHO also over XnAP, and basically the location based parameters can be translated to and reflected by time based parameters already specified over XnAP up to source node implementation, we see no strong need to adopt Option 1. In addition, the settings of parameters in Option 1 including Hysteresis as well as timeToTrigger may be dependent on vendor’s specific implementation or policy, which is not proper to be disclosed over XnAP. That is also the reason why time based parameters are more appropriate to be signalled over XnAP.

Moreover, in our opinion, deciding whether to adopt any option mainly depends on whether such option can bring any delta to the current spec. We do not see Option 1 brings delta benefits based on signalling time based parameters; while compared to Option 1, Option 2 may have additional benefits on top of the time based parameters. So we propose,

Proposal 3: Agree to introduce coarse location information over XnAP for location based CHO.

Conclusion

In this paper, we provide primary discussion on support of regenerative payload for NR NTN. The following proposals are provided,

Proposal 1: Introduce Suspend/Resume indicators in AMF CONFIGURATION UPDATE message, to support NG suspend/resume.

Proposal 2: Do not make any enhancement to support RRC inactive UEs in Rel-19.

Proposal 3: Agree to introduce coarse location information over XnAP for location based CHO.

References