3GPP TSG-RAN WG3 Meeting #127bis	R3-251815

# Wuhan, China, 7– 11 April 2025

Agenda item:	22.2

Source: 	Samsung

Title: 	Discussion on multi-hop for SL relay

Document for:	Discussion and decision

Introduction

Last meeting primarily discussed the multi-hop SL relay, and the following agreements are captured,

RAN3 can work on the new signaling flow for multi-hop relay for the purpose of Initial Access procedure based on the existing procedures in TS 38.401.

RAN3 agrees to support the Multi-hop relay in CU-DU split architecture.

The existing F1AP UE Context Setup and UE Context Modification procedure can be used as a baseline to establish the PC5/Uu Relay RLC channel(s) for relaying of Remote UE’s SRB/DRB.

For the initial UL RRC transfer message, the gNB-DU informs the gNB-CU of the multi-hop relay related information including at least gNB-DU UE F1AP ID of the Last Relay UE and Remote UE Local ID.

In this contribution, we discuss the fundamental aspects to be specified.

Discussion

The first thing, in our understanding, RAN3 should determine the terminology to be used for further discussion.

Firstly we can reuse the terminology “Last Relay UE”, “First Relay UE” and “Intermediate Relay UE” as RAN2 defined for further RAN3 discussion.

Secondly, we can define the following terminologies used for further discussion, e.g.,

The Parent Relay UE of an Intermediate Relay UE is the Intermediate Relay UE’s next hop Relay UE in UL.

The Child Relay UE of an Intermediate Relay UE is the Intermediate Relay UE’s next hop Relay UE in DL.

Proposal 1: Introduce the following terminologies to help RAN3’s further discussion,

- The Parent Relay UE of an Intermediate Relay UE is the Intermediate Relay UE’s next hop Relay UE in UL.

- The Child Relay UE of an Intermediate Relay UE is the Intermediate Relay UE’s next hop Relay UE in DL.

Recall that in R17/18, to support the single-hop relay, over F1AP,

- For the Relay UE context, one list of Uu RLC channel (between gNB and Relay UE) and one list of PC5 RLC channel (between Relay UE and Remote UEs) are needed to be coordinated between gNB-CU and gNB-DU.

- For the Remote UE context, one list of PC5 RLC channel (between Relay UE and Remote UE) is needed to be coordinated between gNB-CU and gNB-DU.

While for R19 intermediate Relay UE, two separate lists of PC5 RLC channel (each list for one of two adjacent UEs) are needed to be coordinated between gNB-CU and gNB-DU. And the current F1AP spec is not enough for the intermediate relay UE context setup/modification.

Proposal 2: Two sets of PC5 RLC Channel to be Setup/Modified/Release list are needed for the intermediate relay UE context setup/modification.

Another topic is about the intra-gNB service continuity. According to the WID, the intra-gNB multi-hop indirect to direct/single-hop indirect path switching is with the first priority; while the intra-gNB direct/single-hop indirect to multi-hop indirect path switching is with the second priority. In our understanding, we can start with 1st priority call flows, and the following call flows are tentative ones,

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework (Inter-gNB-DU switch case)

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework (Intra-gNB-DU switch case)

Proposal 3: For intra-gNB multi-hop indirect to single-hop indirect path switching scenario, consider both inter-gNB-DU switch case and intra-gNB-DU switch case.

Conclusion

In this paper, we provide primary discussion on multi-hop relay. The following proposals are provided,

Proposal 1: Introduce the following terminologies to help RAN3’s further discussion,

- The Parent Relay UE of an Intermediate Relay UE is the Intermediate Relay UE’s next hop Relay UE in UL.

- The Child Relay UE of an Intermediate Relay UE is the Intermediate Relay UE’s next hop Relay UE in DL.

Proposal 2: Two sets of PC5 RLC Channel to be Setup/Modified/Release list are needed for the intermediate relay UE context setup/modification.

Proposal 3: For intra-gNB multi-hop indirect to single-hop indirect path switching scenario, consider both inter-gNB-DU switch case and intra-gNB-DU switch case.

Reference