3GPP TSG-RAN WG3 Meeting #127bis														R3-251830

Wuhan, China, 7th – 11th Apr 2025

Agenda item:	21.3

Source:	Canon

Title:	Discussion on Multimodality and uplink congestion

Document for:	Discussion and Decision

# 1	Introduction

RAN plenary updated the following two XR objectives ([1]):

- Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]: - Specify uplink congestion signalling [RAN2, RAN3]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

# 2	Discussion on Multi-modality awareness

SA2 LS ([2]) states:

1. Overall Description: SA2 thanks SA plenary, RAN2 and RAN3 for their liaisons, SA2 has agreed the attached CRs. 2. Actions: To RAN2, RAN3 and SA Plenary: ACTION: SA2 kindly asks RAN2, RAN3 and SA Plenary to take the above information into account.

The attached CRs implement MMSID communication from PCF to AMF, then from AMF to NGRAN as part of PCC rules applied to QoS flows participating to the same multi-modal application ([4]).

Observation 1: MMSID is provided to NGRAN as part of PDU session modification procedure

SA2 did not specify how the MMSID can be sent to the UE. However, we think that for uplink applications, the MMSID shall be provided to the UE. In case UE side application layer does not provide the MMSID to the UE and since the network side will have the information anyway, we think that it is possible for the network to send the MMSID to the UE.

Proposal 1: For uplink the NGRAN shall provide the MMSID to the UE

Since the MMSID is provided to the NGRAN as part of the PDU session modification procedure, it is possible to consider sending the MMSID to the UE as part of the PDU session modification procedure.

QoS flow binding to MMSID is not supposed to be dynamic, it is performed by the AF using the Nnef_AFsessionWithQoS service ([5]). Hence, semi static solution may be considered to communicate the MMSID to the UE.

Observation 2: It is not necessary to provide the MMSID to the UE dynamically

Proposal 2: As part of PDU session modification procedure, use semi-static signalling to provide the MMSID to the UE

# 3	Uplink congestion

An LS have been sent by RAN2 to RAN3 and SA2 ([6]):

1. Overall Description: For XR rate control, RAN2 discussed how the gNB gets the information of QoS flows which are subject to uplink rate control, i.e., whether the information is from CN or UE, and agreed that: - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. 2. Actions: To SA2, RAN3. ACTION: RAN2 respectfully asks SA2 and RAN3 to take the above decision into consideration for further work on XR uplink rate control and provide feedback.

In legacy systems, IEs are exchanged between the CN and the NGRAN carrying L4S configuration information. Similar to what RAN2 wants to implement for XR rate control, the L4S IEs provide per QoS flow configuration information ([7] clauses 9.3.1.266 and 9.3.1.267). By taking a similar approach for XR rate control configuration information, specification efforts can be saved.

Proposal 3: Create new IEs, similar to ECN marking IEs, to handle XR rate control configuration from CN to NGRAN

ECN marking IEs can be added to PDU SESSION RESOURCE SETUP REQUEST and PDU SESSION RESOURCE MODIFY REQUEST. Similarly, these two PDU sessions management messages may include news IEs indicating XR rate control configuration.

Proposal 4: Add XR rate control information element to the PDU SESSION RESOURCE SETUP REQUEST message.

At handover it is necessary to provide the same level of information to the target gNB. By applying the same concept as defined for L4S/ECN marking, specification efforts can be saved.

Observation 3: Rate control information shall be provided to target gNB during handover.

Similar to L4S, handover request (Xn or NG) received at target gNB shall contain information about which QoS flow can be subject to rate adaptation. The target gNB receiving this XR rate control IE can understand that the handover request is related to the handover of a UE that accepts XR rate control on designated QoS flows. Thus, when triggering the handover, the source gNB, sends a handover request (or required) message to the target gNB (or AMF) including an information element related to XR rate control.

Proposal 5: XR rate control IEs are added in (Xn) handover request message and (NG) handover required message.

# 4	Conclusion

Our proposals for multi-modal are:

Observation 1: MMSID is provided to NGRAN as part of PDU session modification procedure

Proposal 1: For uplink the NGRAN shall provide the MMSID to the UE

Observation 2: It is not necessary to provide the MMSID to the UE dynamically

Proposal 2: As part of PDU session modification procedure, use semi-static signalling to provide the MMSID to the UE

Our proposals for uplink congestion are:

Proposal 3: Create new IEs, similar to ECN marking IEs, to handle XR rate control configuration from CN to NGRAN

Proposal 4: Add XR rate control information element to the PDU SESSION RESOURCE SETUP REQUEST message.

Observation 3: Rate control information shall be provided to target gNB during handover.

Proposal 5: XR rate control IEs are added in (Xn) handover request message and (NG) handover required message.

# References

- RP-250107	Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)

- S2-2502465	LS reply on multi-modality awareness, SA2 (China Mobile)

- RP-250106	Status Report to TSG, Nokia (Rapporteur)

- S2-2502498	Support of multi-modal awareness at NG-RAN, China Telecom, Nokia, CATT, vivo, Lenovo

- S2-2502463 Provisioning Multi-modal Service ID to NG-RAN, Qualcomm, Ericsson, Nokia, Xiaomi, CMCC, Huawei, Interdigital, CATT, China Telecom, Futurewei, Samsung, Sony

- R3-251513 LS on uplink rate control, RAN2 (Nokia)

- TS 38.413, NG Application Protocol (NGAP) (Release 18)