3GPP TSG-RAN WG3 Meeting #127bis	R3-251851

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	12.2

Source:	Huawei

Title:	 (TP for WAB BL CR for TS 38.401) Radio Resource Multiplexing Coordination for WAB-node

Document for:	Discussion

1	Introduction

At the previous meeting, RAN3 didn’t discuss how to handle the resource multiplexing (i.e., the radio resource coordination) for WAB which was one objective of the WID [1].

The objectives of the Wireless Access Backhaul (WAB) work are as follows:

-	Specifications for the support of WAB including [RAN3]:

…

-	The handling of:

-	PCI collision avoidance.

-	Reconfiguration of TAC and RANAC on WAB-gNBs.

-	Mechanisms to avoid multi-hop WAB topology.

-	Radio-resource coordination between access and backhaul links.

-	NG connection management.

This contribution will dicusses on the resource multiplexing coordination for WAB.

2	Discussion

2.1	Resource multiplexing coordination for WAB-node

2.1.1 General case

For out-band mode, the WAB-gNB and WAB-MT use different frequency, so, there is no interference between the WAB-gNB and the WAB-MT. For in-band mode, to avoid interference between access link and backhaul link, the resources (e.g., frequency domain, time domain, spatial domain, etc.) to be used by access link and backhaul link must be coordinated.Specifically, the BH-RAN node should be upgraded to be aware of which WAB-gNB is co-located with a WAB-MT, and it also needs to know the multiplexing capability of the WAB-MT and its co-located WAB-gNB, as well as the resource configurations of the WAB-gNB. For the co-location discovery, the WAB-gNB sends collocated WAB-MT’s ID to the BH-gNB over Xn, or the WAB-MT sends collocated WAB-gNB’s ID to the BH-gNB over RRC. To reduce the RRC impact, we prefer the Xn-based solution. It is proposed:

- For in-band deployment, the resource multiplexing coordination must be conducted between access link and backhaul link.

- For the co-location discovery on the BH-gNB, the WAB-gNB sends WAB-MT’s ID to the BH-gNB over Xn.

- For the IAB-DU to donor-CU resource reporting, the IAB-DU includes the IAB Info IAB-DU IE which contains the multiplexing info and the IAB STC info. The multiplexing info represents the multiplexing capabilities between the IAB-DU’s cell and the cells configured on the co-located IAB-MT. The IAB STC info contains cell SSB Transmission Configurations (STC) information of the IAB-DU.

- In addition, for the donor-CU to IAB-DU messages, e.g., the GNB-DU RESOURCE CONFIGURATION message, can provide the following information:

  - the resource configuration of the cells served by the IAB-DU;

  - the child node’s cell resource configuration and multiplexing capabilities;

  - the neighbour nodes’ cell resource configuration;

  - the NA resource configuration of the serving cell of the co-located IAB-MT.

Similarly, in IAB, there are some procedures related to the resouce multiplexing coordination over F1AP since R16.

For the BH-gNB, it would be helpful if the BH-gNB can know the information of WAB-node to avoid scheduling confliction between WAB-MT’s backhaul link and the WAB-gNB’s access link, similar as the parent node to know the child IAB node’s configuration in IAB case. So, the WAB-gNB should report its cell resource configuration and multiplexing capabilities to the BH-gNB.

- The WAB-gNB sends the multiplexing capabilities and the resource configuration information of its served cells to the BH-gNB.

For the WAB-gNB, since it is a full gNB and the relation between WAB-gNB and BH-gNB is somehow different from the relation between IAB-DU and donor-CU, it is FFS whether the BH-gNB can configure the WAB-gNB’s cell resource like the donor-CU provides resource configuration to IAB-DU, such issue needs to be coordinated with RAN1.

Furthermore, it would be helpful if the WAB-gNB is informed with the neighbour nodes’ cell resource configuration, for the collision avoidance. FFS on whether it is informed by the BH-gNB or the neighbour node itself.

For the child node and serving cell configurations, since the WAB-gNB should not have any child node, the “child node” related configurations in IAB is not needed for WAB.

The “NA resource configuration of the serving cell of the co-located WAB-MT” is also not needed, since the parent node should not be an IAB-node, and there is no “NA resource” for a non-IAB-DU.

In summary, it is proposed:

- The WAB-gNB is informed the neighbour nodes’ cell resource configuration. FFS on whether it is informed by the BH-gNB or the neighbour node itself.

It is worth noting that the above proposals are derived based on the assumption that the whole resource multiplexing framework (e.g., the HSNA attribute, the multiplexing capability of access and backhaul, the backhaul link scheduling should avoid hard resource in access link, etc.) defined for the IAB can be reused for the WAB. But the IAB resource multiplexing framework was defined by RAN1, so it is better to check with RAN1 to confirm on such assumption. A draft LS to RAN1 is provided in the Annex.

- RAN3 to coordinate with RAN1 on whether the resource multiplexing coordination framework between backhaul link and access link for IAB can be reused for WAB, and whether the BH-gNB configures the resource of the WAB-gNB cells.

2.1.2 Resource multiplexing coordination during WAB-node mobility

During WAB mobility, the WAB-node resource multiplexing coordination with the target BH-gNB may be untimely, causing the performance degradation. This issue can be categorized into two cases:

- Case 1: If the Xn interface between the WAB-gNB and the target BH-gNB does not exist, the resource multiplexing related signalling cannot be exchanged between the WAB-gNB and the target BH-gNB.

- Case 2: Even if the Xn interface is established, the performance degradation may still occur in this case, since the WAB-node may only perform resource multiplexing coordination with the serving BH-gNB but not the candidate target BH-gNBs before the WAB-MT handover to the target gNB.

Therefore, before the WAB-MT handover execution, it will be beneficial if the target BH-gNB and the WAB-gNB can be aware of each other, and implement resource multiplexing coordination timely. If the Xn interface does not exist, the Xn interface setup can be triggered at that time, and then the resource multiplexing can be implemented. To address this issue, two solutions are provided here:

- Solution 1. The target BH-gNB trigger the resource multiplexing coordination towards the WAB-gNB during the co-located WAB-MT’s handover. This solution requires that the target BH-gNB can be aware of the co-location between the WAB-gNB and the WAB-MT, and establishes the Xn interface towards the WAB-gNB if needed. Then the resource multiplexing is implemented.

- Solution 2. The WAB-gNB trigger the resource multiplexing coordination towards the target BH-gNB during the co-located WAB-MT’s handover. This solution requires that the WAB-gNB knows the BH-gNB’s ID and can provide information for co-location discovery at the target BH-gNB. Then the resource multiplexing can be implemented.

It is proposed:

- For WAB-node mobility, the resource multiplexing coordination between target BH-gNB and the WAB-gNB can be conducted before the completion of WAB-MT’s handover.

3	Conclusion

This paper provides analysis about the resource multiplexing coordination for WAB-node. The proposals are as follows:

- For in-band deployment, the resource multiplexing coordination must be conducted between access link and backhaul link.

- For the co-location discovery on the BH-gNB, the WAB-gNB sends WAB-MT’s ID to the BH-gNB over Xn.

- The WAB-gNB sends the multiplexing capabilities and the resource configuration information of its served cells to the BH-gNB.

- The WAB-gNB is informed the neighbour nodes’ cell resource configuration. FFS on whether it is informed by the BH-gNB or the neighbour node itself.

- RAN3 to coordinate with RAN1 on whether the resource multiplexing coordination framework between backhaul link and access link for IAB can be reused for WAB, and whether the BH-gNB configures the resource of the WAB-gNB cells.

- For WAB-node mobility, the resource multiplexing coordination between target BH-gNB and the WAB-gNB can be conducted before the completion of WAB-MT’s handover.

4. Reference

- RP-242395, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T .

Annex A:	LS to RAN1 on resource multiplexing of WAB

3GPP TSG-RAN WG3 Meeting #127bis	R3-XXXXXX

Wuhan, China, 7th – 11th April, 2025

Title:	[DRAFT] LS on resource multiplexing of WAB

Response to:	xxxxxx

Release:	Rel-19

Work Item:	NR_WAB_5GFemto-Core

Source:	Huawei [to be: RAN3]

To:	RAN1

Cc:	RAN2

Contact Person:

Name:	Yuanping Zhu

Tel. Number:

E-mail Address:	zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 -

1. Overall Description:

RAN3 is currently working on additional topological enhancements for NR, including the Rel-19 WAB (Wireless Access Backhaul). The WAB-node comprises a gNB component (WAB-gNB) and an MT component (WAB-MT), The WAB-MT is served by a BH-gNB via an NR-Uu backhaul link, and the WAB-gNB provides NR-Uu access link to UE.

Regarding the resource multiplexing coordination between the access link and the backhaul link of WAB-node, RAN3 assumes that the resource multiplexing framework defined for IAB (e.g., the HSNA attribute, the multiplexing capability of access and backhaul, the backhaul link scheduling should avoid hard resource in access link, etc.) can be reused, and take this as baseline for the signalling design in WAB topic.

For the detailed signalling design, the IAB-donor-CU can provide cell resource configuration to the IAB-DU via F1 interface. But considering the WAB-gNB is a full gNB, it is uncertain that it is still reasonable to let the BH-gNB to decides the cell resource configuration for WAB-gNB to achieve the resource multiplexing coordination.

With that in mind, RAN3 would like to seek input on the following issues:

Issue-1:

Whether the resource multiplexing framework defined for IAB (e.g., the HSNA attribute, the multiplexing capability of access and backhaul, the backhaul link scheduling should avoid hard resource in access link, etc.)is also applicable for WAB?

Issue-2:

Whether the BH-gNB is capable of configuring the resources of the WAB-gNB?

Issue-3:

If the BH-gNB is able to configure WAB-gNB resources, which specific resources should be configured?

2. Actions:

To RAN1:

ACTION: 	RAN3 asks RAN1 to provide feedback on the 3 issues above.

3. Date of Next RAN3 Meetings:

RAN3#128			19th May – 23rd May, 2025			Malta, MT

RAN3#129			25th August – 29th August, 2025		India, IN

Annex B:	TP for WAB BL CR for TS 38.401

CHANGES START

# X	Wireless Access Backhaul

## X.1	WAB-node integration procedure

>>>>>>>>>>unchanged parts are skipped<<<<<<<<<<

## X.8 	Resource multiplexing

For in-band mode, to avoid interference between WAB-node’s access link and backhaul link, the resources (e.g., frequency domain, time domain, spatial domain, etc.) to be used by access link and backhaul link needs to be coordinated, via XnAP signalings between WAB-node and the BH-gNB. To achieve this, the BH-gNB needs to discover the co-location relationship between the WAB-MT and the WAB-gNB, via the WAB-MT’s ID contained in the XN SETUP REQUEST message from the WAB-gNB. In addition, to ensure the in-band mode WAB can selects a proper BH-gNB, the BH-gNB will broadcast whether it supports resource multiplexing coordination for WAB.

During the mobility of the WAB-node, the resource multiplexing coordination between target BH-gNB and the WAB-gNB can be conducted before the completion of WAB-MT’s handover.